3GPP TSG-RAN WG2 Meeting #111e	R2-2006856

Online, August 17th - 28th

Agenda item:	8.7.2

Source:	Nokia, Nokia Shanghai Bell

Title:	NR SL-based UE-to-UE relay for unicast SL

WID/SID:	FS_NR_SL_relay - Release 17

Document for:	Discussion and Decision

# 1	Introduction

The objective of SI on NR SL-based relay is stated in RP-193253 as follows.

This study item targets to study single-hop NR sidelink-based relay.

- Study mechanism(s) with minimum specification impact to support the SA requirements for sidelink-based UE-to-network and UE-to-UE relay, focusing on the following aspects (if applicable) for layer-3 relay and layer-2 relay [RAN2];

  - Relay (re-)selection criterion and procedure;

  - Relay/Remote UE authorization;

  - QoS for relaying functionality;

  - Service continuity;

  - Security of relayed connection after SA3 has provided its conclusions;

  - Impact on user plane protocol stack and control plane procedure, e.g., connection management of relayed connection;

- Study mechanism(s) to support upper layer operations of discovery model/procedure for sidelink relaying, assuming no new physical layer channel / signal [RAN2];

This paper discusses some general aspects on NR SL-based UE-to-UE relay for providing service continuity, power efficiency and QoS enhancement for unicast SL.

# 2	Discussion

Figure 1 illustrates a single-hop NR SL-based UE-to-UE relay for a unicast SL between a transmitting UE (Tx UE) and a receiving UE (Rx UE) via a Relay UE (R-UE).

Figure 2-1: SL-based UE-to-UE relay for unicast SL

SL-based UE-to-UE relay may be used for providing or enhancing service continuity, power efficiency, or QoS for an ongoing unicast SL between Tx UE and Rx UE. For examples, SL-based UE to-UE relay may be used as a primary option for a possible recovery from a SL RLF, especially for out-of-coverage operation. SL-based UE-to-UE relay may be used as a replacement of the direct SL for a proactive avoidance of SL RLF or to reduce transmit power and therefore power consumption for Tx UE. SL-based UE-to-UE relay may also be used as a primary or a secondary connection, in addition to the direct SL, for providing or enhancing QoS for the unicast SL between Tx UE and Rx UE, as needed for high-reliability communications over SL for examples.

Proposal 1: RAN2 considers the use of SL-based UE-to-UE relay for providing or enhancing SL service continuity with SL RLF recovery, power efficiency or QoS for unicast SL.

SL-based UE-to-UE relay has not been supported so far in 3GPP specifications. The backward-compatibility issue in which Rx UE may be a legacy UE and therefore not capable of supporting SL-based UE-to-UE relay is anticipated. That is, the backward-compatible support of NR SL-based UE-to-UE relay means that NR SL-based UE-to-UE relay may be invisible to Rx UE. This is feasible because the SL is based on transmitter-oriented one-to-many broadcast principle, provided that Tx UE and R-UE are capable of supporting NR SL-based UE-to-UE relay. Therefore, the backward-compatible support of NR SL-based UE-to-UE relay may be considered.

Proposal 2: RAN2 considers the backward-compatibility issue with SL-based UE-to-UE relay for unicast SL in which the transmitting UE and the relay UE are supporting SL-based UE-to-UE relay and the receiving UE may be a pre-Rel-17 UE, which does not support SL-based UE-to-UE relay.

# 3	Conclusions

In this paper we have the following proposals:

Proposal 1: RAN2 considers the use of SL-based UE-to-UE relay for providing or enhancing SL service continuity with SL RLF recovery, power efficiency or QoS for unicast SL.

Proposal 2: RAN2 considers the backward-compatibility issue with SL-based UE-to-UE relay for unicast SL in which the transmitting UE and the relay UE are supporting SL-based UE-to-UE relay and the receiving UE may be a pre-Rel-17 UE, which does not support SL-based UE-to-UE relay.