3GPP TSG-RAN WG2 Meeting #111-e	R2-2006879

# Electronic, 17 – 28 August 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0261 | rev |  | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the support of NR-DC for PNI-NPN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo, Motorola Mobility |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RAN_PRN-Core |  |  |  |  |  | Date: |  |  | 2020-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | CR254r1 for introducing RAN3 input to NPN was approved in RP#88-e in RP-201182. In this context the following below was added in subclause 16.7.1 with regards to NR-DC support: NR-NR Dual Connectivity is supported within PNI-NPN and across PLMN and PNI-NPN. The latter part underlined means NR-DC between a PLMN cell and PNI-NPN cell. However, this use-case was neither discussed nor agreed in RAN2. Furthermore, it is not clear which NR-DC or bearer options shall or can be supported across PLMN cell and PNI-NPN cell: - NR-DC options: 1) PNI-NPN cell as MN, PLMN cell as SN; 2) PLMN cell as MN, PNI-NPN cell as SN. - Bearer options: 1) MCG bearer; 2) SCG bearer; 3) Split bearer. Furthermore, considering the fact that there is no requirement for NR-DC support across PLMN and PNI-NPN, and the unclarity of the value for supporting such use-case the concerned description should be better removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In subclause 16.7.1 the NR-DC support across PLMN and PNI-NPN has been removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear support of NR-DC across PLMN and PNI-NPN remains in stage 2 description. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

## 16.7	Public Network Integrated NPN

### 16.7.1	General

A PNI-NPN is a network deployed for non-public use which relies on network functions provided by a PLMN (see subclause 4.8). In PNI-NPN, a Closed Access Groups (CAG) identifies a group of subscribers who are permitted to access one or more CAG cells associated to the CAG. A CAG is identified by a CAG identifier broadcast in SIB1.

A CAG-capable UE can be configured with the following per PLMN (see clause 5.30.3.3 of TS 23.501 [3]):

-	an Allowed CAG list containing the CAG identifiers which the UE is allowed to access; and

-	a CAG-only indication if the UE is only allowed to access 5GS via CAG cells.

### NR-NR Dual Connectivity is supported within PNI-NPN.

End of changes