3GPP TSG-RAN WG2 #111-e	Tdoc R2-2006911

Electronic meeting, August 17th – 28th 2020

Agenda Item:	8.12.2.1

Source:	Ericsson

Title:	Framework and Principles for Reduced Capability

Document for:	Discussion, Decision

# 1	Introduction

In RAN#86, a new study item on the support of reduced capability NR devices for use cases such as industrial wireless sensors, video surveillance, and wearables was approved and then revised in RAN#88e [1]. One of the requirements for these three use cases, as described in [1], is lower device cost and complexity compared to high-end eMBB and URLLC devices of Rel-15/16. The SID includes the following objectives:

Study standardization framework and principles for how to define and constrain such reduced capabilities – considering definition of a limited set of one or more device types and considering how to ensure those device types are only used for the intended use cases [RAN2, RAN1].

Furthermore, the following relevant agreements have been done in RAN1#101e [5]:

Agreements:

For safety related sensors, latency requirements apply to traffic initiated from RRC_CONNECTED.

Agreements:

- For FR1, study at least 20MHz maximum UE bandwidth at least for initial access

  - Other bandwidths FFS

- For FR2, study 50MHz and 100 MHz maximum UE bandwidth at least for initial access

  - Other bandwidths FFS

In this contribution, we discuss potential solutions for these objectives.

# 2	Discussion

## 2.1	UE capabilities

One of the differences in NR compared to LTE is the absence of UE categories. In NR, UE is characterized only by a set of capabilities in order to separate the technological domain from the commercial domain. If a certain feature requires the UE to have a certain capability, the network does not rely on knowing if the UE is of a certain category, but only if the UE has such capability or not. Then, from the commercial point of view, a UE vendor implements a specific set of capabilities which define a certain chipset.

Unlike LTE, NR does not rely on UE categories to define a set of uplink and downlink capabilities that a certain UE supports.

In NR, the supported maximum data rate is one general discriminator of different chipset capabilities. The supported maximum data rate is computed according to the following formula in Section 4.1.2 of [3]:

J is the number of aggregated component carriers in a band or band combination, Rmax = 948/1024, Tsμ is the average OFDM symbol duration in a subframe for numerology μ, i.e. Tsμ=10-314⋅2μ, Qmj is the maximum supported modulation order, and OH(j) is the overhead in a certain FR and direction considered.

- This formula takes into account the number of MIMO layers (vLayers(j), related to the number of antennas) and the maximum RB allocation (NPRBBWj,μ, related to the maximum bandwidth).

- In particular, the scaling factor (f(j)) considers further capabilities that indirectly affect the overall data rate.

If we consider the aforementioned formula to represents a “device type”, the UEs implementing support for RedCap use cases may end up belonging to different device types due to the different requirements and resulting data rates and, consequently, degrees of complexity reductions to be defined.

The current definition of UE capabilities in NR allows a large degree of freedom in terms of configurations. They are divided in sub-sets depending on the cases in which they can vary within the same device. The capabilities can be defined:

- Per UE – there is only one configuration for the UE in all cases;

- Per Frequency Range – the UE can have different capabilities in FR1 and FR2;

- Per Duplexing mode – the UE can have different capabilities if TDD or FDD is used;

- Per Band – the UE can have different capabilities in each band (or band combination) to which is connected.

The number of combinations is virtually unlimited, although some minimum requirements are assumed in the standard. For eMBB and URLLC this might not be a problem, but for RedCap having many combinations could be a problem from the economy-of-scale point of view. In order to benefit from the economy of scale, a chipset with specific and reduced capabilities should be produced in very large volumes. Therefore, it can be beneficial if only a limited number of combinations are to be present in the market. Which capabilities to include is a trade-off between UE complexity and cost reduction and economy of scale.

Although these are strictly market-related strategies and discussions, 3GPP can facilitate the market development by defining specific device types with limited range of possible configurations.

3GPP could introduce a definition of a RedCap UE with certain capabilities (or certain combinations of capabilities) or the standard can allow any combination taking into account the minimum requirements and further leave it for markets to avoid the production of RedCap chipsets with multiple combinations of capabilities.

Restrictions in the number of UE capabilities combinations can be implemented in the standard or left up to the market.

## 2.2	Initial access

One of the most critical phases in data transmission is the initial access. Before the UE has been identified in Msg3 or Msg5, the gNB is not aware of its capabilities. In RAN1#101-e it was agreed that for FR1 a 20MHz maximum bandwidth, and for FR2 a 50MHz and 100MHz bandwidth should be studied for the initial access [5]. As discussed in [4], for FR1 it means that the UE is in principle able to perform the legacy initial access as CORESET#0 always has a bandwidth lower than the UE bandwidth. Although this topic is still under discussion, it is likely that a RedCap UE will be able to reuse the legacy initial access scheme. For FR2 there are some configurations where the CORESET#0 has a bandwidth exceeding 50MHz. In this case it might be necessary to design a new initial access scheme (new CORESET#0 design, and possibly extending SIB or MIB), to restrict the possible configurations if the cell supports RedCap UE or to review the UE bandwidth limit. Since the discussion on this topic is still ongoing in RAN1 for the sake of this discussion we focus only on FR1.

At least for FR1, the legacy initial access scheme can likely be used by RedCap UEs.

It may also be the case that special features, not present for the lowest complexity RedCap UEs and not relevant for the initial access, are required for some of the use cases considered in the RedCap SI, for instance higher MIMO capabilities for high end wearables.

## 2.3		Use case requirements and standardization framework

The three main use cases included in the study item and mentioned in the Introduction present very different requirements in terms of data rate, latency, and battery life. Also, for a given use case, it is possible to identify specific sub-cases with different requirements that could potentially lead to a need to support different capabilities. For instance, the Industrial Wireless Sensor Network (IWSN) use case is characterized by 100 ms latency requirement, apart from safety-related sensors, for which the latency should be 5-10 ms. Another example is the wearable use case that includes many different scenarios, from demanding services requiring a peak rate of 150 Mbps to infrequent sensor reporting of, e.g., heart rate.

Based on these considerations and what is discussed in [4], we summarized the use cases requirements in Table 1, together with legacy eMBB as a reference.

Table 1: Use case requirements and comparison to eMBB

|  | IWSN | IWSN - safety | Video surveillance | Low-end wearables | High-end wearables | eMBB |
| --- | --- | --- | --- | --- | --- | --- |
| Data rate | UL: < 2 Mbps | UL: < 2 Mbps | UL: 2 - 25 Mbps | UL: 2 Mbps DL: 5 Mbps | UL: peak bit rate up to 50 Mbps DL: peak bit rate up to 150 Mbps | < 2 Gbps |
| Latency | 100 ms | 5-10 ms (In CONNECTED) | 500 ms | n.a. | n.a. | CP: 10 ms UP: 4 ms |
| Battery Life | few years | n.a. | n.a. | 2 weeks | n.a. | < 1 day |
| Reliability (1-10-x) | x=4 | x=4 | x=2-3 | n.a. | n.a. | n.a. |

Depending on market considerations, considering the different range of requirements for the use cases of interest, we can imagine two possible ways of progressing in the normative phase.

It should be possible to find a set of capabilities that are sufficient to satisfy all the use cases at least until the UE is in RRC_CONNECTED; as mentioned before, some of the more demanding requirements are needed only while connected to the network. For this reason, all the RedCap UEs can be grouped in a single device type. If more advanced features need to be enabled during the connection, it is possible to reuse the existing UE capabilities framework to signal this beside the minimum set of capabilities of the device type. Moreover, by having only one device type, reaching economy of scale is facilitated as there will not be the need for many different designs tailored to a specific use case.

The existing UE capabilities framework can be reused to enable, beside the minimum set of capabilities RedCap UEs, more advanced features in RRC_CONNECTED.

It is possible that the definition of RedCap UEs will diverge in terms of features, capabilities and constraints regarding FR1 and FR2. For this reason, we believe two device types should be defined, one for each Frequency Range.

One set of requirements can cover most of the targeted use cases.

One RedCap device type is defined per frequency range, corresponding to minimum set of capabilities addressing the RedCap use cases.

As a final remark, the exact details of how to capture such device types in the specifications can be left for normative phase after the study item has completed. The study should focus on discussion of the possible number of different device types and whether there is need to (normatively) restrict the number of possible capability combinations, as discussed in Section 2.1.

The exact definition of device types and how they are captured in the standard is determined in the normative phase.

Study whether there is need for any restrictions on signaled additional optional UE capabilities for RedCap UEs on top of the device type.

## 2.4	Service Rejection

Once the UE capabilities are known to the network (after Msg3 or Msg5) it is possible for the network to configure the UE appropriately. One potential problem could be when a RedCap UE requests a service that does not match the reduced UE capabilities. This would be similar to if e.g. an NB-IoT UE requested a video call to be set up. RAN can already reject an RRC connection establishment attempt e.g. based on the establishment cause provided in Msg3 or through higher layer mechanisms. Further access attempts can be delayed if rejected and it can be discussed whether these mechanisms should be updated or extended.

If UE requires a service which is not intended for RedCap UEs, the request can be rejected. RAN2 should further study whether the existing mechanisms should be extended.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	Unlike LTE, NR does not rely on UE categories to define a set of uplink and downlink capabilities that a certain UE supports.

Observation 2	Restrictions in the number of UE capabilities combinations can be implemented in the standard or left up to the market.

Observation 3	At least for FR1, the legacy initial access scheme can likely be used by RedCap UEs.

Observation 4	The existing UE capabilities framework can be reused to enable, beside the minimum set of capabilities RedCap UEs, more advanced features in RRC_CONNECTED.

Observation 5	One set of requirements can cover most of the targeted use cases.

Observation 6	The exact definition of device types and how they are captured in the standard is determined in the normative phase.

Observation 7	If UE requires a service which is not intended for RedCap UEs, the request can be rejected. RAN2 should further study whether the existing mechanisms should be extended.

Based on the discussion in the previous sections we provided the following proposal:

Proposal 1	One RedCap device type is defined per frequency range, corresponding to minimum set of capabilities addressing the RedCap use cases.

Proposal 2	Study whether there is need for any restrictions on signaled additional optional UE capabilities for RedCap UEs on top of the device type.

# References

RP-201386, Revised SID on Study on support of reduced capability NR devices, Ericsson, RAN#88e, Online, June 2020.

TR 36.888, Machine-Type Communications (MTC) User Equipments (UEs) based on LTE, v12.0.0, June 2013.

TS 38.306, User Equipment (UE) radio access capabilities, v16.0.0, March 2020.

R1-2005234, Potential UE complexity reduction features for RedCap, Ericsson, R1#102e, August 2020.

RP-200821, Status Report for Study on support of reduced capacity NR devices, RAN#88e, Online, June 2020