3GPP TSG-RAN WG2 Meeting#111e	R2-2006925

Online, August 17th – 28th, 2020

Agenda Item:	8.10.3.1

Source:	Loon, Google

Title:	HAPS-Terrestrial PCI confusion mitigation

Document for:	Discussion and decision

# Introduction

At the RAN#86 meeting, upon successful completion of the “Study Item on NR support non-terrestrial network” [1], the corresponding work item was approved [2], with core part slated for completion by June 2021 (RAN#92), and the RAN4 performance part by December 2021 (RAN#94).

In this contribution, we provide our views regarding HAPS-Terrestrial PCI confusion mitigation.

# Discussion

The WID in [2] has called out the following specifically.

- Verify the applicability of existing Rel-16 ANR techniques to solve PCI confusion in order to support co-channel operation between HAPS terrestrial networks and develop enhancements if needed [RAN2/3]

HAPS can be used to facilitate connectivity to unserved and underserved areas in partnership with MNOs using the MNO’s licensed and operated frequencies. These services can also be used to provide emergency service in case of disasters. The primary differences between HAPs based deployment and terrestrial deployments is the height of the base station, and the fact that HAPS are slowly moving. The increased height of HAPS base stations leads to a larger coverage footprint. The figure below shows how terrestrial towers and HAP based coverage can coexist in the same region.

A typical terrestrial deployment has carefully planned PCI’s with none of them overlapping. However, these PCI’s can and are reused in different regions. For example, a PCI in region A can be reused in region C without any confusion for the terrestrial operator.

The geographical area served by a particular HAPS platform is subject to change because of movement, both controlled and uncontrolled. Note that some HAP platforms are susceptible to wind forces. For example, HAP-1 may move away from the region and HAP-2 may take over the role of providing service to the region below. In these scenarios’ handover from terrestrial to HAP platforms face some unique challenges which we will describe below.

We note that in typical deployment scenarios, a small set of PCIs are reserved for HAP usage.

## Terrestrial to HAP handover

Consider the situation where a HAP based platform has moved into a region for the first time.

A terrestrial gNB tower notices from its measurement reports that a HAP based PCI has higher RSRP than the terrestrial PCI. The terrestrial gNB is unaware that the PCI with better RSRP is a HAP based PCI. The gNB notices that this PCI does not have a corresponding ECGI mapping in the ANR table maintained by the gNB. The terrestrial tower asks the UE to query the ECGI from the HAP gNB. The gNB uses this ECGI value to forward data and the handover is completed. This PCI-ECGI mapping table is then stored in the terrestrial tower for future uses.

Now consider the situation when HAP-1 is replaced by HAP-2. HAP-2 may use the same PCI that was used by HAP-1. However, the ECGI of HAP-1 is different from the ECGI of HAP-2. The terrestrial gNB is unaware that its PCI-ECGI mapping table is now invalid. This causes handovers to fail.

Observation 1: In HAPS scenario, PCI confusion is quite likely.

One solution would be for the terrestrial gNB to always ask for new PCI-ECGI mapping in all of its terrestrial towers. But this is burdensome and impractical.

## HAP to terrestrial handover

A similar problem happens when the HAP attempts to handover to a terrestrial tower. Since the HAP footprint is large, it can cover multiple terrestrial regions. Since PCI’s are reused across terrestrial regions, a HAP can see multiple PCI’s with the same value. Again, this implies that the PCI-ECGI mapping table is not unique and cannot be used. This situation can be handled by implementation without any standards support because the HAP gNB’s are aware of the situation.

# Proposal

The key concept in the proposal is to be able to indicate to the terrestrial gNB that some gNB’s are mobile and thus the PCI-ECGI mapping for these PCI’s are not valid and thus should not be stored. We discuss three proposals here:

Option 1: SIB1 carries an indicator to notify that the cell is mobile.

This is also part of our other proposal ‘HAPS-Satellite ephemeris broadcast’. This allows the UE to realize that the PCI-ECGI mapping is unsuitable for long term storage. The UE then indicates this to the requesting terrestrial gNB

Option 2: Temporary PCI-ECGI mapping indicator.

Every time the UE queries for the PCI-ECGI mapping table the mobile gNB indicates to the UE that this mapping is unsuitable for long term storage. The UE reports this back to the terrestrial gNB allowing the terrestrial gNB to remove this mapping

Option 3: Core provides indication for temporary PCI-ECGI mapping

In this proposal, the core provides some indication to every terrestrial gNB which set of ECGI have constantly changing PCI-ECGI mapping. This would allow the gNB to remove these ECGI from its mapping table.

Option 1 is preferred by Loon as it also helps with initial access as discussed in our other proposal HAPS-Satellite ephemeris broadcast. Option 2 and 3 are workable but is considered as burdensome

Proposal: SIB1 carries an indicator to notify that the cell is mobile

# Conclusions

In this document, we discussed several aspects related to PCI confusion for HAP terrestrial coexistence. Our proposal is summarized below.

Proposal 1: SIB1 carried an indicator to notify that the cell is mobile.

# References

3GPP TR 38.821, Solutions for NR to support non-terrestrial networks (NTN)

RP-201256, Solutions for NR to support non-terrestrial networks (NTN)

R2-2006924, HAPS-Satellite ephemeris broadcast