3GPP TSG-RAN WG2 Meeting #111-e	R2-2006939

# Electronic meeting, 17 – 28 August 2020

Agenda item:		8.5.3

Source:	Intel Corporation

Title:	Uplink enhancements for URLLC in unlicensed controlled environments

Document for:		Discussion and Decision

# Introduction

According to Rel-17 NR IIoT WID RP-201310 [1], RAN2 will harmonize “UL configured-grant enhancements in NR-U and URLLC introduced in Rel-16” for “URLLC in unlicensed controlled environments”.

In this contribution, we discuss how to handle the overlapping features for configured grant defined in Rel-16 IIoT and NR-u.

# Discussion

## Autonomous transmission for configured grant

In Rel-16, autonomous transmission for CG was introduced in NR-u due to LBT, and is always enabled. Autonomous transmission for CG was also introduced in IIoT for deprioritized CG whose MAC PDU has already been obtained, and is configured by RRC parameter autonomousTx. The question for Rel-17 URLLC in shared spectrum is whether autonomousTx parameter should be configured, i.e. whether autonomous transmission for deprioritized CG can be handled by NR-u mechanism without explicit RRC configuration.

In Rel-16 NR-u, a HARQ process is considered pending according to TS 38.321 clause 5.4.2.2 below. Autonomous retransmission can be performed for pending HARQ process if cg-RetransmissionTimer is not running.

When cg-RetransmissionTimer is configured and the HARQ entity obtains a MAC PDU to transmit, the corresponding HARQ process is considered to be pending. For a configured uplink grant, configured with cg-RetransmissionTimer, each associated HARQ process is considered as not pending when: - a transmission is performed on that HARQ process and LBT failure indication is not received from lower layers; or - the configured uplink grant is initialised and this HARQ process is not associated with another active configured uplink grant; or - the HARQ buffer for this HARQ process is flushed.

In Rel-16 IIoT, autonomous transmission (if configured with autonomousTx) is performed for deprioritized CG whose MAC PDU has already been obtained, as in TS 38.321 clause 5.4.2.1 below:

3> else if this uplink grant is a configured grant configured with autonomousTx; and 3> if the previous configured uplink grant, in the BWP, for this HARQ process was de-prioritized; and 3> if a MAC PDU had already been obtained for this HARQ process; and 3> if the uplink grant size matches with size of the obtained MAC PDU; and 3> if a transmission of the obtained MAC PDU has not been performed: 4> consider the MAC PDU has been obtained.

Autonomous transmission in IIoT is due to intra-UE prioritization, and the corresponding HARQ process is considered pending in NR-u. Therefore, autonomous transmission in IIoT is a subset of autonomous transmission in NR-u.

Observation 1: Autonomous retransmission of CG in NR-u is supported and enabled upon the condition of cg-RetransmissionTimer which is configured for shared spectrum only.

Observation 2: Autonomous retransmission of CG in IIoT is supported for deprioritized CG when autonomousTx is configured.

Observation 3: Autonomous transmission of CG in IIoT in NR-u can be enabled regardless of whether autonomousTx is configured or not.

Just to avoid potential confusion in configuration, it is proposed to only use the autonomous transmission defined in NR-u in Rel-17 URLLC in shared spectrum, i.e. autonomousTx should not be configured in shared spectrum. This can be implemented by adding a restriction to the field description of autonomousTx, e.g. as below:

autonomousTx If this field is present, the Configured Grant configuration is configured with autonomous transmission, see TS 38.321 [3]. This field is not configured for operation with shared spectrum channel access.

Proposal 1: For Rel-17 URLLC in shared spectrum, autonomous transmission follows what is defined for Rel-16 NR-u operation, and autonomousTx field defined for Rel-16 IIoT WI shall not be configured.

## HARQ process ID deriviation

In Rel-16 IIoT, HARQ process ID for initial transmission is linked to time domain resource, and controlled by RRC parameter harq-ProcID-Offset2, as from TS 38.321 clause 5.4.1 below. The motivation to introduce harq-ProcID-Offset2 is to avoid HARQ process ID collisions between different CG configurations.

For configured uplink grants with harq-ProcID-Offset2, the HARQ Process ID associated with the first symbol of a UL transmission is derived from the following equation: HARQ Process ID = [floor(CURRENT_symbol / periodicity)] modulo nrofHARQ-Processes + harq-ProcID-Offset2

In Rel-16 NR-u, HARQ process ID selection is up to UE implementation based on RRC parameter harq-ProcID-Offset, as from TS 38.321 clause 5.4.1:

For configured uplink grants configured with cg-RetransmissionTimer, the UE implementation select an HARQ Process ID among the HARQ process IDs available for the configured grant configuration.

and the field description of harq-ProcID-Offset:

harq-ProcID-Offset For operation with shared spectrum channel access, this configures the range of HARQ process IDs which can be used for this configured grant where the UE can select a HARQ process ID within [harq-procID-offset, .., (harq-procID-offset + nrofHARQ-Processes – 1)].

The selected HARQ process ID is signalled explicitly in physical layer CG-UCI, as in TS 38.212 clause 6.3.2.1.3.

Observation 4: HARQ process ID derivation methods for Rel-16 IIoT and NR-u are not compatible with each other, and one scheme among the two should be selected for Rel-17 URLLC operation in shared spectrum.

Following is the field description for cg-RetransmissionTimer, and it can be seen that cg-RetransmissionTimer is always configured in NR-u, and is not configured simultaneously with harq-ProcId-Offset2.

cg-RetransmissionTimer Indicates the initial value of the configured retransmission timer (see TS 38.321 [3]) in multiples of periodicity. The value of cg-RetransmissionTimer is always less than the value of configuredGrantTimer. This field is always configured for operation with shared spectrum channel access together with harq-ProcID-Offset. This field is not configured for operation in licensed spectrum or simultaneously with harq-ProcID-Offset2.

For Rel-17 URLLC in unlicensed controlled environments, it is expected that HARQ process ID cannot be linked to time domain resource as it was done in Rel-16 IIoT due to the need of overcome LBT in share spectrum. Therefore, it is proposed to use Rel-16 NR-u approach to handle HARQ process ID derivation, i.e. HARQ process ID selection is up to UE implementation based on configured harq-ProcID-Offset. Therefore harq-ProcID-Offset2 cannot be configured for Rel-17 URLLC in shared spectrum.

Given that the field description of cg-RetransmissionTimer already excludes the simultaneous configuration of cg-RetransmissionTimer with harq-ProcID-Offset2, there is no specification impact.

Proposal 2: For Rel-17 URLLC in shared spectrum, HARQ process ID derivation follows what is defined for Rel-16 NR-u operation (i.e. HARQ process ID selection is up to UE implementation based on configured harq-ProcId-Offset). There is no specification impact.

# Conclusion

In this contribution, we discuss how to handle the overlapping features for configured grant defined in Rel-16 IIoT and NR-u. We have the following observations:

Observation 1: Autonomous retransmission of CG in NR-u is supported and enabled upon the condition of cg-RetransmissionTimer which is configured for shared spectrum only.

Observation 2: Autonomous retransmission of CG in IIoT is supported for deprioritized CG when autonomousTx is configured.

Observation 3: Autonomous transmission of CG in IIoT in NR-u can be enabled regardless of whether autonomousTx is configured or not.

Observation 4: HARQ process ID derivation methods for Rel-16 IIoT and NR-u are not compatible with each other, and one scheme among the two should be selected for Rel-17 URLLC operation in shared spectrum.

We propose the following:

Proposal 1: For Rel-17 URLLC in shared spectrum, autonomous transmission follows what is defined for Rel-16 NR-u operation, and autonomousTx field defined for Rel-16 IIoT WI shall not be configured.

Proposal 2: For Rel-17 URLLC in shared spectrum, HARQ process ID derivation follows what is defined for Rel-16 NR-u operation (i.e. HARQ process ID selection is up to UE implementation based on configured harq-ProcId-Offset). There is no specification impact.

# References

[1] RP-201310, Nokia et al, "Revised WID: Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR"