3GPP TSG-RAN WG2 Meeting #111 electronic	                                                               R2-2006959

Online, August 17th - 28th, 2020

Agenda Item:	6.2.5

Source:	AT&T

Title:	Remaining details of UE capabilities for IAB

Document for:	Discussion/Approval

# Introduction

This contribution discusses the remaining details of the definition of IAB-MT features.

# Mandatory IAB-MT Features

During RAN#88e the following agreements were reached:

T2-P2’’: The following Rel-15 Layer-2 and Layer-3 UE Feature is mandatory with capability signaling for wide-area and local-area IAB-MTs (it is up to the IAB node to set the capability bit, and the feature will not be captured into the minimum set table specified in TS 38.306):

4-1          Intra-NR measurements and reports

The following Rel-15 Layer-2 and Layer-3 UE Feature is optional with capability signaling for wide-area and local-area IAB-MTs:

7-1          Handover:

1) Intra-frequency HO

2) Inter-frequency HO

For Feature 4-1, a similar parameter is defined for NE-DC and (NG)EN-DC, and this can be used as the template for IAB-MT signaling:

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| intraAndInterF-MeasAndReport Indicates whether the UE supports NR intra-frequency and inter-frequency measurements and at least periodical reporting. This field only applies to NE-DC and SN configured measurement when (NG)EN-DC is configured. For NR MCG, this feature is mandatory supported. | UE | Yes | Yes | No |

Proposal 1: Add the following parameter for Feature 4-1 in 38.306:

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| intraAndInterF-MeasAndReport-IAB-r16 Indicates whether the IAB-MT supports NR intra-frequency and inter-frequency measurements and at least periodical reporting. Note: It is up to the IAB node to set the capability bit | UE | Yes | Yes | No |

Additionally, Feature 7-1, component 1) has already been captured in the latest version of 38.306 with optional capability signaling for IAB-MTs, however component 2) needs to be added:

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| handoverIntraF-IAB-r16 Indicates whether the IAB-MT supports intra-frequency HO. It indicates the support for intra-frequency HO from the corresponding duplex mode if this capability is included in fdd-Add-UE-NR-Capabilities or tdd-Add-UE-NR-Capabilities. It indicates the support for intra-frequency HO in the corresponding frequency range if this capability is included in fr1-Add-UE-NR-Capabilities or fr2-Add-UE-NR-Capabilities. | IAB-MT | No | Yes | Yes |

Proposal 2: Add the following parameter for Feature 7-1, component 2) in 38.306:

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| handoverInterF-IAB-r16 Indicates whether the IAB-MT supports inter-frequency HO. It indicates the support for inter-frequency HO from the corresponding duplex mode if this capability is included in fdd-Add-UE-NR-Capabilities or tdd-Add-UE-NR-Capabilities. It indicates the support for inter-frequency HO in the corresponding frequency range if this capability is included in fr1-Add-UE-NR-Capabilities or fr2-Add-UE-NR-Capabilities. | IAB-MT | No | Yes | Yes |

# Conclusion

The following proposals were made:

Proposal 1: Add the following parameter for Feature 4-1 in 38.306:

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| intraAndInterF-MeasAndReport-IAB-r16 Indicates whether the IAB-MT supports NR intra-frequency and inter-frequency measurements and at least periodical reporting. Note: It is up to the IAB node to set the capability bit | UE | Yes | Yes | No |

Proposal 2: Add the following parameter for Feature 7-1, component 2) in 38.306:

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| handoverInterF-IAB-r16 Indicates whether the IAB-MT supports inter-frequency HO. It indicates the support for inter-frequency HO from the corresponding duplex mode if this capability is included in fdd-Add-UE-NR-Capabilities or tdd-Add-UE-NR-Capabilities. It indicates the support for inter-frequency HO in the corresponding frequency range if this capability is included in fr1-Add-UE-NR-Capabilities or fr2-Add-UE-NR-Capabilities. | IAB-MT | No | Yes | Yes |