3GPP TSG-RAN WG2 Meeting #111 electronic	R2-2007006

# Online, August 17th - 28th, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 1768 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the Configuration of sCellState for 38.331 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-Core |  |  |  |  |  | Date: |  |  | 2020-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In current specification, the sCellState can be configured in case of SCell addition, reconfiguration with sync, and resuming an RRC connection. It is absent otherwise. In order to specify this scenarios for the SCell modficiation case, a clause is added in last meeting as following: “if the sCellToAddModList was received in an RRCReconfiguration message including reconfigurationWithSync, in an RRCResume message or in an E-UTRA RRCConnectionResume message” However “in an E-UTRA RRCConnectionResume message” is redundant. The NR sCellToAddModList is not included in an E-UTRA RRCConnectionResume message directly. The NR sCellToAddModList is included in an NR RRCReconfiguration message which is embedded in the E-UTRAN RRCConnectionResume message for (NG) EN-DC case with resuming NR SCG. And it was agreed for SCG resuming during the RRC resume procedure, the secondaryCellGroup with at least reconfigurationWithSync should be included.“if the sCellToAddModList was received in an RRCReconfiguration message including reconfigurationWithSync” can cover RRC resume with SCG resume for NE-DC and NR-DC. The case of “sCellToAddModList was received in an RRCReconfiguration message including reconfigurationWithSync” can include the cases of NR PCell change, NR PSCell change, and NR SCG resume during RRC resume for NR-DC and (NG) EN-DC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Delete the “or in an E-UTRA RRCConnectionResume message” in 5.3.5.5.9; Impact analysis Impacted 5G architecture options: (NG) EN-DC Impacted functionality: RRC resume with SCG configuration Inter-operability: - If the network is implemented according to the CR and the UE is not, there is no iter-operability problem, due to the NR sCellToAddModList can’t be included in an E-UTRA RRCConnectionResume message directly. - If the UE is implemented according to the CR and the network is not, there is no iter-operability problem, due to the NR sCellToAddModList can’t be included in an E-UTRA RRCConnectionResume message directly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NR sCellToAddModList is not included in an E-UTRA RRCConnectionResume message directly, the case is included by “the sCellToAddModList was received in an RRCReconfiguration message including reconfigurationWithSync”, so the “in an E-UTRA RRCConnectionResume message” is useless. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.5.5.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

5.3.5.5.9	SCell Addition/Modification

The UE shall:

1>	for each sCellIndex value included in the sCellToAddModList that is not part of the current UE configuration (SCell addition):

2>	add the SCell, corresponding to the sCellIndex, in accordance with the sCellConfigCommon and sCellConfigDedicated;

2>	if the sCellState is included:

3>	configure lower layers to consider the SCell to be in activated state;

2>	else:

3>	configure lower layers to consider the SCell to be in deactivated state;

2>	for each measId included in the measIdList within VarMeasConfig:

3>	if SCells are not applicable for the associated measurement; and

3>	if the concerned SCell is included in cellsTriggeredList defined within the VarMeasReportList for this measId:

4>	remove the concerned SCell from cellsTriggeredList defined within the VarMeasReportList for this measId;

1>	for each sCellIndex value included in the sCellToAddModList that is part of the current UE configuration (SCell modification):

2>	modify the SCell configuration in accordance with the sCellConfigDedicated;

2>	if the sCellToAddModList was received in an RRCReconfiguration message including reconfigurationWithSync, in an RRCResume message:

3>	if the sCellState is included:

4>	configure lower layers to consider the SCell to be in activated state;

3>	else:

4>	configure lower layers to consider the SCell to be in deactivated state.

END OF CHANGE