3GPP TSG-RAN WG2 Meeting #111 electronic	R2-2007007

# Online, August 17th - 28th, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 4366 | rev | - | Current version: | 16.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the Configuration of sCellState for 36.331 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-Core |  |  |  |  |  | Date: |  |  | 2020-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In current specification, the sCellState can only be configured upon handover, SCell addition, E-UTRA RRC resume, and NR RRC resume: If the RRCConnectionReconfiguration message includes the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall: … 1> for each SCell configured for the UE other than the PSCell: 2> if the received RRCConnectionReconfiguration message includes sCellState for the SCell and indicates activated: 3> configure lower layers to consider the SCell to be in activated state; 2> else if the received RRCConnectionReconfiguration message includes sCellState for the SCell and indicates dormant: 3> configure lower layers to consider the SCell to be in dormant state; 2> else: 3> configure lower layers to consider the SCell to be in deactivated state;. And 5.3.10.3b SCell addition/ modification The UE shall: 1> for each sCellIndex value included either in the sCellToAddModList or in the sCellToAddModListSCG that is not part of the current UE configuration (SCell addition): … 2> if sCellState is configured for the SCell and indicates activated: 3> configure lower layers to consider the SCell to be in activated state; 2> else if sCellState is configured for the SCell and indicates dormant: 3> configure lower layers to consider the SCell to be in dormant state; 2> else: 3> configure lower layers to consider the SCell to be in deactivated state; … 1> for each sCellIndex value included either in the sCellToAddModList or in the sCellToAddModListSCG that is part of the current UE configuration (SCell modification): … 2> if the sCellToAddModList was received within an RRCConnectionResume or an NR RRCResume message: 3> if the sCellState is configured for the SCell and indicates activated: 4> configure lower layers to consider the SCell to be in activated state; 3> else if sCellState is configured for the SCell and indicates dormant: 4> configure lower layers to consider the SCell to be in dormant state; 3> else: 4> configure lower layers to consider the SCell to be in deactivated state;. However, in NR the SCell state can be configured upon PSCell change. Therefore for NE-DC, the SCell state should also be configured upon PSCell change, which algins with NR-DC and (NG)EN-DC. Furthermore, the E-UTRA sCellToAddModList can’t be configured in NR RRCResume message directly. It should be the sCellToAddModListSCG included in an E-UTRA RRCConnectionReconfiguration message which embedded in NR RRCResume message. |
| Summary of change: |  | - add the case of the sCellToAddModListSCG included in E-UTRA RRCConnectionReconfiguration message which included in NR RRCResume message or NR RRCReconfiguration message to support SCell state configuration for NE-DC RRC resume and PSCell change; Impact analysis Impacted 5G architecture options: NE-DC Impacted functionality: RRC resume with SCG configuration, PSCell change Inter-operability: - If the network is implemented according to the CR and the UE is not, the UE may assume the configuration is an error when the NW includes the sCellToAddModListSCG inculding the sCellState included in E-UTRA RRCConnectionReconfiguration message with mobilitControlInfoSCG which included in NR RRCReconfiguration message. - If the UE is implemented according to the CR and the network is not, there is no inter-operability problem, E-UTRA sCellToAddModList can’t be included in an NR RRCResume message directly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | the SCG SCell state can’t be configured upon NE-DC PSCell change, and the NE-DC RRC resume procedure |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.10.3b |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

#### 5.3.10.3b	SCell addition/ modification

The UE shall:

1>	for each sCellIndex value included either in the sCellToAddModList or in the sCellToAddModListSCG that is not part of the current UE configuration (SCell addition):

2>	add the SCell, corresponding to the cellIdentification, in accordance with the radioResourceConfigCommonSCell and radioResourceConfigDedicatedSCell, both included either in the sCellToAddModList or in the sCellToAddModListSCG;

2>	if sCellState is configured for the SCell and indicates activated:

3>	configure lower layers to consider the SCell to be in activated state;

2>	else if sCellState is configured for the SCell and indicates dormant:

3>	configure lower layers to consider the SCell to be in dormant state;

2>	else:

3>	configure lower layers to consider the SCell to be in deactivated state;

2>	for each measId included in the measIdList within VarMeasConfig:

3>	if SCells are not applicable for the associated measurement; and

3>	if the concerned SCell is included in cellsTriggeredList defined within the VarMeasReportList for this measId:

4>	remove the concerned SCell from cellsTriggeredList defined within the VarMeasReportList for this measId;

1>	for each sCellIndex value included either in the sCellToAddModList or in the sCellToAddModListSCG that is part of the current UE configuration (SCell modification):

2>	modify the SCell configuration in accordance with the radioResourceConfigDedicatedSCell, included either in the sCellToAddModList or in the sCellToAddModListSCG;

2>	if the sCellToAddModList was received within an RRCConnectionResume or sCellToAddModListSCG was received within RRCConnectionReconfiguration with mobilityControlInfoSCG embedded in an NR RRCResume or embedded in an NR RRCReconfiguration message:

3>	if the sCellState is configured for the SCell and indicates activated:

4>	configure lower layers to consider the SCell to be in activated state;

3>	else if sCellState is configured for the SCell and indicates dormant:

4>	configure lower layers to consider the SCell to be in dormant state;

3>	else:

4>	configure lower layers to consider the SCell to be in deactivated state;

END OF CHANGE