3GPP TSG-RAN2 Meeting # 111 Electronic	R2- 2007074

# 17th August - 28th August 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 1778 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to discarding segments of SIB 12 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2020-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Release 16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to current procedure, SIB 12 segments are discarded in following cases: - Upon receiving SIB12, if the UE has stored at least one segment of SIB12 and the value tag of SIB12 has changed since a previous segment was stored: UE discards all stored segments. - The UE should discard any stored segments for SIB12 if the complete SIB12 has not been assembled within a period of 3 hours. Scenario: - UE is camped on Cell A - UE receives a segment of SIB 12 and stores the segment - UE perform cell reselection to Cell B Case 1: SIB 12 is cell specific. - In this case, irrespective of whether the valuetag used in Cell A and Cell B is same or different, contents of SIB12 in Cell A and Cell B can be different. UE can not know whether contents of SIB12 are same or not. Case 2: SIB 12 is area specific - 2-1: Cell A and Cell B belongs to same SI Area - In this case if value tag used in Cell A and Cell B is not same, contents of SIB12 broadcasted by Cell A and Cell B are different. - In this case if value tag used in Cell A and Cell B is same, contents of SIB12 broadcasted by Cell A and Cell B is same. However, number of segments and content of each segment may or may not be same. - 2-2: Cell A and Cell B belongs to different SI Area - In this case, irrespective of whether the valuetag used in Cell A and Cell B is same or different, contents of SIB12 in Cell A and Cell B can be different. UE can not know whether contents of SIB12 are same or not. In this scenario explained above, SIB 12 contents in cell A and cell B can be same or different. Even if the SIB 12 contents is same, content of each segment in cell A and cell B can be different. So, stored segments should be discarded upon cell (re-)selection. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The UE shall discard any stored segments for SIB 12 upon cell (re-) selection. Impacted functionality: SIB 12 Acquisition Inter-operability: - If the network is implemented according to the CR and the UE is not, there are no interoperability issues. - If the UE is implemented according to the CR and the network is not, there are no interoperability issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE may combine incorrect SIB 12 segments. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.4.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### <Start of Change>

##### 5.2.2.4.13	Actions upon reception of SIB12

Upon receiving SIB12, the UE shall:

1>	if the UE has stored at least one segment of SIB12 and the value tag of SIB12 has changed since a previous segment was stored:

2>	discard all stored segments;

1>	store the segment;

1>	if all segments have been received:

2>	assemble SIB12-IEs from the received segments;

2>	if sl-FreqInfoList is included in sl-ConfigCommonNR:

3>	if configured to receive NR sidelink communication:

4>	use the resource pool(s) indicated by sl-RxPool for NR sidelink communication reception, as specified in 5.8.7;

3>	if configured to transmit NR sidelink communication:

4>	use the resource pool(s) indicated by sl-TxPoolSelectedNormal, or sl-TxPoolExceptional for NR sidelink communication transmission, as specified in 5.8.8;

4>	perform CBR measurement on the transmission resource pool(s) indicated by sl-TxPoolSelectedNormal and sl-TxPoolExceptional for NR sidelink communication transmission, as specified in 5.5.3.1;

34>	use the synchronization configuration parameters for NR sidelink communication on frequencies included in sl-FreqInfoList, as specified in 5.8.5;

2>	if sl-RadioBearerConfigList or sl-RLC-BearerConfigList is included in sl-ConfigCommonNR:

3>	perform sidelink DRB reconfiguration as specified in 5.8.9.1a;

2> if sl-MeasConfigCommon is included in sl-ConfigCommonNR:

3> store the NR sidelink measurement configuration.

The UE should discard any stored segments for SIB12 if the complete SIB12 has not been assembled within a period of 3 hours. The UE shall discard any stored segments for SIB12 upon cell (re-) selection.

#### <End of Change>