3GPP TSG-RAN WG2 Meeting #110-e	R2-2007090

# Electronic, 1-12 June 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 0797 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on SL-SCH MAC header |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple, InterDigital Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2020-08-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RAN2#109bis has agreed that “Working assumption for the CR: The V field is supported in a SL-SCH MAC subheader at least for future extensibility.” Hence, V field is only included for forward-compatibility in case there is a different version of MAC header introduced in later release(s). There is no agreement to use V field to indicate cast type of SL communication in the MAC header. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed the incorrect description of V field usage for the cast type indication of the associated address represented in DST field. Impact analysis Impacted architecture options: NR SL Impacted functionality: SL-SCH header Inter-operability: No inter-operability impacts are foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect normative requirments will remain in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

### 6.2.4	MAC subheader for SL-SCH

The MAC subheader consists of the following fields:

-	V: The MAC PDU format version number field indicates which version of the SL-SCH subheader is used. The V field size is 4 bits;

-	SRC: The SRC field carries the 16 most significant bits of the Source Layer-2 ID field set to the identifier provided by upper layers as defined in TS 23.287 [19]. The length of the field is 16 bits;

-	DST: The DST field carries the 8 most significant bits of the Destination Layer-2 ID set to the identifier provided by upper layers as defined in TS 23.287 [19]. The length of the field is 8 bits;

-	LCID: The Logical Channel ID field identifies the logical channel instance or the type of the corresponding MAC CE within the scope of one Source Layer-2 ID and Destination Layer-2 ID pair of the corresponding MAC SDU or padding as described in Tables 6.2.4-1 for SL-SCH. There is one LCID field per MAC subheader except for SL-SCH subheader. The LCID field size is 6 bits;

-	L: The Length field indicates the length of the corresponding MAC SDU in bytes. There is one L field per MAC subheader except for subheaders corresponding to the SL-SCH subheader or padding. The size of the L field is indicated by the F field;

-	F: The Format field indicates the size of the Length field. There is one F field per MAC subheader except for subheaders corresponding to the SL-SCH subheader or padding. The size of the F field is 1 bit. The value 0 indicates 8 bits of the Length field. The value 1 indicates 16 bits of the Length field;

-	R: Reserved bit, set to 0.

The MAC subheader is octet aligned.

Table 6.2.4-1 Values of LCID for SL-SCH

| Index | LCID values |
| --- | --- |
| 0 | SCCH carrying PC5-S messages that are not protected |
| 1 | SCCH carrying PC5-S messages "Direct Security Mode Command" and "Direct Security Mode Complete" |
| 2 | SCCH carrying other PC5-S messages that are protected |
| 3 | SCCH carrying PC5-RRC messages |
| 4–19 | Identity of the logical channel |
| 20–61 | Reserved |
| 62 | Sidelink CSI Reporting |
| 63 | Padding |

Next Change