3GPP TSG-RAN WG2 Meeting #111-e	R2-2007093

# Electronic, 17-28 August 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 0800 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Sidelink LCP procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2020-07-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the description of selection of SL destinations, the NOTE does not cover the case when a destination having LCH priority 1 is compared to a destination with SL CSI report MAC CE. This shall be also left to UE implementation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Improved the NOTE to cover the following case: SL destination(s) which have traffic of highest of priority of logical channels vs. SL destinations which has SL CSI reporting MAC CE.The selection of destination is up to UE implementation Impact analysis Impacted architecture options: NR SL Impacted functionality: Logical Channel Prioritization Inter-operability: No inter-operability impacts are foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of clear guide for UE implementation of destination selection among destinations of equal priority. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.22.1.4.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

###### 5.22.1.4.1.2	Selection of logical channels

The MAC entity shall for each SCI corresponding to a new transmission:

1>	select a Destination associated to one of unicast, groupcast and broadcast, having the logical channel with the highest priority or the MAC CE, among the logical channels that satisfy all the following conditions and MAC CE(s), if any, for the SL grant associated to the SCI:

2>	SL data is available for transmission; and

2>	SBj > 0, in case there is any logical channel having SBj > 0; and

2>	sl-configuredSLGrantType1Allowed, if configured, is set to true in case the SL grant is a Configured Grant Type 1; and

2>	sl-allowedCG-List, if configured, includes the configured grant index associated to the SL grant; and

2>	sl-HARQ-FeedbackEnabled set to disabled, if PSFCH is not configured for the SL grant associated to the SCI.

NOTE:	If multiple Destinations have the logical channels satisfying all conditions above with the same highest priority or if multiple Destinations have the MAC CE, or between destination(s) have the logical channels with the same highest priority as MAC CE and destination(s) having the MAC CE, which Destination is selected among them is up to UE implementation.

1>	select the logical channels satisfying all the following conditions among the logical channels belonging to the selected Destination:

2>	SL data is available for transmission; and

2>	sl-configuredSLGrantType1Allowed, if configured, is set to true in case the SL grant is a Configured Grant Type 1; and.

2>	sl-allowedCG-List, if configured, includes the configured grant index associated to the SL grant; and

2>	if the MAC entity has been configured with Sidelink resource allocation mode 1 and PSFCH is configured for the sidelink grant associated to the SCI:

3>	sl-HARQ-FeedbackEnabled is set to enabled, if sl-HARQ-FeedbackEnabled is set to enabled for the highest priority logical channel satisfying the above conditions; or

3>	sl-HARQ-FeedbackEnabled set to disabled, if sl-HARQ-FeedbackEnabled is set to disabled for the highest priority logical channel satisfying the above conditions.

2>	else:

3>	sl-HARQ-FeedbackEnabled set to disabled.

End of Changes