3GPP TSG-RAN WG2#111-e Meeting	Tdoc  R2-2007229

# Online, 17th- 28th August 2020

Agenda Item:		6.7.2

Souce:	Samsung

Title:	Internode signalling upon reconfiguration of source PCell

Document for:		Discussion and decision

# Introduction

This document discusses the internode signalling for the following case

- @T1 the source node initiates preparation of a CHO candidate target #1 by using the RAN# HANDOVER REQUEST (ACKNOWLEDGE) messages, including RRC inter-node messages (HandoverPreparationInformation and HandoverComplete)

- @T2 the source node initiates preparation of a CHO candidate target #2, again using the same Xx and RRC INM

- @T3 the source node initiates some essential change of the source PCell configuration. The source node contacts the node(s) controlling the CHO candidate target, again using the RAN# HANDOVER REQUEST (ACKNOWLEDGE) messages

  - The node(s) controlling the CHO candidate target(s) may need to provide an updated CHO configuration for candidate target #1 and #2, as source PCell configuration is baseline for the configuration of the CHO candidate

This contribution we are primarily discussing the following aspects:

- Whether source can determine if target CHO configuration will be affected by a change of PSCell configuration

- Whether it is acceptable to have signalling to the UE in case CHO configuration remains same (e.g. is not affected by a PSCell reconfiguration) and if not, how to achieve this

And we propose:

- Source node always initiates HO preparation towards node controlling candidate target PCell

- Node controlling candidate target PCell returns handoverCommandMessage octet string of 0 size if CHO configuration does not change

# Discussion

## Current standards status

In this section we try to clarify the current status in standards

HANDOVER REQUEST: We understand RAN3 agreed to include some CHO related changes. For the issue discussed in this paper, the main relevant change concerns addition of field CHO trigger that indicates whether the CHO information concerns a CHO initiation or a CHO replacement, see extract from 38.323 v16.2.0 below. It is further important to note that RRC INM HandoverPreparationInformation is carried within the mandatory field RRC Context.

#### 9.1.1.1	HANDOVER REQUEST

This message is sent by the source NG-RAN node to the target NG-RAN node to request the preparation of resources for a handover.

Direction: source NG-RAN node  target NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | reject |
| Target Cell Global ID | M |  | 9.2.3.25 | Includes either an E-UTRA CGI or an NR CGI | YES | reject |
| GUAMI | M |  | 9.2.3.24 |  | YES | reject |
| UE Context Information |  | 1 |  |  | YES | reject |
| >NG-C UE associated Signalling reference | M |  | AMF UE NGAP ID 9.2.3.26 | Allocated at the AMF on the source NG-C connection. | – |  |
| >Signalling TNL association address at source NG-C side | M |  | CP Transport Layer Information 9.2.3.31 | This IE indicates the AMF’s IP address of the SCTP association used at the source NG-C interface instance. Note: If no UE TNLA binding exists at the source NG-RAN node, the source NG-RAN node indicates the TNL association address it would have selected if it would have had to create a UE TNLA binding. | – |  |
| >UE Security Capabilities | M |  | 9.2.3.49 |  | – |  |
| >AS Security Information | M |  | 9.2.3.50 |  | – |  |
| >Index to RAT/Frequency Selection Priority | O |  | 9.2.3.23 |  | – |  |
| >UE Aggregate Maximum Bit Rate | M |  | 9.2.3.17 |  | – |  |
| >PDU Session Resources To Be Setup List |  | 1 | 9.2.1.1 | Similar to NG-C signalling, containing UL tunnel information per PDU Session Resource; and in addition, the source side QoS flow DRB mapping | – |  |
| >RRC Context | M |  | OCTET STRING | Either includes the HandoverPreparationInformation message as defined in subclause 10.2.2. of TS 36.331 [14], or the HandoverPreparationInformation-NB message as defined in subclause 10.6.2 of TS 36.331 [14], if the target NG-RAN node is an ng-eNB, or the HandoverPreparationInformation message as defined in subclause 11.2.2 of TS 38.331 [10], if the target NG-RAN node is a gNB. | – |  |
| >Location Reporting Information | O |  | 9.2.3.47 | Includes the necessary parameters for location reporting. | – |  |
| >Mobility Restriction List | O |  | 9.2.3.53 |  | – |  |
| >Management Based MDT PLMN List | O |  | MDT PLMN List 9.2.3.133 |  | YES | ignore |
| >5GC Mobility Restriction List Container | O |  | 9.2.3.100 |  | YES | ignore |
| > NR UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.3.107 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| > LTE UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.3.108 | This IE applies only if the UE is authorized for LTE V2X services. | YES | ignore |
| >UE Radio Capability ID | O |  | 9.2.3.138 |  | YES | reject |
| Trace Activation | O |  | 9.2.3.55 |  | YES | ignore |
| Masked IMEISV | O |  | 9.2.3.32 |  | YES | ignore |
| UE History Information | M |  | 9.2.3.64 |  | YES | ignore |
| UE Context Reference at the S-NG-RAN node | O |  |  |  | YES | ignore |
| >Global NG-RAN Node ID | M |  | 9.2.2.3 |  | – |  |
| >S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | – |  |
| Conditional Handover Information | O |  |  |  | YES | reject |
| >CHO Trigger | M |  | ENUMERATED (CHO-initiation, CHO-replace, …) |  |  |  |
| >Target NG-RAN node UE XnAP ID | C-ifCHOmod |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node |  |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  |  |  |
| NR V2X Services Authorized | O |  | 9.2.3.105 |  | YES | ignore |
| LTE V2X Services Authorized | O |  | 9.2.3.106 |  | YES | ignore |
| PC5 QoS Parameters | O |  | 9.2.3.109 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to the handover; the source NG-RAN node provides it in order to enable later analysis of the conditions that led to a wrong HO. | YES | ignore |
| UE History Information from the UE | O |  | 9.2.3.110 |  | YES | ignore |
| IAB Node Indication | O |  | ENUMERATED (true, ...) |  | YES | reject |

| Condition | Explanation |
| --- | --- |
| ifCHOmod | This IE shall be present if the CHO Trigger IE is present and set to "CHO-replace". |

| Range bound | Explanation |
| --- | --- |
| maxnoofMDTPLMNs | PLMNs in the Management Based MDT PLMN list. Value is 16. |

HANDOVER REQUEST ACKNOWLEDGE: RAN3 did also agree to introduce some changes for CHO but these are not relevant for the issue raised in this paper. Contents of the RAN3 message is again as shown below. Main thing to note is that RRC INM HandoverCommand is carried within the mandatory field Target NG-RAN node To Source NG-RAN node Transparent Container (highlighted box)

#### 9.1.1.2	HANDOVER REQUEST ACKNOWLEDGE

This message is sent by the target NG-RAN node to inform the source NG-RAN node about the prepared resources at the target.

Direction: target NG-RAN node  source NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | ignore |
| Target NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | YES | ignore |
| PDU Session Resources Admitted List | M |  | 9.2.1.2 |  | YES | ignore |
| PDU Session Resources Not Admitted List | O |  | 9.2.1.3 |  | YES | ignore |
| Target NG-RAN node To Source NG-RAN node Transparent Container | M |  | OCTET STRING | Either includes the HandoverCommand message as defined in subclause 10.2.2 of TS 36.331 [14], if the target NG-RAN node is an ng-eNB, or the HandoverCommand message as defined in subclause 11.2.2 of TS 38.331 [10], if the target NG-RAN node is a gNB. | YES | ignore |
| UE Context Kept Indicator | O |  | 9.2.3.68 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| DRBs transferred to MN | O |  | DRB List 9.2.1.29 | In case of DC, indicates that SN Status is needed for the listed DRBs from the S-NG-RAN node. | YES | ignore |
| DAPS Response Information | O |  | 9.2.1.34 |  | YES | reject |
| Conditional Handover Information | O |  |  |  | YES | reject |
| >Requested Target Cell ID | M |  | Target Cell Global ID 9.2.3.25 | Target cell indicated in the corresponding HANDOVER REQUEST message |  |  |
| >Maximum Number of CHO Preparations | O |  | 9.2.3.101 |  |  |  |

38.331 HandoverCommand: The RRN INM includes the mandatory field handoverCommandMessage used to transfer an RRCReconfiguration message within a variable size octet string. As the size of the octet string container can be 0, it seems possible to not transfer such RRCReconfiguration message (as may be useful if the source PCell configuration does not result in change of the configuration for the CHO candidate target, see discussion below).

#### –	HandoverCommand

This message is used to transfer the handover command as generated by the target gNB.

Direction: target gNB to source gNB/source RAN.

HandoverCommand message

-- ASN1START

-- TAG-HANDOVER-COMMAND-START

HandoverCommand ::=                 SEQUENCE {

criticalExtensions                  CHOICE {

c1                                  CHOICE{

handoverCommand                     HandoverCommand-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture            SEQUENCE {}

}

}

HandoverCommand-IEs ::=             SEQUENCE {

handoverCommandMessage              OCTET STRING (CONTAINING RRCReconfiguration),

nonCriticalExtension                SEQUENCE {}                                        OPTIONAL

}

-- TAG-HANDOVER-COMMAND-STOP

-- ASN1STOP

HandoverCommand field descriptions

handoverCommandMessage Contains the RRCReconfiguration message used to perform handover within NR or handover to NR, as generated (entirely) by the target gNB.

## Change of PSCell configuration while CHO is configured (CHO update)

In the introduction we introduced the following scenario:

- @T1 the source node initiates preparation of a CHO candidate target #1 by using the RAN# HANDOVER REQUEST (ACKNOWLEDGE) messages, including RRC inter-node messages (HandoverPreparationInformation and HandoverComplete)

- @T2 the source node initiates preparation of a CHO candidate target #2, again using the same Xx and RRC INM

- @T3 the source node initiates some essential change of the source PCell configuration. The source node contacts the node(s) controlling the CHO candidate target, again using the RAN# HANDOVER REQUEST (ACKNOWLEDGE) messages

Some remarks:

- We assume source is unable to determine whether CHO configuration may be affected by change of the source PCell configuration i.e. that only the node controlling the candidate target PCell can perform this check

- This means that source node may initiate handover preparation for a case in which target node determines that there is no need to modify the CHO configuration (i.e. not affected by source PCell change)

- In such a case, target node has the following options:

- Return a HANDOVER REQUEST ACKNOWLEDGE including HandoverCommand set to the same value as before

- Return a HANDOVER REQUEST ACKNOWLEDGE including a handoverCommandMessage octet string of 0 size

- We think that if CHO configuration is not affected, signalling to the UE should be avoided. From this perspective, option b) seems the right approach (although source might also be able to detect that the included Reconfiguration message included with option a is the same as previous)

In order to avoid any interoperability problems, we think it would be good to clarify the behaviour of the involved network nodes. As we think option b) is preferrable, we propose:

Proposal 1	When modifying configuration of PCell while CHO is configured, node controlling PCell (source MN) initiates HO preparation to node controlling candidate target PCell (candidate target MN), as only target node can properly determine whether configuration of candidate target PCell is affected

Proposal 2	If target node determines that configuration of candidate target PCell is not affected, it returns a HANDOVER REQUEST ACKNOWLEDGE including a handoverCommandMessage octet string of 0 size (ensures there will be no signalling towards the UE in such a case)

A CR including the corresponding changes is provided in [2].

# Conclusion & recommendation

This document discusses the Internode coordination upon change of PSCell configuration while CHO is configured (CHO update. The document includes the following proposals that RAN2 is requested to discuss and conclude:

Proposal 1	When modifying configuration of PCell while CHO is configured, node controlling PCell (source MN) initiates HO preparation to node controlling candidate target PCell (candidate target MN), as only target node can properly determine whether configuration of candidate target PCell is affected

Proposal 2	If target node determines that configuration of candidate target PCell is not affected, it returns a HANDOVER REQUEST ACKNOWLEDGE including a handoverCommandMessage octet string of 0 size (ensures there will be no signalling towards the UE in such a case)

# References

[1] TS 38.331 Radio Resource Control

[2] R2-2007230 CR to 38.331 REL-16 on Clarifications regarding CHO (Samsung)