3GPP TSG-RAN WG2 Meeting #111e	R2-2007263

Elbonia, 17 August – 28 August 2020

Agenda item:	6.4.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Further RRC Issue on the presence "Cond CBR" sl-MaxTxPower in SL-PSSCH-TxParameters

WID/SID:	5G_V2X_NRSL-Core

Document for:	Discussion and Decision

# 1	Introduction

There seems to be a mismatch in RRC Specification TS38.331 between RAN1 intention and ASN.1 representation with respect to the presence of sl-MaxTxPower  parameter in SL-PSSCH-TxParameters.

This TDoc discusses the issue.

# 2	Discussion

From RAN1 point of view, the field sl-MaxTxPower should be present in SL-PSSCH-TxParameters only when TX parameters are restricted by congestion control (CBR), not when TX parameters are restricted by UE speed or sync source.

SL-PSSCH-TxParameters-r16 ::=    SEQUENCE {

sl-MinMCS-PSSCH-r16              INTEGER (0..27),

sl-MaxMCS-PSSCH-r16              INTEGER (0..31),

sl-MinSubChannelNumPSSCH-r16     INTEGER (1..27),

sl-MaxSubchannelNumPSSCH-r16     INTEGER (1..27),

sl-MaxTxTransNumPSSCH-r16        INTEGER (1..32),

sl-MaxTxPower-r16                SL-TxPower-r16                                      OPTIONAL    -- Cond CBR

}

In other words, the field sl-MaxTxPower should be present under SL-CBR-CommonTxConfigList/SL-CBR-PSSCH-TxConfig, but not under SL-PSSCH-TxConfigList/SL-PSSCH-TxConfig. However, the current condition “Cond CBR” in TS38.331 marked in yellow above and the given description in the RRC specification document, seems to do rather the opposite:

| Conditional Presence | Explanation |
| --- | --- |
| CBR | The field is optionally present, Need R, when in SL-CBR-CommonTxConfigList in SL-UE-SelectedConfig in SIB12 or SL-PreconfigurationNR; otherwise the field is not present, need R. |

RAN2 needs to clarify the behaviour and description of the above-mentioned issue.

Proposal 1: RAN2 needs to discuss and clarify the correctness of the current presence of “Cond CBR” in the sl-MaxTxPower in SL-PSSCH-TxParameters.

Since ASN.1 for Rel.16 is already frozen, it seems that field sl-MaxTxPower could not be presented (i.e. moved to) under SL-CBR-CommonTxConfigList/SL-CBR-PSSCH-TxConfig, which would resolve the issue but necessitate a change in the current ASN.1 representation.

Proposal 2: RAN2 needs to discuss how to correct the current condition of field sl-MaxTxPower SL-PSSCH-TxParameters.

# 3	Conclusion

Proposal 1: RAN2 needs to discuss and clarify the correctness of the current presence of “Cond CBR” in the sl-MaxTxPower in SL-PSSCH-TxParameters.

Proposal 2: RAN2 needs to discuss how to correct the current condition of field sl-MaxTxPower SL-PSSCH-TxParameters.