3GPP TSG RAN WG2#111-e	R2-2007400

Electronic meeting, 17th – 28th August, 2020

Agenda Item	: 8.12.2.1

Source	: LG Electronics Inc.

Title	: Discussion on how to define reduced capability devices

Document for	: Discussion and Decision

1.	Introduction

According to NR-REDCAP SI description [1], we would like to discuss how to define a limited set of device types and how networks identify such devices.

Study standardization framework and principles for how to define and constrain such reduced capabilities – considering definition of a limited set of one or more device types and considering how to ensure those device types are only used for the intended use cases [RAN2, RAN1]. Study functionality that will allow devices with reduced capabilities to be explicitly identifiable to networks and network operators, and allow operators to restrict their access, if desired [RAN2, RAN1].

2.	Discussion

Considering different level of service requirements of target use cases such as Industrial wireless sensors, Video surveillance and Wearables, it may be inefficient for resource management if only one group is defined to identify all RedCap devices. For example, a UE supporting only FR1 may use services to provide low bandwidth and high latency while a UE supporting only FR2 may use services to provide high bandwidth and low latency with different level of capability requirements.

Reduced capabilities may be grouped based on a set different range of capabilities rather than UE/device itself because one UE may be configured with different level of reduced capabilities. For example, if a UE supporting both FR1 and FR2 can be defined as a RedCap device, the UE may have two RedCap capability sets. It can be further discussed whether one UE can be configured with more than one RedCap capabilities.

Proposal 1. Discuss whether RAN2 will study standardization framework to support more than one reduced capability set.

Proposal 2. Discuss whether one UE can be configured with more than one reduced capability set.

Before beginning the discussion on whether existing NR framework such as capability signallings can be used or not, the parameters list to define RedCap devices should be finalised. Depending on the decision, the existing NR capability framework can be used or new capability framework could be defined for RedCap devices. With predefined values per RedCap capability set, the corresponding capability parameters could be additionally defined. This can be further discussed.

Proposal 3. Suggest to finalise the parameters list to define reduced capability devices before beginning the discussion on UE capability signallings.

3.	Conclusion

Based on the discussion, we propose the following:

Proposal 1. Discuss whether RAN2 will study standardization framework to support more than one reduced capability set.

Proposal 2. Discuss whether one UE can be configured with more than one reduced capability set.

Proposal 3. Suggest to finalise the parameters list to define reduced capability devices before beginning the discussion on UE capability signallings.

4.	Reference

[1] RP-201386, “Study on support of reduced capability NR devices”, Ericsson, RANP#88e, July 2020.