3GPP TSG-RAN WG2 #111e electronic                                    R2-2007428

Online: 17th – 28th, August, 2020

Agenda Item:	8.10.2.1

Source:	CMCC

Title:	Discussion of HARQ feedback for NTN

Document for:	Discussion, Decision

Introduction

For HARQ in NTN, RAN2#104 started to achieve solid agreements as following [1]:

Agreements:

Both options (enhancing HARQ and disabling HARQ) will be studied.

During RAN2#105 meeting, HARQ for NTN was discussed and following agreements were achieved [2]:

Agreements: 1. Retransmissions at one or several layers shall be supported for NTN and configurable by the network. 2. The network should be able to configure the UE, mentioning whether HARQ is “turned off”. There is no feedback for transmission if HARQ is turned off. FFS the impact on other procedures and how to configure.

Following agreements were made during RAN2#106 meeting [3]:

Agreements

1. If HARQ feedback is disabled, blind HARQ (re)transmissions are still possible to improve robustness.  What blind HARQ retransmissions mean will be captured in email discussion.

2. Even if HARQ feedback is disabled, the HARQ processes are still configured.

3. Enabling / disabling of HARQ feedback is a network decision.

And in RAN2#107 meeting, the configuration of HARQ feedback enabling/disabling was reached the following agreement[4]:

5 It should be possible to semi-statically enable / disable HARQ feedback by RRC signalling. 6 The enabling / disabling of HARQ feedback can be configurable on a per UE and per HARQ process basis via RRC signalling.

In this contribution, we will provide further considerations on the HARQ feedback for NTN.

Discussion

As discussed in email discussion [106#71] [NR/NTN][5] HARQ, the configuration granularity of enabling/disabling HARQ feedback had the following options:

- Configurable per cell (HARQ feedback of a specific cell is either enabled or disabled for all UEs)

- Configurable per UE (HARQ feedback of a specific UE is either enabled or disabled)

- Configurable per HARQ process

- Configurable per Logical Channel

- Configurable on a per packet basis

And per UE and per HARQ process were agreed by most companies and reached corresponding conclusion at the 107 meeting as mentioned above. However, the per packet basis option is clearly opposed by majority companies due to too small configuration granularity. Then the per cell configuration and the per LCH configuration still have some supporters, so the final decision is left to the WI. For the per cell configuration, if all UEs in a particular cell which may go through the same RTT and have similar channel quality, this option may be available.

Observation 1: The per cell configuration and the per LCH configuration still have some supporters, so the final decision is left to the WI.

For regular radio resource management (RRM) strategy, the services with similar QoS requirements, in terms of delay, reliability etc., are mapped into one radio bearer and corresponding logical channel. Therefore, a simple and straightforward method is that the LCH configured for high reliability service, e.g. URLLC service, should be selected to configured via RRC with HARQ feedback enabled, while some other LCHs configured for relax requirement for reliability, but strict requirement for throughput service, e.g. some eMBB services, should be selected to configured via RRC with HARQ feedback disabled.

Observation 2: LCH based HARQ feedback enable/disenable can fulfill different requirement for reliability and latency.

Although some concerns may exist that it(per LCH) can be implemented by enable / disable of HARQ feedback on a per HARQ process basis via RRC signalling. At this time, it may be needed to design a mapping relationship between different service types and HARQ processes. For example, to configure mapping between different services and HARQ process number from high to low according to the reliability requirements of the service by RRC signaling, e.g. URLLC service requires higher reliability than eMBB service. However, this mapping relationship may limit the flexibility of the usage HARQ process number, and a certain HARQ process number(e.g. HARQ process number#1…4) would be semi-statically reserved for the URLLC service, which HARQ feedback function is configured in enabled status. For NTN system, this semi-static reservation method will be more serious, where the long RTD cause the increasing requirement of HARQ process number.

Observation 3: Semi-static reservation HARQ process number for different services via RRC will be more serious in NTN system, where the long RTD cause the increasing requirement of HARQ process number.

Proposal 1: RAN2 still need consider to support differentiated HARQ operations for services with different requirement. Enabling / disabling of HARQ feedback can be configured in the granularity of LCH.

As we know, in NTN, the maximum round trip delay is 541.46ms for GEO and 25.77ms for LEO. Hence, configured grant/SPS can reduce the PDCCH load for DCI scheduling and decrease the latency for uplink resource application. One of the main benefits for NTN with configured grant is due to the large delays associated with UE getting uplink resources when the propagation delays are high. In a normal case, the UE has to go through the SR->Grant(for BSR)->BSR->Grant(for UL data) procedure in order to get sufficient uplink resources.

Observation 4: Configured grant/SPS can avoid DCI scheduling and uplink resource application, which will largely reduce the scheduling delay.

There are two different types of uplink grants: type 1 is the method where the configured grant parameters, such as periodicity, MCS, the start offset in time domain and other scheduling information are configured via RRC only. And for type 2, which is similar as DL SPS, the information such as periodicity and HARQ-processes are configured by RRC and the start offset in time domain and other is indicated via DCI scrambled by CS-RNTI.

At the other hand, for configured grant, the HARQ-processes ID is calculated based on the following formula:

HARQ Process ID = [floor(CURRENT_symbol/periodicity)] modulo nrofHARQ-Processes

In the downlink, for Semi-persistent, the HARQ-processes ID is calculated based on the following formula:

HARQ Process ID = [floor (CURRENT_slot × 10 / (numberOfSlotsPerFrame × periodicity))] modulo nrofHARQ-Processes

This means the HARQ Process IDs of configured grant type 2 and SPS, are derived from assigned periodicity, start offset and nrofHARQ-Processes from network. And if the HARQ Process ID is been reserved by RRC, this means the start offset of SPS and configured grant type 2 cannot be selected randomly. This is different from dynamic scheduling, where there is no relationship between HARQ Process ID and radio resource allocation in time domain. Hence, the enable / disable of HARQ feedback on a SPS/configured grant type 2 configurations cannot be implemented by enable / disable of HARQ feedback on a per HARQ process basis via RRC signalling. On the other hand, in Rel-16 industrial IoT/URLLC WID, multiple SPS/configured grant configurations per cell can be configured to a UE has already been agreed in RAN1 and RAN2.

Observation 5: The HARQ Process IDs of configured grant type 2 and SPS, are derived from assigned periodicity, start offset and nrofHARQ-Processes from network.This is different from dynamic scheduling, where there is no relationship between HARQ Process ID and radio resource allocation in time domain.

Observation 6: In Rel-16 industrial IoT/URLLC WID, multiple SPS/configured grant configurations per cell can be configured to a UE has already been agreed in RAN1 and RAN2.

Proposal 2: Consider to support configuration on the enabling / disabling of HARQ feedback of SPS/configured grant configurations via RRC signalling.

Maybe we can consider a more simplified way besides directly configuring HARQ feedback enable or disable through RRC signaling. Obviously, the RTT in NTN networks is much longer than that in terrestrial networks, when HARQ feedback is enabled, the whole HARQ RTD will be too much long, especially in GEO scenario with the maximum propagation delay 541.46 ms. Maybe in some cases, such as when the quality of physical channel is better, and the data transmission can always be successfully received and decoded, at this moment the HARQ feedback or retransmission may be redundant. However, if directly disabling HARQ feedback may have an effect on the transmission reliability for some specific services, therefore simplifying HARQ feedback may be a better compromise.

Observation 7: In some cases, such as when the quality of physical channel is better, and the data transmission can always be successfully received and decoded, at this moment the HARQ feedback or retransmission may be redundant.

Observation 8: If directly disabling HARQ feedback may have an effect on the transmission reliability for some specific services.

Proposal 3: In order to deal with the RTT in NTN, and reducing the impact on data transmission reliability, in some scenarios, a simplified HARQ procedure with feedback may be considered.

To simplify the HARQ feedback, we can consider to carry some indication information in the data packet to indicate that whether the data packet needs feedback according to the reliability requirements of the service type and/or the current channel quality. For example, if the data packets to be transmitted are services that require high reliability, such as URLLC, then the feedback should be required. If current channel quality is very good and lasts for a period of time, then the feedback may be unnecessary.

When no feedback is indicated, in order to ensure reliability as much as possible. In order to solve the problem that the UE did not receive data packets or did not decode successfully, it should originally transmit NACK to the gNB and the gNB needs to retransmit the data packet. The gNB could not discard the few data packets recently sent to the UE, according to the channel quality reported by the UE, once it is determined that the channel quality is below a certain threshold, the gNB retransmits several data packets recently sent to the UE and instructs to the UE feedback is needed from now on.

Observation 9: Whether HARQ feedback is needed can be determined based on service type and/or the current channel quality.

Proposal 4: To simplify the HARQ feedback, we could consider to carry some indication information in the data packet to indicate that whether the data packet needs feedback.

Conclusion

Based on the discussions mentioned above, in this contribution we provide some further discussions on the HARQ feedback for NTN and have the following observations and proposals:

Observation 1: The per cell configuration and the per LCH configuration still have some supporters, so the final decision is left to the WI.

Observation 2: LCH based HARQ feedback enable/disenable can fulfill different requirement for reliability and latency.

Observation 3: Semi-static reservation HARQ process number for different services via RRC will be more serious in NTN system, where the long RTD cause the increasing requirement of HARQ process number.

Proposal 1: RAN2 still need consider to support differentiated HARQ operations for services with different requirement. Enabling / disabling of HARQ feedback can be configured in the granularity of LCH.

Observation 4: Configured grant/SPS can avoid DCI scheduling and uplink resource application, which will largely reduce the scheduling delay.

Observation 5: The HARQ Process IDs of configured grant type 2 and SPS, are derived from assigned periodicity, start offset and nrofHARQ-Processes from network.This is different from dynamic scheduling, where there is no relationship between HARQ Process ID and radio resource allocation in time domain.

Observation 6: In Rel-16 industrial IoT/URLLC WID, multiple SPS/configured grant configurations per cell can be configured to a UE has already been agreed in RAN1 and RAN2.

Proposal 2: Consider to support configuration on the enabling / disabling of HARQ feedback of SPS/configured grant configurations via RRC signalling.

Observation 7: In some cases, such as when the quality of physical channel is better, and the data transmission can always be successfully received and decoded, at this moment the HARQ feedback or retransmission may be redundant.

Observation 8: If directly disabling HARQ feedback may have an effect on the transmission reliability for some specific services.

Proposal 3: In order to deal with the RTT in NTN, and reducing the impact on data transmission reliability, in some scenarios, a simplified HARQ procedure with feedback may be considered.

Observation 9: Whether HARQ feedback is needed can be determined based on service type and/or the current channel quality.

Proposal 4: To simplify the HARQ feedback, we could consider to carry some indication information in the data packet to indicate that whether the data packet needs feedback.

Reference

- R2-1900002, Report of 3GPP TSG RAN2#104 meeting, Spokane, USA

- R2-1903001, Report of 3GPP TSG RAN2#105 meeting, Athens, Greece

- R2-1908601, Report of 3GPP TSG RAN2#106 meeting, Reno, USA

- R2-1912001, Report of 3GPP TSG RAN2#107 meeting, Prague, Czech Republic

- R2-1908987, Report of Email Discussion [106#71][NR/NTN] HARQ (Nomor)