3GPP TSG-RAN2#111e			    	 					 R2-2007449

17-28 August, 2020

Source: 			ZTE Corporation, Sanechips

Title: 	Details of RRC-based IDT

Agenda item:	8.6.2

Document for: 	Discussion and Decision

# Introduction

The intention of this contribution is to discuss the detailed aspects of RRC-based Inactive-state small-Data Transmission (IDT).

# Handling of UE context and data forwarding

## Configuration of RLC entity

Since user plane data transmission will be supported in IDT, and RLC entities will be used for the IDT, one issue that needs to be discussed is regarding the RLC mode that shall be supported for IDT: AM RLC, UM RLC or both.

In-frequent and small data traffic can be generated by a wide variety of applications. These applications include traffic from IM, heart-beat/keep-alive messages, push notifications, traffic from wearables, sensors and smart meters. This traffic may be mapped either to TCP or UDP in the higher layers and typically it is mapped to both RLC AM/UM bearers in the radio layer. Given this, both AM RLC and UM RLC should be supported IDT.

Proposal 1: Both AM RLC and UM RLC shall be supported for IDT.

In order to process the RLC PDU, the RLC configuration will be required to establish the RLC entities on NW side (e.g. in DU). For establishing the RLC configuration, the following two alternatives can be considered:

- Alt1: UE specific RLC configuration received within the UE context will be used to establish the RLC entities in the target DU.

- Alt2: Default RLC configuration will be used to establish the RLC entities

For the alternative 1: UE specific RLC configuration

If alternative 1 is adopted, then the new serving node shall buffer the RLC PDU received, and process the buffered RLC PDU after the UE context is fetched and the corresponding RLC entities are established based on the fetched UE context.

Two examples are shown as follows to illustrate the procedure. The figure below is only for the example on the handling of RLC, the detail procedure will be left to RAN3

Figure x: Example of alternative 1 with anchor relocation

Figure x: Example of alternative 1 without anchor relocation

Based on the procedure discussed above, we give our proposal as follow:

Observation 1: If UE specific RLC configuration is adopted, the new serving gNB (or new DU in CU/DU split case) shall buffer the RLC PDU received, and process the buffered RLC PDU after UE context is fetched and the corresponding RLC entities are established based on the fetched UE context.

For the alternative 2: Default RLC configuration

For the alterantive2 with default RLC configuration, one example is shown as follow to illustrate the procedure. The figure below is only for the example on the handling of RLC, the detail procedure will be left to RAN3

Figure x: Example of alternative 2 without anchor relocation

Figure x: Example of alternative 2 without anchor relocation

If alternative 2 is adopted, then the following two alternatives can be considered for the default configuration.

- Default configuration shall be specified in specs

- Default configuration shall be broadcasted in system information.

To provide more flexibility on NW side, we prefer to have configurable default RLC configuration for INACTIVE data transmission, and the default configuration shall be broadcasted in system information.

Observation 2: If default RLC configuration is adopted, then the default configuration shall be broadcasted in system information.

The pros and cons of each alternative are summarized below.

| Alternatives | Pros | Cons |
| --- | --- | --- |
| Alt1: UE specific RLC configuration | QoS can be guaranteed based on the UE specific configuration. No RLC reconfiguration/re-establishment is needed after entering CONNECTED state | The RLC PDU can only be processed after the context fetch procedure. This results in extra latency. The context fetch procedure is needed before data forwarding, even for the case without anchor relocation. Note also that there will be some changes needed for the context fetch without anchor relocation (since the RLC context needs to be fetched regardless of anchor relocation/path-switch) |
| Alt2: Default configuration | With the default RLC configuration, the RLC entities can be established in DU once the RLC PDU is received, and the RLC PDU can be forwarded to CU or anchor gNB together with the RRCResumeReq message. | It is not clear whether the default RLC configuration can satisfy the QoS requirements. However, given that the target applications are for infrequent, small and generally latency tolerant traffic, it seems some default configuration might be good enough. RLC reconfiguration/re-establishment may be needed after entering CONNECTED state, in case the UE specific RLC configuration is different from the default RLC configuration |

Even though the solution with default configuration may have some negative impact on the QoS, considering the target traffic or INACTIVE data transmission is latency tolerant traffic, the default configuration seems sufficient for such kind of traffic. Therefore, to avoid unnecessary complexity, we propose to adopt alternative with default configuration for IDT.

Proposal 2: Default RLC configuration will be used for RLC entities in IDT

## Context fetch and data forwarding

Based on the WID, the “Context fetch and data forwarding (with and without anchor relocation) in INACTIVE state for RACH-based solutions” shall be supported in INACTIVE data transmission.

In the legacy RRC resume procedure, the RRCResumeReq message received in the serving node will be forwarded back to the anchor node, and it is up to anchor node to determine whether to perform anchor relocation or not. Anchor relocation and context fetch will be performed in case the change of anchor node is required

In the INACTIVE data transmission with RRC involved, since the RRCResumeReq message will be sent from UE to NW anyway, it is straight forward to take the anchor relocation procedure for legacy resume procedure as baseline. However, compared to the legacy resume procedure, one different thing is that the data PDU can be transmitted together with the RRCResumeReq message in the same MAC PDU, thus some discussion will be required to understand how to handle the data PDU in context fetch and data forwarding.

Although this is mainly a RAN3 issue, some input from RAN2 is required. For example, as discussed in section 4.1, whether UE specific RLC configuration or default RLC configuration will be used will lead to impact on the overall RAN3 procedure. Therefore, we think the detail procedure on context fetch and data forwarding be left to RAN3, and one LS shall be sent to RAN3 to trigger the discussion once we have some necessary progress in RAN2 (e.g RLC configuration).
Proposal 3: Once we have the necessary progress in RAN2 (e.g. regarding handling of the RLC configuration), an LS shall be sent to RAN3 to trigger the necessary work in RAN3

# Radio Resource Configuration and resource selection

## RACH resource configuration

When 2-step RACH was specified, both common and separate RACH resources (compared to the 4-step RACH) were specified to avoid the impacts to the legacy RACH resources. Similarly, For the RACH based solution for IDT, separate RACH resource pool for IDT shall be allowed. And the following two alternatives can be supported:

- Alt1: Common RACH resource, in which case the RACH resources are shared by both legacy RACH procedure (i.e. RACH initiated by legacy purpose) and IDT

- Alt2: IDT specific RACH resource, in which case a separate RACH resource pool (e.g. in frequency domain) can be configured for IDT

To enable the alternative 2, the following aspects should be considered:

- Separate PRACH resource configuration (for both 2-step RACH and 4-step RACH)

- Separate MSGA PUSCH resource configuration (for 2-step RACH only)

- Separate CORESET/Search Space for Msg2/MSGB reception or a separate RNTI (for both 2-step RACH and 4-step RACH)

Proposal 4: For the configuration of RACH resource, the following two kinds of configurations shall be supported:

- Alt1: Common RACH resource, in which case the RACH resources are shared by both legacy RACH procedure (i.e. RACH initiated by legacy purpose) and IDT

- Alt2: IDT specific RACH resource, in which case a separate RACH resource pool can be configured for IDT

Proposal 5: To enable the configuration of specific RACH resource for IDT, the following shall be supported:

- Separate PRACH resource configuration. (for both 2-step RACH and 4-step RACH)

- Separate MSGA PUSCH resource configuration (for 2-step RACH only)

- Separate CORESET/Search Space for MSG2/MSGB reception or a separate RNTI (for both 2-step RACH and 4-step RACH)

## CG resource configuration

For the configuration of CG type 1 resource valid in INACTIVE state shall be configured to UE before UE enters the INACTIVE state, and the CG resource configured is only valid in the cell where the UE enters INACTIVE state.

Proposal 6: The CG resource for INACTIVE data transmission shall be configured to UE before UE enters INACTIVE state. And the CG resource configured is only valid in the cell where the UE enters INACTIVE state.

## Transmission type selection and switching

Initial transmission type selection

If IDT is supported, whenever UL data arrives on UE side, the UE needs to select the transmission type between the normal RRC resume procedure and the IDT procedure. To perform the transmission type selection, the following aspects shall be considered:

- Whether the IDT is allowed in the cell

- Whether the IDT is applicable to the on-going services

The UE can determine whether the IDT is allowed in a given cell based on the system information of the corresponding cell, by either an explicit indicator or some configuration related to the INACTIVE data transmission.

With regards to the applicability to on-going services, the following two aspects shall be considered:

- The Logical channel/LCG for which there is data available for transmission

  - Different logical channels have different QoS requirement, and only some of them can be satisfied by the IDT. Therefore, we think the logical channel with data available shall be considered in the transmission type selection.

- The buffer size of the corresponding LCH/LCG on UE side:

  - Since the IDT aims to support the data transmission with limited data volume, it is quite straight forward to take the buffer size into account. If there are lots of data buffered on UE side, then the normal RRC resume procedure shall be performed.

    - It should however be noted that the state transition decisions should be under network control. Hence, if the volume of data exceeds a threshold, the network can perform full RESUME (even if the UE includes small data packet in the initial UL message).

Proposal 7: For the transmission type selection between legacy RRC resume procedure and IDT procedure, the UE shall take the following into account:

- Whether the IDT procedure is allowed in the cell (indication in system information is needed for this purpose)

- The Logical channel/LCG for which there is data buffered for transmission (this can be based on explicit network configuration)

- FFS whether a buffer size related threshold is also needed at the UE side (or if this can be left to network implementation, e.g. based on the BSR)

Transmission type switch

Once the transmission type is determined by UE, the UE will initiate legacy RRC resume procedure or IDT procedure accordingly. Since the buffer size information for each LCG may change on UE side, and whenever the criteria for IDT is no longer satisfied, the UE is expected to be switched to CONECTED mode and perform normal UL transmission.

Observation 3: During the IDT, whenever the criteria for IDT is no longer satisfied, the network may switch the UE to CONECTED mode and perform normal UL transmission instead.

For the transmission type switch, the following two issues need to be considered:

- How to switch the UE from INACTIVE to CONNECTED state during the INACTIVE data transmission.

- How to detect the change of buffer status on UE side (i.e. the criteria for INACTIVE data transmission is no longer satisfied)

For the detection of change of buffer status on UE side, the BSR based mechanism seems sufficient. For the switch from INACTIVE to CONNECTED state, in the RRC involved procedure, the NW can simply reconfigure the UE to normal RRC_CONNECTED mode directly with RRCResume message at any point during the IDT procedure.

Proposal 8: The NW can send RRCResume message instead of RRCRelease message to switch the UE to RRC_CONNECTED state at any point during the IDT phase.

# RLM and RRM related aspects

## Handling of cell reselection and measurements

Handling of the user plane upon Cell re-selection during IDT

According to current specs, the UE will continue cell re-selection related measurements as well as cell re-selection evaluation after the transmission of RRCResumeRequest message. However, if cell reselection occurs while T319 is running, the UE will perform the actions upon going to RRC_IDLE as specified in 5.3.11 with release cause RRC Resume failure.

Similar to the legacy resume procedure, the UE should continue the cell re-selection related measurements and cell re-selection evaluation as well. However, unlike the legacy resume procedure, there will be data transmission and hence potentially unacknowledged data after the transmission of RRCResumeRequest and before the reception of RRCRelease/RRCResume. Hence, if the UE moves to IDLE mode when there is unacknowledged data, this will lead to data loss.

Proposal 9: During IDT, the UE continues the cell re-selection related measurements and cell re-selection evaluation (i.e. as done today while T319 is running).

Observation 4: If UE moves to IDLE mode upon Cell reselection during IDT, then this may lead to data loss

Firstly, RAN2 should discuss whether loss less data transmission for IDT shall be specified in Rel-17. If lossless data transfer during IDT is to be supported during cell reselection, then some further changes are needed. As a starting point, the UE should stay in INACTIVE state. Further to this, the UE shall try to recover the control plane (security) and the user plane in the new cell. For the user plane, PDCP level retransmission may be used as a baseline.

Proposal 10: RAN2 to discuss and agree whether mechanisms for lossless data transfer for IDT should be specified.

Proposal 11: To enable lossless data transfer during IDT, if cell re-selection occurs, the UE should remain in INACTIVE state.

In order to recover the user plane using PDCP level retransmission, the following procedure can be considered as baseline:

Upon triggering cell reselection during IDT,

- UE suspends DRB and remains in INACTIVE state

- UE performs cell re-selection

- UE initiates IDT in the target cell (if IDT is supported) and performs PDCP level retransmission for the unacknowledged PDCP PDUs (e.g. perform PDCP re-establishment)

Proposal 12: In case of cell re-selection during IDT, to enable the lossless data transmission, the following procedure can be considered as baseline for user plane recovery.

- UE suspends the DRB and remains in INACTIVE

- UE performs cell re-selection

- UE initiates IDT in the target cell and performs PDCP level retransmission for the unacknowledged PDCP PDUs (e.g. perform PDCP re-establishment)

Measurements during IDT

In IDT, to support the subsequent data transmission, UE needs to monitor PDCCH with C-RNTI within the search space configured for the IDT. In general, during the T319, although cell reselection is possible, we don’t allow measurement gap based measurements. In general the search space for monitoring the C-RNTI during this phase is sparse enough to allow the UE to perform the necessary measurements and hence cell reselection is possible without explicit measurement gaps whilst T319 is running. If similar approach is taken, then we can extend the same mechanism also for IDT and no new measurement gaps will be needed for handling the cell reselection during IDT phase. Based on the above the following proposal is made:

Proposal 13: RAN2 to select between the following options for measurements during IDT:

- Alt1: If cell reselection related measurements during IDT can follow the existing mechanisms then there is no need to define new measurement gap based measurements during IDT phase.

- Alt2: measurement gap based measurements are enabled for IDT for the purpose of inter-frequency measurements during T319

Handling of security context upon cell reselection during IDT

The security context stored at the UE is used to derive security keys for IDT. For the key derivation, the following variables will be used:

- FC = 0x70

- P0 = PCI (target physical cell id)

- L0 = length of PCI (i.e. 0x00 0x02)

- P1 = ARFCN-DL (the absolute frequency of SSB of the target PCell as specified in clause 13.3 of TS 38.300 [52])

- L1 = length of ARFCN-DL (i.e. 0x00 0x03)

- The input key KEY shall be the 256-bit NH when the index NCC in UE’s security context is incremented, otherwise the current 256-bit KgNB

For the calculation of resumeMAC-I, the following variables will be used:

- sourcePhysCellId

- targetCellIdentity

- source-c-RNTI

- current KRRCint

Observation 5: In the UE INACTIVE AS context, the KgNB stored will be used in the key derivation, and KRRCint keys stored will be used in the calculation of resumeMAC-I.

Further, it should also be noted that the security context will be stored/updated whenever the UE enter INACTIVE state based on the reception of RRC Release.

Observation 6: UE will replace the KgNB and KRRCint keys with the current KgNB and KRRCint keys stored in UE Inactive AS context only upon reception of RRCRelease message from the network.

One further question is whether the UE needs to update the security input stored in UE Inactive AS context upon cell reselection or not. The following table shows the pros and cons of updating the security material upon cell reselection.

|  | Pros | Cons |
| --- | --- | --- |
| Don’t update keys in the security context | - This is aligned with current procedure (i.e. UE updates security context only upon receiving RRCRelease). i.e. fewer changes to specs - Network doesn’t need to keep track of the keys used in different cells where the UE may have performed cell reselection from | - When UE reverts to the same cell after multiple reselection steps (e.g. reselection from cell A -> B ->A, then UE will derive same keys in cell A), it may generate the same keys. It is unclear if this is an issue though. |
| Update keys in the security context | - Key reuse won’t happen even if UE reselects to the same cell multiple times. | - Network has to keep track of all the intermediate keys generated in different cells from which the cell reselection has happened - If the contention resolution has not yet finished before reselection, the UE will not have a valid C-RNTI to use (this means that keys in security context can only be replace after successful contention resolution in a given cell) |

Based on the above, it can be seen that there are pros and cons for both approaches.

So, RAN2 should discuss and conclude on one of the options:

Proposal 14: In case of cell reselection after contention resolution during IDT in a given cell:

- Alt1: UE replaces the security keys in the UE INACTIVE context with the current keys

- Alt2: UE does not replace the security keys in the UE INACTIVE state with current keys

## Beam management

Besides the cell re-selection, the beam management (i.e. beam mobility) shall be considered as well. In the legacy initial access procedure, the NW will use the same beam as the one used in the preamble transmission in RACH procedure. For the INACTIVE data transmission, in order to support the subsequent data transmission, the UE is expected to perform data transmission in IDT for a while. Based on the discussion in previous section, if the proposal for the extension of T319 is agreed, then some enhancements may be needed for the beam management during INACTIVE data transmission.

Since the beam management is mainly a RAN1 issues, and no RAN1 TU is allocated for the WI, one LS is suggested to ask RAN1 that whether any enhancement is needed for the beam management during INACTIVE data transmission, and how to do the beam management if it is needed.

Proposal 15: One LS shall be sent to RAN1 to ask whether any enhancement is needed for the beam management during INACTIVE data transmission, and how to do the beam management if it is needed.

## Maximum duration of IDT

For IDT, it is important to allow the UE to kept in the IDT phase until any possible ACK/NACK in the opposite direction is received by the UE. Unless this is ensured, the gains of IDT will be lost since the UE will need to be paged immediately to deliver the ACK/NACK in the opposite direction. The T319 value covers the IDT phase. In case of LTE and NB-IoT, in order to support EDT, T300 has been extended. However, the values used in LTE and NB-IoT were extended considering the fact that there repetitions on the air interface and up 60 seconds is used for LTE whilst up to 120 secs is specified for NB-IoT. Such large round trip times are unrealistic in modern networks – especially in the 5G networks. The typical TCP round trip times in modern cellular networks are in the range of a few hundreds of milli seconds. Considering this, the range of T319 could be extended by a few more seconds (exact values FFS) but not the same range as the LTE/NB-IoT EDT values.

Proposal 16: The T319 value could be extended by a few seconds (but not up the range of LTE EDT values) to support the delivery of the ACK/NACK in the opposite direction. The exact values are TBD.

## Failure detection and handling

In legacy RRC resume procedure, the RRC Resume failure will be triggered upon expiry of T319. For the RRC-based IDT, since the RRC resume message will be sent to NW, and the RRC release (or RRC Resume) message will be sent to UE to terminate the IDT procedure, the T319 can be reused to trigger the failure as well. Once the T319 is expired (i.e. neither RRC Release nor RRC Resume is received before the expiration of T319), the UE will enter IDLE mode.

Proposal 17: IDT will be considered as failed once the T319 expires. The UE enters IDLE mode in such case.

For the failure handling, one issue is whether radio link monitoring and beam failure detection are applicable during IDT. Although user plane data transmission is supported, the UE is still in INACTIVE state during IDT. Also considering the time duration for IDT is relatively short, to avoid unnecessary complexity, we think the radio link monitoring and beam failure detection is not needed in INACTIVE data transmission.

Proposal 18: The Radio Link Monitoring and Beam Failure Detection are not supported in during IDT.

Besides the RLF, the following failure indication can be received from lower layers as well.

- RACH failure

- Consistent uplink LBT failure

- RLC failure (e.g. maximum number of retransmissions has been reached)

In legacy RRC Resume procedure, the RACH failure will be ignored if T319 is running. For the INACTIVE data transmission, we think the similar principle can be applied as well that the RACH failure will be ignored in RRC layer during the INACTIVE data transmission.

For the Consistent uplink LBT failure and RLC failure, there may be some benefit to trigger an early failure handling (i.e. enter IDLE mode) immediately and reduce the interruption time. However, considering the IDT is not designed for the service with strict latency requirement, we think the extra latency is acceptable. In order to save complexity, we think the failure detection can be relied on the expiration of T319 as well, and the RACH failure, Consistent uplink LBT failure and RLC failure can be ignored during IDT.

Proposal 19: The RACH failure, Consistent uplink LBT failure and RLC failure received from lower layer can be ignored by RRC during INACTIVE data transmission (i.e. the failure detection can be relied on the expiration of T319).

Besides the failure handling in UE side, the failure handling in NW side shall be considered as well. In normal resume procedure, in case no corresponding UE context can be identified on NW side, the NW can send RRCConnectionSetup message and fallback to RRC setup procedure. In the INACTIVE data transmission procedure, we think similar fallback approach can be supported as well.

Proposal 20: In case no corresponding UE context can be identified on NW side, the NW can send RRCConnectionSetup to UE and fallback to RRC connection setup procedure.

# Conclusion and proposals

Based on the analysis above, we give our proposals as follows:

Configuration of RLC entity

Proposal 1: Both AM RLC and UM RLC shall be supported for IDT.

Observation 1: If UE specific RLC configuration is adopted, the new serving gNB (or new DU in CU/DU split case) shall buffer the RLC PDU received, and process the buffered RLC PDU after UE context is fetched and the corresponding RLC entities are established based on the fetched UE context.

Observation 2: If default RLC configuration is adopted, then the default configuration shall be broadcasted in system information.

Proposal 2: Default RLC configuration will be used for RLC entities in IDT

Context fetch and data forwarding

Proposal 3: Once we have the necessary progress in RAN2 (e.g. regarding handling of the RLC configuration), an LS shall be sent to RAN3 to trigger the necessary work in RAN3

RACH resource configuration

Proposal 4: For the configuration of RACH resource, the following two kinds of configurations shall be supported:

- Alt1: Common RACH resource, in which case the RACH resources are shared by both legacy RACH procedure (i.e. RACH initiated by legacy purpose) and IDT

- Alt2: IDT specific RACH resource, in which case a separate RACH resource pool can be configured for IDT

Proposal 5: To enable the configuration of specific RACH resource for IDT, the following shall be supported:

- Separate PRACH resource configuration. (for both 2-step RACH and 4-step RACH)

- Separate MSGA PUSCH resource configuration (for 2-step RACH only)

Separate CORESET/Search Space for MSG2/MSGB reception or a separate RNTI (for both 2-step RACH and 4-step RACH)

CG resource configuration

Proposal 6: The CG resource for INACTIVE data transmission shall be configured to UE before UE enters INACTIVE state. And the CG resource configured is only valid in the cell where the UE enters INACTIVE state.

Initial transmission type selection

Proposal 7: For the transmission type selection between legacy RRC resume procedure and IDT procedure, the UE shall take the following into account:

- Whether the IDT procedure is allowed in the cell (indication in system information is needed for this purpose)

- The Logical channel/LCG for which there is data buffered for transmission (this can be based on explicit network configuration)

- FFS whether a buffer size related threshold is also needed at the UE side (or if this can be left to network implementation, e.g. based on the BSR)

Transmission type switch

Observation 3: During the IDT, whenever the criteria for IDT is no longer satisfied, the network may switch the UE to CONECTED mode and perform normal UL transmission instead.

Proposal 8: The NW can send RRCResume message instead of RRCRelease message to switch the UE to RRC_CONNECTED state at any point during the IDT phase.

Handling of the user plane upon Cell re-selection during IDT

Proposal 9: During IDT, the UE continues the cell re-selection related measurements and cell re-selection evaluation (i.e. as done today while T319 is running).

Observation 4: If UE moves to IDLE mode upon Cell reselection during IDT, then this may lead to data loss

Proposal 10: RAN2 to discuss and agree whether mechanisms for lossless data transfer for IDT should be specified.

Proposal 11: To enable lossless data transfer during IDT, if cell re-selection occurs, the UE should remain in INACTIVE state.

Proposal 12: In case of cell re-selection during IDT, to enable the lossless data transmission, the following procedure can be considered as baseline for user plane recovery.

- UE suspends the DRB and remains in INACTIVE

- UE performs cell re-selection

- UE initiates IDT in the target cell and performs PDCP level retransmission for the unacknowledged PDCP PDUs (e.g. perform PDCP re-establishment)

Measurements during IDT

Proposal 13: RAN2 to select between the following options for measurements during IDT:

- Alt1: If cell reselection related measurements during IDT can follow the existing mechanisms then there is no need to define new measurement gap based measurements during IDT phase.

- Alt2: measurement gap based measurements are enabled for IDT for the purpose of inter-frequency measurements during T319

Handling of security context upon cell reselection during IDT

Observation 5: In the UE INACTIVE AS context, the KgNB stored will be used in the key derivation, and KRRCint keys stored will be used in the calculation of resumeMAC-I.

Observation 6: UE will replace the KgNB and KRRCint keys with the current KgNB and KRRCint keys stored in UE Inactive AS context only upon reception of RRCRelease message from the network.

Proposal 14: In case of cell reselection after contention resolution during IDT in a given cell:

- Alt1: UE replaces the security keys in the UE INACTIVE context with the current keys

- Alt2: UE does not replace the security keys in the UE INACTIVE state with current keys

Beam management

Proposal 15: LS shall be sent to RAN1 to ask whether any enhancement is needed for the beam management during INACTIVE data transmission, and how to do the beam management if it is needed.

Maximum duration of IDT

Proposal 16: The T319 value could be extended by a few seconds (but not up the range of LTE EDT values) to support the delivery of the ACK/NACK in the opposite direction. The exact values are TBD.

Failure detection and handling

Proposal 17: IDT will be considered as failed once the T319 expires. The UE enters IDLE mode in such case.

Proposal 18: The Radio Link Monitoring and Beam Failure Detection are not supported in during IDT.

Proposal 19: The RACH failure, Consistent uplink LBT failure and RLC failure received from lower layer can be ignored by RRC during INACTIVE data transmission (i.e. the failure detection can be relied on the expiration of T319).

Proposal 20: In case no corresponding UE context can be identified on NW side, the NW can send RRCConnectionSetup to UE and fallback to RRC connection setup procedure.

# References

- RP-201305, Updated Work Item on NR small data transmissions in INACTIVE state, ZTE, RAN#88e

- R2-1700672, Report of 3GPP TSG RAN WG2 AdHoc on NR

- R2-1702207, Reply LS on R2-1700656 on RRC INACTIVE, SA3, RAN2#97