3GPP TSG-RAN WG2 Meeting #111 electronic	                      R2-2007450

Online, August 17th - 28th, 2020

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0277 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the CAPC selection for MSG3 and MSGA PUSCH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-Core |  |  |  |  |  |  | Date: |  | 2020-08-02 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In TS 37.213, it has been specified that “A UE shall use Type 1 channel access procedure for PRACH transmissions and PUSCH transmissions without user plane data related to random access procedure that initiate a channel occupancy with UL channel access priority class p=1 in Table 4.2.1-1.” Hence, for MSG3 and MSGA PUSCH transmissions without user-plane data, the highest channel access priority class p=1 would be used. Hence, the CAPC selection would only be applicable for MSG3 and MSGA PUSCH transmissions with user-plane data. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify that: MSG3 and MSGA PUSCH transmissions without user-plane data would use the highest channel access priority class p=1 directly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The CAPC selection may not be consistent with RAN1 specifications in TS 37.213. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of Changes

### 5.6.2	Channel Access Priority Classes

Table 5.6.2-1 shows which CAPC should be used by traffic belonging to the different standardized 5QIs. A non-standardized 5QI (i.e. operator specific 5QI) should use suitable CAPC based on the below table, i.e. CAPC used for a non-standardized 5QI should be the CAPC of the standardized 5QIs which best matches the traffic class of the non-standardized 5QI.

For DRBs, the gNB selects the CAPC by taking into account the 5QIs of all the QoS flows multiplexed in this DRB while considering fairness between different traffic types and transmissions. For SRB0, SRB1, SRB3, and MSG3 and MSGA PUSCH transmissions which don’t include data from DRBs, the CAPC is always the highest priority (i.e. the lowest number in Table 5.6.2-1). The padding BSR and recommended bit rate MAC CEs use the lowest priority CAPC (i.e. highest number in Table 5.6.2-1) while other MAC CEs use the highest priority CAPC. For uplink transmissions on configured grants, MSG3 and MSGA PUSCH transmissions including data from DRBs, and other uplink transmissions where the UE performs Type 1 LBT (see TS 37.213 [37], clause 4.2.1.1) and CAPC is not indicated in the DCI, the gNB configures the UE for the CAPC to be used for SRB2 and DRBs and the UE shall select the CAPC as follows:

-	highest priority CAPC of MAC CE(s) if only MAC CE(s) are included;

-	highest priority CAPC of DCCH(s) if DCCH SDU (s) are included;

-	lowest priority CAPC of the logical channel(s) with MAC SDU multiplexed in this MAC PDU otherwise.

# Table 5.6.2-1: Mapping between Channel Access Priority Classes and 5QI

| Channel Access Priority Class () | 5QI |
| --- | --- |
| 1 | 1, 3, 5, 65, 66, 67, 69, 70, 79, 80, 82, 83, 84, 85 |
| 2 | 2, 7, 71 |
| 3 | 4, 6, 8, 9, 72, 73, 74, 76 |
| 4 | - |

End of Changes