3GPP TSG-RAN WG2 Meeting #111 Electronic	R2-2007562

Elbonia, 17 – 28 August 2020

Agenda item:	8.9.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Potential TRS/CSI-RS occasion(s)

WID/SID:	NR_UE_pow_sav_enh-Core - Release 17

Document for:	Discussion and Decision

# 1	Introduction

New WID on UE power saving was approved in RP-200938

In this contribution we discuss potential TRS/CSI-RS occasion(s) available in connected mode to idle/inactive-mode UEs as per the following study item objective:

- Specify enhancements for idle/inactive-mode UE power saving, considering system performance aspects [RAN2, RAN1] - Study and specify paging enhancement(s) to reduce unnecessary UE paging receptions, subject to no impact to legacy UEs [RAN2, RAN1] - NOTE: RAN1 to check and update, if needed, evaluation methodology in RAN1 #102-e meeting - Specify means to provide potential TRS/CSI-RS occasion(s) available in connected mode to idle/inactive-mode UEs, minimizing system overhead impact [RAN1] - NOTE: Always-on TRS/CSI-RS transmission by gNodeB is not required

# 2	Discussion

In the work item objective it is explicitly stated that ”Always-on TRS/CSI-RS transmission by gNodeB is not required”.  In addition, it is stated that the intention is to provide potential TRS/CSI-RS occasion(s) available in connected mode to idle/inactive-mode UEs. In our understanding this means that TRS/CSI-RS transmission in CONNECTED mode UEs can be used by IDLE/INACTIVE UEs

Proposal 1: Additional TRS/CSI-RS transmission is not introduced for IDLE/INACTIVE mode UEs

Proposal 2: IDLE/INACTIVE mode UEs can utilize TRS/CSI-RS occasion(s) which are transmitted for CONNECTED mode UEs

Proposal 3: There is no guarantee that the TRS/CSI-RS are constantly available for IDLE/INACTIVE mode UEs

Even though there is no guarantee that the TRS/CSI-RS are available, the network may provide a configuration of the RS to the UEs. The configuration may become large and infeasible to signal via SIBs, which are cell-specific i.e. common to all beams. However, configuration content should be decided by RAN1. In the work item it stated to “Specify means to provide potential TRS/CSI-RS occasion(s) available in connected mode to idle/inactive-mode UEs, minimizing system overhead impact [RAN1]”. RAN2 should be responsible for signalling aspects and RAN1 should decide what exactly is signalled to the UE. Therefore we propose the following:

Proposal 4: RAN2 decides signalling aspects of potential TRS/CSI-RS occasion(s) available in connected mode to idle/inactive-mode UEs after RAN1 has concluded the content of the signaling

# 3	Conclusion

In this contribution potential TRS/CSI-RS occasion(s) available in connected mode to idle/inactive-mode UEs was discussed, and the following is proposed:

Proposal 1: Additional TRS/CSI-RS transmission is not introduced for IDLE/INACTIVE mode UEs

Proposal 2: IDLE/INACTIVE mode UEs can utilize TRS/CSI-RS occasion(s) which are transmitted for CONNECTED mode UEs

Proposal 3: There is no guarantee that the TRS/CSI-RS are constantly available for IDLE/INACTIVE mode UEs

Proposal 4: RAN2 decides signalling aspects of potential TRS/CSI-RS occasion(s) available in connected mode to idle/inactive-mode UEs after RAN1 has concluded the content of the signaling