3GPP TSG-RAN WG2 Meeting #111 Electronic	R2-2007563

Elbonia, 17 – 28 August 2020

Agenda item:	8.9.2

Source:	Nokia, Nokia Shanghai Bell

Title:	IDLE / INACTIVE mode UE power saving

WID/SID:	NR_UE_pow_sav_enh-Core - Release 17

Document for:	Discussion and Decision

# 1	Introduction

New WID on UE power saving was approved in RP-200938

In this contribution we discuss IDLE/INACTIVE mode UE power saving as per the following study item objective:

- Specify enhancements for idle/inactive-mode UE power saving, considering system performance aspects [RAN2, RAN1] - Study and specify paging enhancement(s) to reduce unnecessary UE paging receptions, subject to no impact to legacy UEs [RAN2, RAN1]

# 2	Discussion

In LTE extended DRX (eDRX) is supported in IDLE mode. The DRX cycle maximum value is 2621.44 seconds (43.69 minutes) and for NB-IoT, the maximum value of the DRX cycle is 10485.76 seconds (2.91 hours). DRX is configurable via broadcast and NAS. The UE starts monitoring paging DRX during a Paging Time Window (PTW). During the PTW, the UE monitors paging for the duration of the PTW (as configured by NAS) or until a paging message is including the UE's NAS identity received for the UE, whichever is earlier. It should be noted that eDRX may not be suitable for smartphone type of devices due to increased call delays.

Proposal 1: LTE RRC IDLE mode eDRX functionality can be considered for UE power saving in NR RRC IDLE mode

For RRC INACTIVE eDRX introduction seems more challenging, because paging is handled by RAN instead of AMF- Also for BL UEs and UEs in enhanced coverage in RRC_INACTIVE, extended DRX cycles up to 10.24 s without PTW are supported. Therefore, we propose the following.

Proposal 2: RAN2 studies more whether and how eDRX can be introduced for RRC INACTIVE

In LTE NB-IoT UEs, BL UEs or UEs in enhanced coverage can use (G)WUS, when configured in the cell, to reduce the power consumption related to paging monitoring. If UE detects the WUS or WUS group, the UE shall monitor the following N paging occasions unless it has received a paging message. Otherwise the UE is not required to monitor paging. In NR WUS for paging would need to be transmitted using beam sweeping which would cause overhead for the network. Therefore, we propose the following:

Proposal 3: RAN2 studies more whether and how WUS for paging would be feasible considering overhead caused by beam based operations.

# 3	Conclusion

In this contribution IDLE/INACTIVE mode UE power saving was discussed, and the following is proposed:

Proposal 1: LTE RRC IDLE mode eDRX functionality can be considered for UE power saving in NR RRC IDLE mode

Proposal 2: RAN2 studies more whether and how eDRX can be introduced for RRC INACTIVE

Proposal 3: RAN2 studies more whether and how WUS for paging would be feasible considering overhead caused by beam based operations.