3GPP TSG-RAN WG2 Meeting #111-e	R2-2007582

# Electronic Meeting, 17th – 28th August 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.300 | CR | 1306 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Misc corrections for Rel-16 DCCA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-Core |  |  |  |  |  | Date: |  |  | 2020-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Miscellaneous corrections: - The broadcast configuration of (E-UTRA and NR) idle/inactive measurements is included in SIB5, i.e. the SIB that also contains E-UTRA inter-frequency cell reselection information. The SSB measurements configuration for NR idle/inactive measurements can also be included in SIB24, i.e. the SIB that contains information for inter-RAT cell reselection to NR. - In 7.4 the description for SIB5 is however missing that it contains information for configuration of idle/inactive measurements and the description of SIB24 does not include that it contains information about NR idle/inactive measurements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 7.4: - The description of SIB5 is updated to add that it contains information for configuration of idle/inactive measurements - The description of SIB24 is updated to include that it contains information about NR idle/inactive measurements. Impact analysis Impacted 5G architecture options: NR SA, NR-DC, (NG)EN-DC, NE-DC Impacted functionality: Miscellaneous minor corrections to TS 38.331 Inter-operability: If the network is implemented according to the CR and the UE is not, there will not be inter-operability problems. If the UE is implemented according to the CR and the network is not, there will not be inter-operability problems. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Miscellaneous non-controversial errors will remain in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR CR… |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

## 7.4	System Information

System information is divided into the MasterInformationBlock (MIB) and a number of SystemInformationBlocks (SIBs):

-	MasterInformationBlock defines the most essential physical layer information of the cell required to receive further system information;

-	SystemInformationBlockPos contains positioning assistance data;

-	SystemInformationBlockType1 and SystemInformationBlockType1-BR (for a BL UE or UE in enhanced coverage) contain information relevant when evaluating if a UE is allowed to access a cell and defines the scheduling of other system information blocks;

-	SystemInformationBlockType2 contains common and shared channel information;

-	SystemInformationBlockType3 contains cell re-selection information, mainly related to the serving cell;

-	SystemInformationBlockType4 contains information about the serving frequency and intra-frequency neighbouring cells relevant for cell re-selection (including cell re-selection parameters common for a frequency as well as cell specific re-selection parameters);

-	SystemInformationBlockType5 contains information about other E-UTRA frequencies and inter-frequency neighbouring cells relevant for cell re-selection (including cell re-selection parameters common for a frequency as well as cell specific re-selection parameters). It also contains information about E-UTRA and NR idle/inactive measurements;

-	SystemInformationBlockType6 contains information about UTRA frequencies and UTRA neighbouring cells relevant for cell re-selection (including cell re-selection parameters common for a frequency as well as cell specific re-selection parameters);

-	SystemInformationBlockType7 contains information about GERAN frequencies relevant for cell re-selection (including cell re-selection parameters for each frequency);

-	SystemInformationBlockType8 contains information about CDMA2000 frequencies and CDMA2000 neighbouring cells relevant for cell re-selection (including cell re-selection parameters common for a frequency as well as cell specific re-selection parameters);

-	SystemInformationBlockType9 contains a home eNB name (HNB name);

-	SystemInformationBlockType10 contains an ETWS primary notification;

-	SystemInformationBlockType11 contains an ETWS secondary notification;

-	SystemInformationBlockType12 contains a CMAS warning notification;

-	SystemInformationBlockType13 contains MBMS-related information;

-	SystemInformationBlockType14 contains information about Extended Access Barring for access control;

-	SystemInformationBlockType15 contains information related to mobility procedures for MBMS reception;

-	SystemInformationBlockType16 contains information related to GPS time and Coordinated Universal Time (UTC );

-	SystemInformationBlockType17 contains information relevant for traffic steering between E-UTRAN and WLAN;

-	SystemInformationBlockType18 contains information related to sidelink communication;

-	SystemInformationBlockType19 contains information related to sidelink discovery;

-	SystemInformationBlockType20 contains information related to SC-PTM;

-	SystemInformationBlockType21 contains information related to V2X sidelink communication;

-	SystemInformationBlockType24 contains information about NR frequencies and NR neighbouring cells relevant for cell re-selection (including cell re-selection parameters common for a frequency) and for NR idle/inactive measurements;

-	SystemInformationBlockType25 contains information about UAC parameters;

-	SystemInformationBlockType26 contains additional information related to V2X sidelink communication;

-	SystemInformationBlockType27 contains assistance information for inter-RAT cell selection to NB-IoT.

-	SystemInformationBlockType28 contains information related to NR sidelink communication.

System information for NB-IoT is divided into the MasterInformationBlock-NB (MIB-NB) and a number of SystemInformationBlocks-NB (SIBs-NB):

-	MasterInformationBlock-NB defines the most essential information of the cell required to receive further system information;

-	SystemInformationBlockType1-NB contains information relevant when evaluating if a UE is allowed to access a cell and defines the scheduling of other system information blocks;

-	SystemInformationBlockType2-NB contains common radio resource configuration information;

-	SystemInformationBlockType3-NB contains cell re-selection information for intra-frequency, inter-frequency;

-	SystemInformationBlockType4-NB contains neighboring cell related information relevant for intra-frequency cell re-selection;

-	SystemInformationBlockType5-NB contains neighboring cell related information relevant for inter-frequency cell re-selection;

-	SystemInformationBlockType14-NB contains information about access barring;

-	SystemInformationBlockType15-NB contains information related to mobility procedures for MBMS reception;

-	SystemInformationBlockType16-NB contains information related to GPS time and Coordinated Universal Time (UTC);

-	SystemInformationBlockType20-NB contains information related to SC-PTM;

-	SystemInformationBlockType22-NB contains common radio resource configuration information for paging and random access procedure on non-anchor carriers;

-	SystemInformationBlockType23-NB contains common additional radio resource configuration information for random access procedure on anchor and non-anchor carriers;

-	SystemInformationBlockType27-NB contains assistance information for inter-RAT cell selection to E-UTRAN and/or GERAN.

On MBMS-dedicated cell, only system information relevant for receiving MBMS service is broadcasted. MasterInformationBlock-MBMS (MIB-MBMS) and SystemInformationBlockType1-MBMS (SIB1-MBMS) are used instead of MIB and SIB1 respectively:

-	MasterInformationBlock-MBMS defines the most essential physical layer information of the cell required to receive further system information on MBMS-dedicated cell;

-	SystemInformationBlockType1-MBMS contains information relevant for receiving MBMS service and defines the scheduling of other system information blocks on MBMS-dedicated cell;

The MIB is mapped on the BCCH and carried on BCH while all other SI messages are mapped on the BCCH and BR-BCCH, and carried on DL-SCH. Except for BL UEs, UEs in enhanced coverage and NB-IoT UEs, all other SI messages than the MIB which are dynamically carried on DL-SCH, can be identified through the SI-RNTI (System Information RNTI). Both the MIB and SystemInformationBlockType1 (SystemInformationBlockType1-BR for BL UEs and UEs in enhanced coverage) use a fixed schedule with a periodicity of 40 and 80 ms respectively. The scheduling of other SI messages is flexible and indicated by SystemInformationBlockType1 (SystemInformationBlockType1-BR for BL UEs and UEs in enhanced coverage, and SystemInformationBlockType1-NB for NB-IoT). For NB-IoT, the MIB-NB is mapped on the BCCH and carried on BCH while all other SI messages are mapped on the BCCH and carried on DL-SCH. Both the MIB-NB and SystemInformationBlockType1-NB use a fixed schedule with a periodicity of 640 and 2560 ms respectively. The MIB-NB contains all information required to acquire SIB1-NB and SIB1-NB contains all information required to acquire other SI messages.

On MBMS-dedicated cell, the MIB-MBMS and SIB1-MBMS use a fixed schedule with a periodicity of 160 ms. Additionally, SIB1-MBMS may be scheduled in additional non-MBSFN subframes indicated in MIB-MBMS.

For NB-IoT, in TDD mode, the MIB-TDD-NB is transmitted on the same NB-IoT carrier as NPSS/NSSS, SystemInformationBlockType1-NB can be transmitted on NB-IoT carrier other than the MIB-NB, and the SI messages can be transmitted on a NB-IoT carrier other than the MIB-NB. At most two NB-IoT carriers are used to transmit the MIB-NB, SystemInformationBlockType1-NB and the SI messages.

Except for NB-IoT, the eNB may schedule DL-SCH transmissions concerning logical channels other than BCCH or BR-BCCH in the same subframe as used for BCCH or BR-BCCH. The minimum UE capability restricts the BCCH or BR-BCCH mapped to DL-SCH e.g. regarding the maximum rate.

The Paging message is used to inform UEs in RRC_IDLE and UEs in RRC_CONNECTED about a system information change. For NB-IoT UEs, BL UEs, and UEs in CE, the UE is not required to detect SIB changes when in RRC_CONNECTED, and the network may release the NB-IoT UE, BL UE or UE in CE to RRC_IDLE if it wants the NB-IoT UE, BL UE or UE in CE to acquire changed SIB(s).

Except for NB-IoT, system information may also be provided to the UE by means of dedicated signalling e.g. upon handover.

END OF CHANGE