3GPP TSG-RAN WG2 #111	R2-2007599

Electronic meeting, 17th – 28th August 2020

Agenda Item:	8.2.3 Conditional PSCell change / addition

Source: 	Ericsson

Title:  	Conditional reconfigurations

Document for:	Discussion and Decision

# Introduction

In the work item for NR MR DC/CA further enhancements [1], one objective is to improve the conditional PSCell addition/change (CPAC) cases which were not covered in rel-16. The remaining CPAC cases are analyzed in this contribution.

Also, it some other conditional reconfiguration cases which were not done in rel-16 are discussed as they are considered important to have conditional reconfigurations complete in the specifications. This is also highlighted in the WID [1]:

There are some leftovers from Release 16 eDCCA and mobility enhancement WIs, which have benefits but not be completed due to limited schedule, e.g. conditional PSCell change etc. These issues can also be addressed in Release 17.

# Discussion

## Conditional PSCell addition and change (CPAC)

The WID description indicates that all remaining scenarios of PSCell addition and change should be supported in rel-17. That is also the assumption in this contribution. The assumption is also that the different scenarios should be supported for NR and for EN-DC.

Proposal 1: Support the remaining CPAC cases in rel-17, i.e.:

- Conditional PSCell addition,

- MN initiated intra-SN conditional PSCell change,

- MN initiated inter-SN conditional PSCell change,

- SN initiated inter-SN conditional PSCell change.

Proposal 2: Support all CPAC cases for NR and for EN-DC.

### Conditional PSCell Addition (MN initiated)

The conditional PSCell Addition is MN initiated as the SN is not configured yet. The flow chart for legacy PSCell Additions looks like this according to 37.340:

For the conditional case, the signalling flow needs to be adjusted. It also needs to be decided which nodes that builds the RRC message sent to the UE, whether it is the MN or the SN. For this case it seems most straightforward that the MN builds the RRC message containing the conditional reconfiguration to be applied upon reception, as the procedure is MN initiated and the MN would be the node deciding upon the conditions.

Proposal 3: The MN builds the RRC message containing the conditional reconfiguration at conditional PSCell Addition.

With the assumption that it is the MN that builds the RRC message containing the conditional reconfiguration, a flow chart for conditional PSCell Addition may look like this:

In the first step the MN receives Measurement Reports indicating that configuration of an SN addition may soon be beneficial. The MN decides to configure conditional PSCell Addition and needs to ask the SN for a configuration. The SN will reply back with an SN target configuration and possibly other information needed. The MN can then build the final message containing the conditions and the RRC message to be applied when the conditions are fulfilled.

One fundamental difference between CPA and CPC in rel-16 is that in CPA the conditional reconfiguration is provided to the UE in MN format, as an MCG configuration. Otherwise, if CPA would have to be configured as in CPC rel-16, the MN would have to provide in the SN Addition Request the configuration of the execution conditions, so that the candidate SN could generate the RRC Reconfiguration as an SCG configuration (i.e. in SN format), which is not very intuitive, requires an unnecessary discussion about how the configuration of execution conditions are provided to the SN, and creates additional burden on a candidate SN for CPA.

### Intra-SN conditional PSCell change (SN initiated) (rel-16)

In rel-16 SN-initiated intra-SN conditional PSCell change (CPC) was standardized. In this case the SN initiates the CPC and also builds the RRC message containing the conditional reconfiguration that is sent to the UE. The message contains the conditions for the CPC in terms of measID and the message to be applied when the conditions are fulfilled. The message may be passed on to the UE via the MN on SRB1 or sent directly from the SN on SRB3. In both cases, it is still the SN that creates the message.

Observation 1: In rel-16, for SN initiated intra-SN conditional PSCell change, the SN builds the RRC message with the conditional reconfiguration.

### Intra-SN conditional PSCell change (MN initiated)

In rel-16 the case with MN initiated intra-SN conditional PSCell Change was not standardized and this case needs to be done in rel-17. The legacy flow chart looks like the following according to 37.340:

For the conditional case some modifications are needed. An updated flow chart may look like this:

Firstly, the MN gets measurements in Measurement Report and decides that it may be beneficial to configure conditional PSCell Change. In this case, the MN can set the conditions for the CPC, as the procedure is MN initiated. The MN needs to ask the SN for a target SN configuration and when the reply has been received, the MN can build the RRC message containing the conditional reconfiguration.

Also here, if this case would be configured as in CPC rel-16, the MN would have to provide in the SN Addition Request the configuration of the execution conditions, so that the candidate SN could generate the RRC Reconfiguration as an SCG configuration (i.e. in SN format), which is not very intuitive, requires an unnecessary discussion about how the configuration of execution conditions are provided to the SN, and creates additional burden on a candidate SN for CPA.

Proposal 4: The MN sets the conditions at MN initiated intra-SN conditional PSCell Change.

Proposal 5: The MN builds the RRC message containing the conditional reconfiguration at MN initiated intra-SN conditional PSCell Change.

### Inter-SN conditional PSCell change (MN initiated)

The MN initiated inter-SN conditional PSCell change looks very similar as the intra-SN case, but the MN needs to ask another SN for the target configuration. An updated flow chart may look like this:

As above, the MN gets measurements in Measurement Report and decides that it may be beneficial to configure conditional PSCell Change. Also in this case, the MN can set the conditions for the CPC, as the procedure is MN initiated. The MN needs to ask the SN for a target SN configuration and when the reply has been received, the MN can build the RRC message containing the conditional reconfiguration. This seems to be the simplest solution with the same reasoning as above.

Proposal 6: The MN sets the conditions at MN initiated inter-SN conditional PSCell Change.

Proposal 7: The MN builds the RRC message containing the conditional reconfiguration at MN initiated inter-SN conditional PSCell Change.

### Inter-SN conditional PSCell change (SN initiated)

The more complicated case is the SN initiated inter-SN conditional PSCell change. The legacy flow chart for this case looks like this:

In this case, it is not obvious which node that sets the conditions and generates the RRC message containing the conditional reconfiguration. As described above, in the SN initiated intra-SN conditional PSCell change case which was standardized in rel-16, the (source) SN has the responsibility to initiate the procedure, to set the conditions and build the RRC message with the conditional reconfiguration. At SN initiated inter-SN CPC, there are several options, e.g.:

- The source SN generates CPC, i.e. sets the conditions and builds the RRC message with the conditional reconfiguration. This is the same principle as in rel-16.

- The MN generates CPC.

- The target SN generates CPC.

- The source SN generates CPC, the source SN communicates with target SN.

- The MN generates CPC, the S-SN communicates directly with the target SN.

The flow charts will most likely be different depending on which option is chosen and different network messages will need to be updated. This is more of a RAN3 discussion and it is therefore proposed that RAN2 does not make any agreements on this case in this meeting, but let RAN3 discuss it first.

Proposal 8: Wait for RAN3 to discuss SN initiated inter-SN conditional PSCell Change.

## Conditional handover and MR-DC

In rel-16 it was agreed that during CHO preparation, a target candidate can add/modify/release an SCG. In the case the target candidate MN decides to release the SN the MN simply includes in the RRCReconfiguration an MR-DC release indication.

However, legacy procedures need to be revisited for the cases where the target candidate MN determines to keep the SN and/or to modify an SCG (in case the UE is not operating in MR-DC), and/or to add an SCG (in case the UE is not operating in MR-DC).

Below we reproduce the legacy stage-2 procedure for Inter-MN handover with/without MN initiated SN change procedure.

Inter-MN handover with/without MN initiated SN change procedure

If the target candidate MN decides to keep the SN, it needs to send an SN Addition Request to the SN. That SN Addition Request may need to include an indication that this is for a conditional reconfiguration, so the SN does not expect the procedure to occur within a short time. Details should be defined by RAN3. It should also be defined whether the SN needs to distinguish the indication in an SN Addition Request in this CHO scenario from the previously discussed CPA scenario. This is similar to the case where the target candidate MN decides to add or change the SN, except that the Addition Request is transmitted to a target candidate SN that is not necessarily the S-SN.

Proposal 9: In CHO with MR-DC, a target candidate MN indicates in the SN Addition Request this is for a CHO. FFS whether that needs to be differentiated from the request due to CPA (up to RAN3).

Upon receiving the Handover Request Ack in response to a CHO related request, as defined in TS 37.340 for the case of SN release during CHO (see notes 1a and 1b in 10.8.2, TS 37.340), the step 3a and step 3b are performed only after the source MN receives an indication that the UE has successfully accessed one of the potential target gNB.

Proposal 10: In CHO with MR-DC, SN release is delayed to be performed (performed upon execution).

## Other conditional reconfiguration cases

### Conditional reconfigurations within HO/CHO Command

In R16 a restriction was added in the signalling: the network cannot include a CHO configuration within a HO command (or a CHO configuration within a CHO configuration). The consequence is that is a target gNodeB is CHO capable, and it receives measurements in the HO Request message enabling it to define a CHO configuration to be used in target after the handover, the target node would first have to wait for the UE to perform the handover to only then have a second RRC reconfiguration procedure to configure CHO.

In our view this is unnecessary and could be avoided by allowing the CHO configuration to be included in an RRCReconfiguration during a handover. In terms of signaling this means that it should be possible to include in the same message an MCG reconfiguration with sync AND a CHO configuration. While it should also be possible for a CHO target candidate to include a CHO configuration itself, there could be more uncertainties since the measurements to determine the candidates may not be up to date compared to the previous HO case, so it could be left FFS.

Proposal 11: CHO configuration in a HO command is supported. FFS Whether CHO configuration can be included in a CHO command.

### Conditional reconfigurations in RRCResume

A UE that transitions from RRC_INACTIVE to RRC_CONNECTED restores its security context to transmit an RRC Resume Request on SRB0, and refresh its security keys so it can receive an encrypted and integrity protected RRC Resume message that basically contains any configuration that could also be included in an RRC Reconfiguration message.

Hence, it should be possible to configure the UE with CHO upon a transition from RRC_INACTIVE to RRC_CONNECTED by allowing an RRC Resume to include a CHO configuration. Another related topic is the possibility to define CHO as part of the UE AS Inactive context, so that upon entering RRC_INACTIVE the UE stores the CHO configuration to be later resumed. As it’s always been assumed that most of the time UEs are suspended and resume in the same cell/Area, it should be possible to resume the same CHO configurations in most cases. However, that can be further discussed during R17, depending whether network has means to determine CHO candidates when the UE resumes e.g. before it receives measurement reports.

Proposal 12: CHO can be configured in RRC Resume. FFS whether CHO configurations are part of the UE AS Inactive context.

# Summary

RAN2 is kindly asked to discuss the following observation and proposals:

Observation 1: In rel-16, for SN initiated intra-SN conditional PSCell change, the SN builds the RRC message with the conditional reconfiguration.

Proposal 1: Support the remaining CPAC cases in rel-17, i.e.:

- Conditional PSCell addition,

- MN initiated intra-SN conditional PSCell change,

- MN initiated inter-SN conditional PSCell change,

- SN initiated inter-SN conditional PSCell change.

Proposal 2: Support all CPAC cases for NR and for EN-DC.

Proposal 3: The MN builds the RRC message containing the conditional reconfiguration at conditional PSCell Addition.

Proposal 4: The MN sets the conditions at MN initiated intra-SN conditional PSCell Change.

Proposal 5: The MN builds the RRC message containing the conditional reconfiguration at MN initiated intra-SN conditional PSCell Change.

Proposal 6: The MN sets the conditions at MN initiated inter-SN conditional PSCell Change.

Proposal 7: The MN builds the RRC message containing the conditional reconfiguration at MN initiated inter-SN conditional PSCell Change.

Proposal 8: Wait for RAN3 to discuss SN initiated inter-SN conditional PSCell Change.

Proposal 9: In CHO with MR-DC, a target candidate MN indicates in the SN Addition Request this is for a CHO. FFS whether that needs to be differentiated from the request due to CPA (up to RAN3).

Proposal 10: In CHO with MR-DC, SN release is delayed to be performed (performed upon execution).

Proposal 11: CHO configuration in a HO command is supported. FFS Whether CHO configuration can be included in a CHO command.

Proposal 12: CHO can be configured in RRC Resume. FFS whether CHO configurations are part of the UE AS Inactive context.

# References

- RP-201040, Work Item on Further Multi-RAT Dual-Connectivity enhancements, Huawei