3GPP TSG-RAN WG2 Meeting #111	R2-2007655

# Electronic meeting, 17 – 28 August 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.321 | CR | 1495 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial changes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh3-Core, LTE_eMTC5-Core, 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2020-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | After the CR implementation following RAN#88 some minor mistakes were found. Additional editorial changes are also added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Mistakes from CR implementation: - Clause 5.4.5a: A removed comma from CR 1479r1 was not implemented. In this case the absence of the comma matters and is removed. It should also be noted that CR 1479r1 is Cat A corresponding to Cat F CR 1478r1 which was implemented correctly (i.e. removing the comma). - Clause 5.4.7.2: The added text "be configured with" from CR 1472r2 was not implemented and is added. - Clause 5.25: When implementing CR 1473r2 a colon was removed erroneously and is added back. Other editorial changes: - Clause 3.2: Capitalization of "stratum". - Clause 5.3.1: Removal of additional "g" from "uplinkg" and removal of "C-" from "Preconfigured Uplink Resource C-RNTI". - Clause 5.4.2.2: A reference ("clause 5.x.1.3.2 of TS 38.321") is corrected to "clause 5.22.1.3.1a of TS 38.321". Impact analysis Impacted functionality: Miscellaneous minor corrections to TS 36.321. Inter-operability: If the network is implemented according to the CR and the UE is not, there will not be inter-operability problems. If the UE is implemented according to the CR and the network is not, there will not be inter-operability problems. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification does not reflect agreed CRs and contains some editorial errors, but no major impacts to functionality are foreseen. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 5.3.1, 5.4.2.2, 5.4.5a, 5.4.7.2, 5.25 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AS	Access Stratum

AUL	Autonomous Uplink

BL	Bandwidth reduced Low complexity

BR	Bandwidth Reduced

BSR	Buffer Status Report

C-RNTI	Cell RNTI

CBR	Channel Busy Ratio

CC-RNTI	Common Control RNTI

CG	Cell Group

CQI	Channel Quality Indicator

CRI	CSI-RS Resource Indicator

CSI	Channel State Information

DAPS	Dual Active Protocol Stack

DCQR	Downlink Channel Quality Report

DRB	Data Radio Bearer

EDT	Early Data Transmission

eIMTA	Enhanced Interference Management and Traffic Adaptation

eIMTA-RNTI	Enhanced Interference Management and Traffic Adaptation - RNTI

E-UTRA	Evolved UMTS Terrestrial Radio Access

E-UTRAN	Evolved UMTS Terrestrial Radio Access Network

G-RNTI	Group RNTI

H-SFN	Hyper SFN

MAC	Medium Access Control

MCG	Master Cell Group

M-RNTI	MBMS RNTI

MPDCCH	MTC Physical Downlink Control Channel

LCG	Logical Channel Group

NB-IoT	Narrow Band Internet of Things

NPDCCH	Narrowband Physical Downlink Control Channel

NPDSCH	Narrowband Physical Downlink Shared channel

NPRACH	Narrowband Physical Random Access Control Channel

NPUSCH	Narrowband Physical Uplink Shared channel

PCell	Primary Cell

PSCell	Primary Secondary Cell

PHR	Power Headroom Report

PMI	Precoding Matrix Index

PPPP	ProSe Per-Packet Priority

P-RNTI	Paging RNTI

ProSe	Proximity-based Services

pTAG	Primary Timing Advance Group

PTI	Precoding Type Indicator

PUR	Preconfigured Uplink Resource

RA-RNTI	Random Access RNTI

RAI	Release Assistance Indication

RI	Rank Indicator

RN	Relay Node

RNTI	Radio Network Temporary Identifier

SCell	Secondary Cell

SC-FDM	Single-Carrier Frequency Division Multiplexing

SCG	Secondary Cell Group

SCI	Sidelink Control Information

SC-N-RNTI	Single Cell Notification RNTI

SC-PTM	Single Cell Point to Multipoint

SC-RNTI	Single Cell RNTI

SI-RNTI	System Information RNTI

SL	Sidelink

SL-RNTI	Sidelink RNTI

SL-V-RNTI	Sidelink V2X RNTI

SR	Scheduling Request

SRS	Sounding Reference Symbols

SRS-TPC-RNTI	Sounding Reference Symbols-Transmit Power Control-RNTI

SpCell	Special Cell

sTAG	Secondary Timing Advance Group

sTTI	Slot or subslot TTI

TAG	Timing Advance Group

TB	Transport Block

TPC-PUCCH-RNTI	Transmit Power Control-Physical Uplink Control Channel-RNTI

TPC-PUSCH-RNTI	Transmit Power Control-Physical Uplink Shared Channel-RNTI

V2X	Vehicle-to-Everything

NEXT CHANGE

### 5.3.1	DL Assignment reception

Downlink assignments transmitted on the PDCCH indicate if there is a transmission on a DL-SCH for a particular MAC entity and provide the relevant HARQ information.

When the MAC entity has a C-RNTI, Semi-Persistent Scheduling C-RNTI, Preconfigured Uplink Resource RNTI or Temporary C-RNTI, the MAC entity shall for each TTI during which it monitors PDCCH and for each Serving Cell:

-	if a downlink assignment for this TTI and this Serving Cell has been received on the PDCCH for the MAC entity's C-RNTI, Preconfigured Uplink Resource RNTI, or Temporary C-RNTI:

-	if this is the first downlink assignment for this Temporary C-RNTI; or

-	if this is the first downlink assignment corresponding to uplink transmission using previous preconfigured uplink grant for this Preconfigured Uplink Resource RNTI:

-	consider the NDI to have been toggled.

-	if the downlink assignment is for the MAC entity's C-RNTI and if the previous downlink assignment indicated to the HARQ entity of the same HARQ process was either a downlink assignment received for the MAC entity's Semi-Persistent Scheduling C-RNTI or a configured downlink assignment:

-	consider the NDI to have been toggled regardless of the value of the NDI.

-	indicate the presence of a downlink assignment and deliver the associated HARQ information to the HARQ entity for this TTI.

-	else, if a downlink assignment for this TTI has been received for this Serving Cell on the PDCCH for the MAC entity's Semi-Persistent Scheduling C-RNTI:

-	if the NDI in the received HARQ information is 1:

-	consider the NDI not to have been toggled;

-	indicate the presence of a downlink assignment and deliver the associated HARQ information to the HARQ entity for this TTI.

-	else, if the NDI in the received HARQ information is 0:

-	if PDCCH contents indicate SPS release:

-	clear the configured downlink assignment (if any);

-	if the timeAlignmentTimer, associated with the TAG containing the serving cell on which the acknowledgement for the downlink SPS release is to be transmitted, is running:

-	indicate a positive acknowledgement for the downlink SPS release to the physical layer.

-	else:

-	store the downlink assignment and the associated HARQ information as configured downlink assignment;

-	initialise (if not active) or re-initialise (if already active) the configured downlink assignment to start in this TTI, or in TTI according to N=0 in clause 5.10.1 for short TTI, and to recur according to rules in clause 5.10.1;

-	set the HARQ Process ID to the HARQ Process ID associated with this TTI;

-	consider the NDI bit to have been toggled;

-	indicate the presence of a configured downlink assignment and deliver the stored HARQ information to the HARQ entity for this TTI.

-	else, if a downlink assignment for this TTI has been configured for this Serving Cell and there is no measurement gap in this TTI and there is no Sidelink Discovery Gap for Reception in this TTI; and

-	if this TTI is not an MBSFN subframe or the MAC entity is configured with transmission mode tm9 or tm10:

-	instruct the physical layer to receive, in this TTI, transport block on the DL-SCH according to the configured downlink assignment and to deliver it to the HARQ entity;

-	set the HARQ Process ID to the HARQ Process ID associated with this TTI;

-	consider the NDI bit to have been toggled;

-	indicate the presence of a configured downlink assignment and deliver the stored HARQ information to the HARQ entity for this TTI.

-	if the MAC entity is configured with rach-Skip or rach-SkipSCG and a UE Contention Resolution Identity MAC control element for this TTI has been received on the PDSCH indicated by the PDCCH of the SpCell addressed to the C-RNTI:

-	indicate to upper layer the successful reception of a PDCCH transmission addressed to the C-RNTI.

For configured downlink assignments, the HARQ Process ID associated with this TTI is derived from the following equation:

-	if the TTI is a subframe TTI:

-	HARQ Process ID = [floor(CURRENT_TTI/semiPersistSchedIntervalDL)] modulo numberOfConfSPS-Processes,

where CURRENT_TTI=[(SFN * 10) + subframe number].

-	else:

-	HARQ Process ID = [floor(CURRENT_TTI/semiPersistSchedIntervalDL-sTTI)] modulo numberOfConfSPS-Processes-sTTI,

where CURRENT_TTI = [(SFN * 10 * sTTI_Number_Per_Subframe) + subframe number * sTTI_Number_Per_Subframe + sTTI_number]. Refer to 5.10.1 for sTTI_Number_Per_Subframe and sTTI_number.

For BL UEs or UEs in enhanced coverage, CURRENT_TTI refers to the TTI where first transmission of repetition bundle takes place.

When the MAC entity needs to read BCCH or BR-BCCH, the MAC entity may, based on the scheduling information from RRC:

-	if the UE is a BL UE or a UE in enhanced coverage:

-	the redundancy version of the received downlink assignment for this TTI is determined by RVK = ceiling(3/2*k) modulo 4, where k depends on the type of system information message.

-	for SystemInformationBlockType1-BR

-	if number of repetitions for PDSCH carrying SystemInformationBlockType1-BR is 4, k = floor(SFN/2) modulo 4, where SFN is the system frame number.

-	else if number of repetitions for PDSCH carrying SystemInformationBlockType1-BR is 8, k = SFN modulo 4, where SFN is the system frame number.

-	else if number of repetitions for PDSCH carrying SystemInformationBlockType1-BR is 16, k = (SFN*10+i) modulo 4, where SFN is the system frame number, and i denotes the subframe within the SFN.

NOTE:	the set of subframes for SystemInformationBlockType1-BR when number of repetitions for PDSCH is 16 are given by Table 6.4.1-2 in TS 36.211 [7].

-	for SystemInformation-BR messages, k=i modulo 4, i =0,1,…, nsw–1, where i denotes the subframe number within the SI window nsw;

-	indicate a downlink assignment and redundancy version for the dedicated broadcast HARQ process to the HARQ entity for this TTI.

-	else if a downlink assignment for this TTI has been received on the PDCCH for the SI-RNTI, except for NB-IoT;

-	if the redundancy version is not defined in the PDCCH format:

-	the redundancy version of the received downlink assignment for this TTI is determined by RVK = ceiling(3/2*k) modulo 4, where k depends on the type of system information message: for SystemInformationBlockType1 message, k = (SFN/2) modulo 4, where SFN is the system frame number; for SystemInformation messages, k=i modulo 4, i =0,1,…, nsw–1, where i denotes the subframe number within the SI window nsw;

-	indicate a downlink assignment and redundancy version for the dedicated broadcast HARQ process to the HARQ entity for this TTI.

When the MAC entity has SC-RNTI and/or G-RNTI, the MAC entity shall for each TTI during which it monitors PDCCH for SC-RNTI as specified in TS 36.331 [8] for UEs other than NB-IoT UEs, BL UEs or UEs in enhanced coverage and in clause 5.7a for NB-IoT UEs, BL UEs or UEs in enhanced coverage and for G-RNTI as specified in clause 5.7a and for each Serving Cell and cell that may be additionally configured as a Serving Cell according to the UE capabilities:

-	if a downlink assignment for this TTI and this Serving Cell has been received on the PDCCH for the MAC entity's SC-RNTI or G-RNTI:

-	attempt to decode the received data.

-	if the data which the MAC entity attempted to decode was successfully decoded for this TB:

-	deliver the decoded MAC PDU to the disassembly and demultiplexing entity.

NEXT CHANGE

#### 5.4.2.2	HARQ process

Each HARQ process is associated with a HARQ buffer.

For synchronous HARQ, each HARQ process shall maintain a state variable CURRENT_TX_NB, which indicates the number of transmissions that have taken place for the MAC PDU currently in the buffer, and a state variable HARQ_FEEDBACK, which indicates the HARQ feedback for the MAC PDU currently in the buffer. When the HARQ process is established, CURRENT_TX_NB shall be initialized to 0.

The sequence of redundancy versions is 0, 2, 3, 1. The variable CURRENT_IRV is an index into the sequence of redundancy versions. This variable is up-dated modulo 4. For serving cells configured with pusch-EnhancementsConfig, BL UEs or UEs in enhanced coverage see clause 8.6.1 in TS 36.213 [2] for the sequence of redundancy versions and redundancy version determination. For NB-IoT UEs see clause 16.5.1.2 in TS 36.213 [2] for the sequence of redundancy versions and redundancy version determination. For an SPS configuration with totalNumberPUSCH-SPS-STTI-UL-Repetitions or totalNumberPUSCH-SPS-UL-Repetitions (TS 36.331 [8]), the redundancy version for each transmission within a bundle are determined by rv-SPS-STTI-UL-Repetitions or rv-SPS-UL-Repetitions in the SPS configuration (TS 36.331 [8]).

For NB-IoT UEs, BL UEs or UEs in enhanced coverage for UL_REPETITION_NUMBER for Mode B operation, the same redundancy version is used multiple times before cycling to the next redundancy version as specified in clauses 16.5.1.2, 8.6.1 and 7.1.7.1 in TS 36.213 [2].

New transmissions are performed on the resource and with the MCS indicated on PDCCH or Random Access Response. Adaptive retransmissions are performed on the resource and, if provided, with the MCS indicated on PDCCH. Non-adaptive retransmission is performed on the same resource and with the same MCS as was used for the last made transmission attempt.

For synchronous HARQ, the MAC entity is configured with a maximum number of HARQ transmissions and a maximum number of Msg3 HARQ transmissions by RRC: maxHARQ-Tx and maxHARQ-Msg3Tx respectively. For transmissions on all HARQ processes and all logical channels except for transmission of a MAC PDU stored in the Msg3 buffer, the maximum number of transmissions shall be set to maxHARQ-Tx. For transmission of a MAC PDU stored in the Msg3 buffer, the maximum number of transmissions shall be set to maxHARQ-Msg3Tx.

For autonomous HARQ, each HARQ process shall maintain a state variable HARQ_FEEDBACK, which indicates the HARQ feedback for the MAC PDU currently in the buffer, and a timer aul-RetransmissionTimer which prohibits new transmission or retransmission for the same HARQ process on the configured autonomous uplink when the timer is running.

When the HARQ feedback is received for this TB, the HARQ process shall:

-	set HARQ_FEEDBACK to the received value;

-	if running, stop the aul-RetransmissionTimer.

When an uplink grant addressed to C-RNTI is received for this HARQ process and if the UL HARQ operation is autonomous, the HARQ process shall:

-	if running, stop the aul-RetransmissionTimer.

When PUSCH transmission is performed for this TB and if the uplink grant is a configured grant for the MAC entity's AUL C-RNTI, the HARQ process shall:

-	start or restart the aul-RetransmissionTimer.

If the HARQ entity requests a new transmission, the HARQ process shall:

-	if UL HARQ operation is synchronous:

-	set CURRENT_TX_NB to 0;

-	set HARQ_FEEDBACK to NACK;

-	set CURRENT_IRV to 0;

-	else:

-	if UL HARQ operation is autonomous asychronous:

-	set HARQ_FEEDBACK to NACK.

-	if the uplink grant was addressed to the AUL C-RNTI:

-	set CURRENT_IRV to 0.

-	else:

-	set CURRENT_IRV to the index corresponding to the redundancy version value provided in the HARQ information;

-	store the MAC PDU in the associated HARQ buffer;

-	store the uplink grant received from the HARQ entity;

-	generate a transmission as described below.

If the HARQ entity requests a retransmission, the HARQ process shall:

-	if UL HARQ operation is synchronous:

-	increment CURRENT_TX_NB by 1;

-	if the HARQ entity requests an adaptive retransmission:

-	store the uplink grant received from the HARQ entity;

-	set CURRENT_IRV to the index corresponding to the redundancy version value provided in the HARQ information;

-	if UL HARQ operation is synchronous; or

-	if UL HARQ operation is autonomous:

-	set HARQ_FEEDBACK to NACK;

-	generate a transmission as described below.

-	else if the HARQ entity requests a non-adaptive retransmission:

-	if UL HARQ operation is asynchronous or HARQ_FEEDBACK = NACK:

-	if both skipUplinkTxSPS and fixedRV-NonAdaptive are configured and the uplink grant of the initial transmission of this HARQ process was performed on a configured grant and UL HARQ operation is not autonomous; or

-	if the uplink grant is a preallocated uplink grant:

-	set CURRENT_IRV to 0;

-	else if UL HARQ operation is autonomous:

-	set CURRENT_IRV to the index corresponding to the redundancy version value selected by the UE implementation.

-	generate a transmission as described below.

NOTE 1:	When receiving a HARQ ACK alone, the MAC entity keeps the data in the HARQ buffer.

NOTE 2:	When no UL-SCH transmission can be made due to the occurrence of a measurement gap or a Sidelink Discovery Gap for Transmission, or prioritization of V2X sidelink communication transmission described in clause 5.14.1.2.2, no HARQ feedback can be received and a non-adaptive retransmission follows.

NOTE 3:	For asynchronous HARQ operation, UL retransmissions are triggered only by adaptive retransmission grants, except for retransmissions within a bundle.

To generate a transmission, the HARQ process shall:

-	if the MAC PDU was obtained from the Msg3 buffer; or

-	if Sidelink Discovery Gaps for Transmission are not configured by upper layers, and there is no measurement gap at the time of the transmission and, in case of retransmission, the retransmission does not collide with a transmission for a MAC PDU obtained from the Msg3 buffer in this TTI; or

-	if Sidelink Discovery Gaps for Transmission are configured by upper layers, and there is no measurement gap at the time of the transmission and, in case of retransmission, the retransmission does not collide with a transmission for a MAC PDU obtained from the Msg3 buffer, and there is no Sidelink Discovery Gap for Transmission in this TTI; or

-	if Sidelink Discovery Gaps for Transmission are configured by upper layers, and there is no measurement gap at the time of the transmission and, in case of retransmission, the retransmission does not collide with a transmission for a MAC PDU obtained from the Msg3 buffer, and there is a Sidelink Discovery Gap for Transmission, and there is no configured grant for transmission on SL-DCH in this TTI:

-	if there is neither transmission of V2X sidelink communication on SL-SCH nor transmission of NR sidelink communication in this TTI; or

-	if the transmission of the MAC PDU is prioritized over sidelink transmission:

-	instruct the physical layer to generate a transmission according to the stored uplink grant with the redundancy version corresponding to the CURRENT_IRV value;

-	increment CURRENT_IRV by 1 if UL HARQ operation is not autonomous;

-	if UL HARQ operation is synchronous and there is a measurement gap or Sidelink Discovery Gap for Reception at the time of the HARQ feedback reception for this transmission and if the MAC PDU was not obtained from the Msg3 buffer:

-	set HARQ_FEEDBACK to ACK at the time of the HARQ feedback reception for this transmission.

After performing above actions, if UL HARQ operation is synchronous the HARQ process then shall:

-	if CURRENT_TX_NB = maximum number of transmissions – 1:

-	flush the HARQ buffer;

The transmission of the MAC PDU is prioritized over sidelink transmission if one of the following conditions is met:

-	if there are both a configured grant for transmission of V2X sidelink communication on SL-SCH in this TTI and a sidelink grant for transmission of NR sidelink communication as described in clause 5.22.1.1 of TS 38.321 [24] at the time of the transmission, and neither the transmissions of V2X sidelink communication is prioritized as described in clause 5.14.1.2.2 nor the transmission of NR sidelink communication is prioritized as described in clause 5.22.1.3.1 of TS 38.321 [24]; or

-	if there are both a configured grant for transmission of V2X sidelink communication on SL-SCH in this TTI and a sidelink grant for transmission of NR sidelink communication as described in clause 5.22.1.1 of TS 38.321 [24] at the time of the transmission, and the MAC entity is able to perform this UL transmission simultaneously with both the transmissions of V2X sidelink communication which are prioritized as described in clause 5.14.1.2.2 and the transmission of NR sidelink communication which is prioritized as described in clause 5.22.1.3.1a of TS 38.321 [24]; or

-	if there is only configured grant(s) for transmission of V2X sidelink communication on SL-SCH in this TTI, and either none of the transmissions of V2X sidelink communication is prioritized or the MAC entity is able to perform this UL transmission and the transmissions of V2X sidelink communication which are prioritized as described in clause 5.14.1.2.2 simultaneously; or

-	if there is only a sidelink grant for transmission of NR sidelink communication in this TTI as described in clause 5.22.1.1 of TS 38.321 [24], and either no transmission of NR sidelink communication is prioritized as described in clause 5.22.1.3.1a of TS 38.321 [24] or the MAC entity is able to perform this UL transmission simultaneously with the transmission of NR sidelink communication which is prioritized as described in clause 5.22.1.3.1a of TS 38.321 [24] simultaneously:

NOTE 4:	Among the UL transmissions where the MAC entity is able to perform all transmissions of V2X sidelink communication prioritized simultaneously, if there are more than one UL transmission which the MAC entity is not able to perform simultaneously, it is up to UE implementation whether this UL transmission is performed.

NOTE 5:	Among the UL transmissions that the MAC entity is able to perform simultaneously with the transmission of NR sidelink communication prioritized, if there are more than one UL transmission which the MAC entity is not able to perform simultaneously, it is up to UE implementation whether this UL transmission is performed.

NOTE 6:	Among the UL transmissions where the MAC entity is able to perform all transmissions of V2X sidelink communication prioritized simultaneously with the transmission of NR sidelink communication prioritized, if there are more than one UL transmission which the MAC entity is not able to perform simultaneously, it is up to UE implementation whether this UL transmission is performed.

NOTE 7:	If there is a sidelink grant for transmission of NR sidelink communication in this TTI as described in clause 5.22.1.1 of TS 38.321 [24] and the MAC entity is not able to perform this UL transmission simultaneously with the transmission of NR sidelink communication, and prioritization-related information is not available prior to the time of the transmission due to processing time restriction, it is up to UE implementation whether this UL transmission is performed.

NEXT CHANGE

### 5.4.5a	Data Volume and Power Headroom Reporting

The Data Volume and Power Headroom reporting procedure is only applicable for NB-IoT UEs and is used to provide the serving eNB with information about the amount of data available for transmission in the UL buffers associated with the MAC entity, and to provide the serving eNB with information about the difference between the nominal UE maximum transmission power and the estimated transmission power for UL-SCH transmission for the Serving Cell. The reporting is done using the DPR MAC control element, which is sent in Msg3 together with a CCCH SDU. For EDT, the Data Volume in DPR MAC control element is set to zero.

If enhancedPHR is configured and the UE supports extended power headroom reporting, the UE shall:

-	if the UE supports power class 14dBm and the MAC entity considers itself to be in enhanced coverage level other than 0:

-	report power headroom level using the DPR MAC control element;

-	else:

-	report extended power headroom level using the DPR MAC control element for Extended Power Headroom level reporting.

NEXT CHANGE

#### 5.4.7.2	Maintenance of PUR Uplink Time Alignment

MAC entity may be configured with timer pur-TimeAlignmentTimer by upper layers as specified in TS 36.331 [8], clause 5.3.8.3.

The MAC entity shall:

-	when pur-TimeAlignmentTimer configuration is received from upper layers:

-	start pur-TimeAlignmentTimer¸ if not running.

-	when pur-TimeAlignmentTimer is released by upper layers:

-	stop the pur-TimeAlignmentTimer, if running.

-	when a Timing Advance Command MAC control element is received or PDCCH indicates timing advance adjustment as specified in TS 36.212 [5]:

-	apply the Timing Advance Command or the timing advance adjustment;

-	start or restart the pur-TimeAlignmentTimer, if configured.

Upon request from upper layers, MAC entity shall indicate whether pur-TimeAlignmentTimer is running.

NEXT CHANGE

## 5.25	Transmission of Downlink Channel Quality Report

The MAC entity of a BL UE or UE in enhanced coverage may be configured by upper layers to report DL channel quality in Msg3. DL channel quality in Msg3 in RRC_CONNECTED is not reported.

If the UE is a BL UE or UE in enhanced coverage or an NB-IoT UE, a Downlink Channel Quality Report (DCQR) shall be triggered if any of the following events occur:

-	DCQR Command MAC control element is received, in which case the DCQR is referred below to as "Regular DCQR";

-	for BL UE or UE in enhanced coverage, transmission of DCQR in Msg3 is configured by upper layers in mpdcch-CQI-Reporting, in which case DCQR is referred below to as "Msg3 DCQR".

If any type of DCQR has been triggered:

-	start performing DL channel quality measurements according to TS 36.133 [9].

If "Regular DCQR" has been triggered:

-	if an uplink grant has been received on the PDCCH for MAC entity's C-RNTI:

-	instruct the Multiplexing and Assembly procedure to generate a DCQR and AS RAI MAC control element as defined in clause 6.1.3.19;

-	cancel the triggered "Regular DCQR".

If "Msg3 DCQR" has been triggered:

-	if an uplink grant has been received on the PDCCH for MAC entity's RA-RNTI:

-	if the allocated resources can accommodate a DCQR and AS RAI MAC control element plus its subheader as a result of logical channel prioritization:

-	instruct the Multiplexing and Assembly procedure to generate a DCQR and AS RAI MAC control element as defined in clause 6.1.3.19;

-	else if the uplink grant is not for EDT:

-	if configured by upper layers in mpdcch-CQI-Reporting, use R and F2 fields in the MAC PDU subheader, to transmit the measurement outcome, as defined in clause 6.2.1;

-	cancel the triggered "Msg3 DCQR".