3GPP TSG-RAN WG2 Meeting #111 electronic	R2-2007770

Online, August 17th – 28th, 2020

Agenda Item:	8.13.3 MDT Scope and requirements

Source:	Huawei, HiSilicon

Title:	Discussion on MDT enhancements

Document for: Discussion and Decision

Introduction

In RAN #88-e meeting, the new WID [1] on enhancement of data collection for SON/MDT in NR and EN-DC was agreed. The objective of WID includes the enhancement of MDT as following:

- Support of data collection for MDT features for identified use cases, including 2-step RACH optimization and leftovers of Rel-16 SON/MDT WI (MDT enhancements and MDT for MR-DC) [RAN2, RAN3, RAN4] - Enhancement of logged and immediate MDT (including coexistence with IDC) [RAN2, RAN3] - Enhancement of reporting e.g. RLF and accessibility measurements, Successful Handover reporting [RAN2, RAN3]. - Specification of MDT for MR-DC [RAN2, RAN3, RAN4] - Specification of L2 measurements, if needed [RAN2, RAN3]

In this contribution, we will discuss the details of the MDT enhancements.

Discussion

IDC

In TS 37.320, the following text is about how the UE should handle logged MDT in IDC scenario. This text is mainly about logged MDT in LTE.

When an E-UTRA UE detects an in-device coexistence problem that may affect the logged measurement results, the UE shall stop measurement logging, indicate in the log that an in-device coexistence problem has occurred, and keep the duration timer running. When the in-device coexistence problem is no longer present, and the duration timer has not expired, the logging resumes, with a leap in time stamp.

Also TS 36.331 specifies the UE behaviour during the logged MDT procedure.

While T330 is running, the UE shall: 1> if measurement logging is suspended: 2> if during the last logging interval the IDC problems detected by the UE is resolved, resume measurement logging; 1> if not suspended, perform the logging in accordance with the following: 2> if targetMBSFN-AreaList is included in VarLogMeasConfig: …… 2> else if: ….. 2> when adding a logged measurement entry in VarLogMeasReport, include the fields in accordance with the following: 3> if the UE detected IDC problems during the last logging interval: 4> if measResultServCell in VarLogMeasReport is not empty: 5> include InDeviceCoexDetected; 5> suspend measurement logging from the next logging interval; 4> else: 5> suspend measurement logging; NOTE 1A: The UE may detect the start of IDC problems as early as Phase 1 as described in clause 23.4 of TS 36.300 [9]. 3> set the relativeTimeStamp to indicate the elapsed time since the moment at which the logged measurement configuration was received;

In NR, IDC has been introduced in Rel-16. It may be possible that a UE is performing logged MDT in NR but suddenly an IDC problem occurs. We think that one simple way to address this is to follow LTE design, and the above text for LTE could be also considered for NR MDT.

Proposal 1: Add the IDC problems detection in the logged MDT. Take the principle of LTE as the baseline.

MDT enhancement in MR-DC

2.2.1 Immediate MDT

In R16, immediate MDT is supported for EN-DC. MN and SN can independently configure and receive measurement from the UE. TS 37.320 has specified the principles for EN-DC.

In signalling based immediate MDT, MME provides MDT configuration for both MN and SN towards MN including multi RAT SN configuration, specifically E-UTRA and NR MDT configuration. MN then forwards the NR MDT configuration towards SN (EN-DC scenario, SN is always NR). In management-based immediate MDT, OAM provides the MDT configuration to both MN and SN independently. For both MN and SN, Management based MDT should not overwrite signalling based MDT. For immediate MDT configuration, MN and SN can independently configure and receive measurement from the UE.

Observation 1: R16 supports the immediate MDT for EN-DC. The NR SN can configure and receive immediate measurement.

In our understanding, for NR-DC there is no impact on RAN2 because RAN2 has supported in Uu. For management-based immediate MDT, the NG-RAN can receive the MDT configuration from the OAM. For signalling based immediate MDT, the MN can forward the immediate MDT configuration to the SN. The CN only sends one immediate MDT configuration to the MN because the MN and SN belong to the same RATs. This is different to EN-DC in which the MN and SN belong to the different RATs.

Proposal 2: Support the immediate MDT for NR-DC.

- For signalling based immediate MDT, AMF only provides only one MDT configuration to the MN. The MN forwards the configuration to the SN.

- For management based immediate MDT, OAM provides the MDT configuration to both MN and SN independently.

In R16, RAN2 supports the following immediate MDT measurements in NR SA:

⁻	M1: DL signal quantities measurement results for the serving cell and for intra-frequency/Inter-frequency/inter-RAT neighbour cells, including cell/beam level measurement for NR cells only

⁻	M2: Power Headroom measurement by UE

⁻	M4: Data Volume measurement separately for DL and UL, per DRB per UE

⁻	M5: Average UE throughout measurement separately for DL and UL, per DRB per UE and per UE for the DL, per DRB per UE and per UE for the UL, by gNB

⁻	M6: Packet Delay measurement separately for DL and UL, per DRB per UE

⁻	M7: Packet loss rate measurement separately for DL and UL, per DRB per UE

⁻	M8: RSSI measurement by UE (for WLAN/Bluetooth measurement).

⁻	M9: RTT Measurement by UE (for WLAN measurement).

In R16, the M1/M2/M4/M8/M9 are supported in EN-DC (i.e. the SN can configure these measurements). For these measurements, there is no impact between MN and SN. Therefore we think these measurements can also be supported in NR-DC.

Proposal 3: Support the M1/M2/M4/M8/M9 in NR-DC.

In R16, RAN2 also agreed that the M5~M7 do not apply to EN-DC SN terminated MCG/split bearers and MN terminated SCG/split bearers. In R16, RAN2 supports the delay measurement of RAN part for MN terminated MCG bearer and SN terminated SCG bearer. The RAN part of DL delay measurement comprises D1 (the DL delay in over-the-air interface), D2 (the DL delay in gNB-DU), D3 (the DL delay on F1-U) and D4 (the DL delay in CU-UP). The RAN part of UL delay measurement comprises D1 (UL PDCP packet average delay), D2.1 (average over-the-air interface packet delay), D2.2 (average RLC packet delay), D2.3 (average delay UL on F1-U) and D2.4 (average PDCP re-ordering delay). The D1 of the UL delay is measured by the UE. Others are measured by the gNB.

D1 measurement for MN terminated MCG bearer is configured by and reported to MN. D1 measurement for SN terminated SCG bearer is configured by and reported to SN. In our understanding, for these bearers, NR-DC can use the same principles of EN-DC

Proposal 4: Support the delay measurement for the MN terminated MCG bearers and SN terminated SCG bearers in NR-DC. The configuration and reporting principles in NR-DC are same as the principles in EN-DC.

R16 does not support the SN terminated MCG/split bearers and MN terminated SCG/split bearers. In our understanding, operators will use split bearers in the real network in order to effectively use the radio resources of two nodes. Also, RAN2 agreed to discuss it in R17 in RAN2#109-e meeting. If RAN2 agreed to support the delay measurements of MN/SN terminated split bearers, it is easy to support the MN terminated SCG bearer and SN terminated MCG bearers.

=> UL delay measurement is not supported for split bearer(s) for EN-DC case in R16. It should be discussed in R17.

Proposal 5: Support the delay measurement for the SN terminated MCG/split bearers and MN terminated SCG/split bearers in EN-DC and NR-DC.

For split bearers, both MCG and SCG radio resources are involved. The packets of one DRB are transmitted via two paths. The latency of these two paths are different.

Figure 1 Split bearer in MR-DC

For the split bearers, the node hosting the PDCP entity receives some packets from another node via the Xn/X2(i.e. there is one additional Xn/X2 latency for these packets). Therefore we think the UL scheduling latency (including HARQ transmission delay and RLC delay) in these two nodes are different in order to ensure the same latency requirement. For example, for the MN terminated split bearer, the SN should schedule the UL packets of this DRB more quickly than the MN. Therefore we think the D1 of these two paths are also different.

Same to the UL delay, the DL scheduling latency (including HARQ transmission delay and RLC delay) in these two nodes are also different.

Observation 2: The scheduling latency of one split bearer are different in the two nodes of MR-DC.

Therefore we think the delay of split bearers should consider the latency of these two paths in order to obtain a more accurate latency measurement.

For the UL delay of a non-split bearer, UE measures D1 and reports the average of D1 to gNB in RRC. gNB measures the D2 and derives UL delay as D1+D2. Since the scheduling latency is different in the two paths, we think the UE should report two D1s in MR-DC. The node hosting the PDCP entity measures the D2 of its path. The corresponding node measures the HARQ (re)transmission delay and RLC delay to the node hosting the PDCP entity. Then, the node hosting the PDCP entity separately derives the UL delays of two paths and derives the UL delays of this DRB based on the UL delays of two paths. During the discussions in R16, for the split bearers, there are three options.

Option 1: UE reports a single D1 value to the node where it receives the measurement configure.

Option 2: MN and SN can independently configure the UE with D1 measurements in the split bearer. UE reports the D1 to each node respectively.

Option 3: only the node hosting the PDCP entity configure the D1 measurement. UE reports two D1s to the node hosting the PDCP entity in one RRC message.

As discussed in the above, we think option 1 cannot obtain the accurate latency results because the scheduling latency is different in the two paths. For option 2, the network configures two D1 measurement messages and the UE reports the results in two RRC messages. The UE periodically reports the results. Therefore the option 2 increases the signalling overhead. For option 3, the network only sends one D1 measurement message and the UE reports the two D1 in one RRC message. Option 3 has less signalling overhead. Also RAN2 agreed that the node hosting the PDCP entity configures the D1 measurements for the MN terminated MCG bearer and the SN terminated SCG bearer. The principle of option 3 is align with the principles of R16.

Proposal 6: For the split bearer (i.e. MN terminated split bearer and SN terminated split bearer), only the node hosting the PDCP entity configure the D1 measurement. UE reports two D1s to the node hosting the PDCP entity in one RRC message.

For a MN terminated SCG bearer or SN terminated MCG bearer, we also think the node hosting the PDCP entity can configure the D1 measurement same to the principle of R16.

Proposal 7: For the MN terminated SCG bearer and SN terminated MCG bearer, the node hosting the PDCP entity configure the D1 measurement.

For a split bearer configured with PDCP duplication, the packets of these two paths are the same therefore the node hosting the PDCP entity can use the min value of these two paths. For a split bearer configured without PDCP duplication, the packets of these two paths are not the same, therefore the node hosting the PDCP entity can use the average value of these two paths.

For the DL delay, the node hosting the PDCP entity measures the DL latency of its path. The corresponding node measures the DL latency (i.e. the latency of scheduling the RLC SDU) in it and reports to the node hosting the PDCP entity. Then, the node hosting the PDCP entity separately derives the DL delays of two paths and derives the DL delays of this DRB based on the DL delays of two paths. Similar to the UL delay, the node hosting the PDCP entity can use the min value and average value of these two paths to derive the DL delay for the split bearer with PDCP duplication and without PDCP duplication respectively.

Proposal 8: For the split bearer, the node hosting the PDCP entity derives the delay of the split bearers based on the delay of two paths.

- For split bearer with PDCP duplication, the final delay is the min value of measured results of two paths

- For split bearer without PDCP duplication, the final delay is the average value of measured results of two paths

2.2.2 Logged MDT

In R16, the SN will not configure the logged MDT for the UE. RAN2 has agreed to postpone it to R17 in RAN2#109-e meeting.

=> Postpone the following issue to R17: Whether to have an indicator of controlling the beam level measurements in the logged MDT report. logged MDT in DC scenario

For EN-DC, according to the discussion in R16, some companies consider the NSA network and some companies consider the NR SA network in which the UE can be in EN-DC. Therefore, we think we need to analyse case by case.

For the NSA case, operators may want to know the coverage of NR. The reason is that the deployment of SN cell is related to capacity expansion and network planning. NR may be deployed in areas where high traffic occurs. The coverage where a NR cell is available may show whether the deployment of NR SN cell is according to the capacity increase needs. Also operators may want to know the coverage of NR SA before deploying the NR SA. Therefore, RAN2 may need to consider how to collect the coverage of NR in NSA. In NSA, both the MN and SN can configure the immediate MDT measurement. In general, the network will configure the UE to be in MR-DC when the quality of SN is above one threshold. Therefore, the NR SA coverage is different from the MR-DC coverage (as shown in Figure 1). Therefore the operators cannot get the coverage of NR SA before deploying the NR SA.

Observation 3: In NSA, the operators may want to know whether the coverage of NR SA before deploying the NR SA.

Figure 2: Different coverage of EN-DC and NR SA

Proposal 9: Support the logged MDT for NR cells in NSA where the UE cannot camp on NR cells.

In R16, the UE logs the measurement results of serving cell and the available measurement results of NR neighbouring cells when UE camps on LTE cells. UE cannot camp on the NR cell in NSA, therefore UE will not perform the logged MDT configuration from SN. In our understanding, the MN can configure the logged MDT measurement of SN and the UE can measure the NR cells like the early measurement, then the operator can get more data about the coverage of SN and get the coverage of NR SA. If the UE supports the NR SA, the UE can support the measurement of the NR cells in RRC_IDLE. Therefore, in NSA, the LTE can broadcast SIB24 then these UEs will measure the NR cells during cell re-selection and record the results. However, the NR cells will only broadcast the MIB and mini-SIB1, therefore the UE will not reselect to the NR cells. It will not increase complexity of UE and the network, only have a little impact on the UE power consumption. Also in R16, RAN2 has supported the early measurement. In the early measurement, the UE will measure the NR cells in the RRC_IDLE/RRC_INACTIVE even the UE cannot camp on NR cells. We think it will not increase the complexity of UE if the UE measures and records the NR cells. Therefore we think the MN can configure the UE to record the NR cells in EN-DC.

Proposal 10: In NSA, the MN can configure the UE to collect the coverage of NR.

Conclusion

Base on the analysis above, we propose the following observations and proposals:

Proposal 1:  User consent for the location information in RLF/CEF/SCG failure reporting should be also applied to LTE.

Proposal 2: The RLF/CEF/SCG failure reporting and MDT use the same user consent

Proposal 1: Add the IDC problems detection in the logged MDT. Take the principle of LTE as the baseline.

Observation 1: R16 supports the immediate MDT for EN-DC. The NR SN can configure and receive immediate measurement.

Proposal 2: Support the immediate MDT for NR-DC.

- For signalling based immediate MDT, AMF only provides only one MDT configuration to the MN. The MN forwards the configuration to the SN.

- For management based immediate MDT, OAM provides the MDT configuration to both MN and SN independently.

Proposal 3: Support the M1/M2/M4/M8/M9 in NR-DC

Proposal 4: Support the delay measurement for the MN terminated MCG bearers and SN terminated SCG bearers in NR-DC. The configuration and reporting principles in NR-DC are same as the principles in EN-DC.

Proposal 5: Support the delay measurement for the SN terminated MCG/split bearers and MN terminated SCG/split bearers in EN-DC and NR-DC.

Observation 2: The scheduling latency of one split bearer are different in the two nodes of MR-DC.

Proposal 6: For the split bearer (i.e. MN terminated split bearer and SN terminated split bearer), only the node hosting the PDCP entity configure the D1 measurement. UE reports two D1s to the node hosting the PDCP entity in one RRC message.

Proposal 7: For the MN terminated SCG bearer and SN terminated MCG bearer, the node hosting the PDCP entity configure the D1 measurement.

Proposal 8: For the split bearer, the node hosting the PDCP entity derives the delay of the split bearers based on the delay of two paths.

- For split bearer with PDCP duplication, the final delay is the min value of measured results of two paths

- For split bearer without PDCP duplication, the final delay is the average value of measured results of two paths

Observation 3: In NSA, the operators may want to know whether the coverage of NR SA before deploying the NR SA.

Proposal 9: Support the logged MDT for NR cells in NSA where the UE cannot camp on NR cells.

Proposal 10: In NSA, the MN can configure the UE to collect the coverage of NR.

Reference

[1]  RP-201281, New WID on enhancement of data collection for SON/MDT in NR