3GPP TSG-RAN WG2 Meeting #111e                  	R2-2007956

E-Meeting, 17th – 28th August, 2020

Source:	Xiaomi

Title:	Discussion of the coordinated leaving problem

Agenda Item:	8.3 Multi SIM

Document for:	Discussion and Decision

- Introduction

During the RAN#86 meeting, MUSIM SID was approved in RP-193263 [1] and further updated in RP-201309 [2] described with the following objectives:

- Specify, if necessary, enhancement(s) to address the collision due to reception of paging when the UE is in IDLE/INACTIVE mode in both the networks associated with respective SIMs [RAN2] - RAT Concurrency: Network A can be NR. Network B can either be LTE or NR. - Applicable UE architecture: Single-Rx/Single-Tx. - Specify mechanism for UE to notify Network A of its switch from Network A (for MUSIM purpose) [RAN2]: - RAT Concurrency: Network A is NR. Network B can either be LTE or NR. - Applicable UE architecture: Single-Rx/Single-Tx, Dual-Rx/Single-Tx - Unless SA2 find an alternative solution or decides otherwise, specify mechanism for an incoming page to indicate to the UE whether the service is voLTE/VoNR. [ RAN2] - RAT Concurrency: Network A is either LTE or NR. Network B is either LTE or NR. - Applicable UE architecture: Single-Rx/Dual-Rx/Single-Tx

In this paper, we discuss the problem indicated in the objective 2 and propose a solution for that.

- Discussion

According to SA2 study about MUSIM in SP-200091 [4], they identify several key issues and also solutions to each key issue in TR 23.761 [3]. We think Key issue 1 and 3 in [3] are related to objective 2 in [2] and 14 solutions are proposed to solve key issue 1 and 3.

Table 1: Mapping of Solutions to Key Issues from TR 23.761 [3]

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 |
| 1 | X |  |  |  |
| 2 | X |  |  |  |
| 3 | X |  |  |  |
| 4 |  |  | X |  |
| 5 |  |  | X |  |
| 6 |  |  | X |  |
| 7 | X | X |  |  |
| 8 | X |  |  |  |
| 9 | X |  |  |  |
| 10 | X |  |  |  |
| 11 | X |  |  |  |
| 12 | X | X |  |  |
| 13 | X |  |  |  |
| 14 |  | X |  |  |
| 15 |  | X |  |  |
| 16 |  | X |  |  |
| 17 |  | X |  |  |
| 18 |  | X |  |  |
| 19 |  | X |  |  |
| 20 |  | X |  |  |
| 21 |  | X |  |  |
| 22 |  |  | X |  |

As we proposed in our another contribution [5], we think it’s better to find a unified solution to solve MUSIM problems indicated in objective 1 and 2 when specifying solutions in RAN2 and reusing the mechanism we used to solve the overheating problem as much as we can to solve the MUSIM problem is one option.

While communicating with the system A with one USIM A, the MUSIM device finds that it needs to perform some kind of activity in the system B associated with USIM B (e.g. paging reception, measurements, respond to a page, mobility update, data communication, etc). Then USIM A needs to send RRC signaling with assistance information to the connected gNB in system A about the time/pattern USIM A needs to switch from system A to system B.

After receiving the assistance information, the connected gNB in system A can make a reconfiguration for USIM A to leave system A to do some activity in the system B so that system A will not interpret this as an error case and will not distort the statistics and misguide the algorithms in the current system.

Proposal 1: MUSIM device sends RRC signaling with assistance information to the connected gNB in system A about the time/pattern USIM A needs to switch from system A to system B.

- Conclusion

This paper discusses the problem indicated in the objective 2 and proposes:

Proposal 1: MUSIM device sends RRC signaling with assistance information to the connected gNB in system A about the time/pattern USIM A needs to switch from system A to system B.

- References

[1] PR-193263 Support for Multi-SIM devices in Rel-17.

[2] RP-201309 Revision on WID Support for Multi-SIM devices.

[3] TR 23.761 Study on system enablers for devices having multiple Universal Subscriber Identity Modules (USIM).

[4] SP-200091 Revised SID: Study on system enablers for multi-SIM devices.

[5] R2-2007952 General consideration for solving MUSIM problems.