3GPP TSG-RAN WG2 Meeting #111-e                             R2-2008020

Online, 17th - 28th August 2020

Agenda item:	8.3

Source:	Samsung

Title:	General considerations on potential RAN2 works for Multi-USIM devices

Document for:	Discussion & Decision

# Introduction

In Rel-17 WI on Multi SIM, the objectives in [1] are as follows:

- Specify, if necessary, enhancement(s) to address the collision due to reception of paging when the UE is in IDLE/INACTIVE mode in both the networks associated with respective SIMs [RAN2]

  - RAT Concurrency: Network A can be NR. Network B can either be LTE or NR.

  - Applicable UE architecture: Single-Rx/Single-Tx.

- Specify mechanism for UE to notify Network A of its switch from Network A (for MUSIM purpose) [RAN2]:

  - RAT Concurrency: Network A is NR. Network B can either be LTE or NR.

  - Applicable UE architecture: Single-Rx/Single-Tx, Dual-Rx/Single-Tx

- Unless SA2 find an alternative solution or decides otherwise, specify mechanism for an incoming page to indicate to the UE whether the service is voLTE/VoNR.[ RAN2]

  - RAT Concurrency: Network A is either LTE or NR. Network B is either LTE or NR.

  - Applicable UE architecture: Single-Rx/Dual-Rx/Single-Tx

This contribution provides our general views on potential RAN2 works for Multi-SIM devices.

# Discussion

## Paging collision avoidance

Single-Rx allows the Multi-USIM UE to monitor paging on one network at one time. If the paging occasions  on two networks overlap in time, it is unavoidable for the UE to make a choice of one out of two networks to monitor paging. It may lead to unsuccessful paging reception on the other network.

The Multi-USIM UE will calculate each paging occasion on two networks, respectively. It means that the UE is able to detect whether potential paging collision would happen. So, it would be more logical for the UE to trigger actions on potential paging collision avoidance.

For (potential) paging conflict avoidance, we think that in high level there needs some design criteria or requirements to be established on the basis of which potential solutions can be exploited i.e.

1.	Potential solutions should not lead to extra power consumption on the UE side

2.	Potential solutions should not lead to wastage of paging resources on the network side

The detection of the paging occasion collision is determined by the UE and the two networks involved do not have any coordination between them. This means UE assistance is needed to resolve the paging collision issue regardless of whatever potential solutions are explored.

Observation 1: UE assistance seems necessary to resolve paging collision since the involved networks are not coordinated.

Observation 2: Potential solutions should not lead to extra power consumption on the UE side.

Observation 3: Potential solutions should not lead to wastage of paging resources on the NW side.

Considering the above design principles or requirements the potential solutions could be designed as a NAS based solution or a AS based solution. Before deciding whether the solution is NAS based or AS based, it seems necessary the solutions are first evaluated against RAN2 agreed design criteria or requirements.

Proposal 1: RAN2 to agree the design principles as mentioned in the above observations and agree upon such principles to further explore the solution space.

## Switching notification mechanism

While actively communicating with one network associated with one USIM, the Multi-USIM UE may need to perform some activities (e.g. paging monitoring, SI acquisition/ PLMN search) on other network associated with other USIM. In the absence of any standard defined procedure of switching notification to one network, it may result in interruptions of the ongoing services/ waste of resources.

For switching notification mechanism, a generic solution should be considered to handle any kind of activity on the other network. The high level solution is to have switching time gaps configured to the UE. These time gaps can be short or long duration gap in one shot or with some periodicities. In any cases, we think that existing RRC procedures can be exploited. For example,

1)	For short duration gaps, UE and network can negotiate them through RRC request/response mechanism.

-	These negotiated gaps can be one shot or periodic

-	These negotiated gaps can be autonomously activated or activated by notifying the network

-	After the gap is expired, the UE can either come back to the network with which it was previously connected or request for long duration gaps through IDLE/ INACTIVE release request

2)	For long duration gaps, UE assistance mechanism can be exploited i.e. UE can request the network to transit itself from CONNECTED to IDLE/INACTIVE state for MUSIM purpose.

Proposal 2: RAN2 to agree that negotiated time gaps are needed to support switching. It is FFS how the time gaps are provisioned e.g., via existing toolkit in the standard.

## Handling of MT voLTE/VoNR service in paging

SA2 have been actively discussing how to handle the MT services (including voLTE/VoNR) for a Multi-USIM UE [2]. The basic principle is to tag the MT service with relevant paging cause. Depending on how many paging causes are introduced by SA1/SA2, potential impacts on RAN2 might be different i.e.

-	If a single paging cause is introduced, it can be indicated to the UE via Short message or paging message.

-	If more than one paging cause is introduced, it would be more efficient to support them in paging message, but it is required to discuss stage 3 details depending on how paging causes are categorized.

We think that it is premature for RAN2 to discuss how to handle the MT voLTE/VoNR service for paging before SA2 make sufficient progress on how many paging causes are introduced.

Proposal 3: RAN2 to start the discussion on handling of MT voLTE/Vo NR service for paging after SA2 make sufficient progress on how many paging causes are introduced.

# Conclusion

In section 2, we made the following observations:

Observation 1: UE assistance seems necessary to resolve paging collision since the involved networks are not coordinated.

Observation 2: Potential solutions should not lead to extra power consumption on the UE side.

Observation 3: Potential solutions should not lead to wastage of paging resources on the NW side.

Based on the above, RAN2 is requested to discuss and if possible agree on the following proposals:

Proposal 1: RAN2 to agree the design principles as mentioned in the above observations and agree upon such principles to further explore the solution space.

Proposal 2: RAN2 to agree that negotiated time gaps are needed to support switching. It is FFS how the time gaps are provisioned e.g., via existing toolkit in the standard.

Proposal 3: RAN2 to start the discussion on handling of MT voLTE/Vo NR service for paging after SA2 make sufficient progress on how many paging causes are introduced.

# References

[1] RP-201309, New WID: Support for Multi-SIM devices for LTE/NR

[2] TR 23.761, Study on system enablers for devices having multiple Universal Subscriber Identity Modules (USIM)