3GPP TSG-RAN2 #111e	R2-2008088

# Electronic meeting, August 17 – August 28, 2020

Agenda item:		6.2.4 (NR_IAB_ Core)

Source:	LG Electronics Inc.

Title: 	Clean-up of misaligned requirements between procedure and field description

Document for:	Discussion and Decision

1. Introduction

IAB MT ignores cellBarred and intraFreqReselection. This requirement relaxation is captured only in field description while in procedural text, UE is still mandated to apply the existing (non-relaxed requirements).

2. Discussion

Existing cell status and cell reservation IEs are not applied to IAB-MT. The IEs, cellBarred, intraFreqReselection, cellReservedForOtherUse, cellReservedForFutureUse are affected by this relaxed requirement.

Among those IE, the cellBarred and intraFreqReselection IE are specified in both procedural and field description, whereas cellReservedForOtherUse and cellReservedForFutureUse are only captured in the field description.

Then, a possible conflict of requirements could occur for the IEs specified in both procedural and field description, because currently the relaxation of requirement is only captured in the field description that “This field is ignored by IAB-MT”, while the procedural text still mandates non-relaxed requirements.

For example, 5.2.2.4.1 of 38.331 mandates the UE to apply the following for intraFreqReselection IE

5.2.2.4.1	Actions upon reception of the MIB

Upon receiving the MIB the UE shall:

1>	store the acquired MIB;

1>	if the UE is in RRC_IDLE or in RRC_INACTIVE, or if the UE is in RRC_CONNECTED while T311 is running:

2>	if the cellBarred in the acquired MIB is set to barred:

3>	consider the cell as barred in accordance with TS 38.304 [20];

3>	if intraFreqReselection is set to notAllowed; and

3>	if the cell operates in licensed spectrum or the cell belongs to a PLMN which is indicated as being equivalent to the registered PLMN or the cell belongs to the registered SNPN of the UE:

4>	consider cell re-selection to other cells on the same frequency as the barred cell as not allowed, as specified in TS 38.304 [20].

3>	else:

4>	consider cell re-selection to other cells on the same frequency as the barred cell as allowed, as specified in TS 38.304 [20].

2>	else:

3>	apply the received systemFrameNumber, pdcch-ConfigSIB1, subCarrierSpacingCommon, ssb-SubcarrierOffset and dmrs-TypeA-Position.

But the field description simply gives the following overriding requirements as highlighted yellow.

MIB field descriptions

cellBarred Value barred means that the cell is barred, as defined in TS 38.304 [20]. This field is ignored by IAB-MT.

…

intraFreqReselection Controls cell selection/reselection to intra-frequency cells when the highest ranked cell is barred, or treated as barred by the UE, as specified in TS 38.304 [20]. This field is ignored by IAB-MT.

Such conflict might be considered hypothetical since proper implementations should avoid this. But the question is whether we should try to clean up the misalignment between the procedural text and field description to maintain cleaner specifications, as we have done so far, rather than having such ‘locally-patched’ specifications.

So we think the following actions can be considered to resolve the issue of such locally-patched specifications/

- Option1: Reflect the relaxed requirements in the procedural text and possibly remove the relaxed requirements from the field descriptions.

- Option2: Do nothing

3. Conclusion

Proposal: To discuss if it is good to clean up the misalignment between the procedural text and field description on the cellBarred and intraFreqReselection.