3GPP TSG-RAN WG2 Meeting #111 Electronic	R2-2008105

Elbonia, 17 – 28 August 2020

Agenda item:	6.2.5

Source:	Nokia (Summary Rapporteur)

Title:	Summary of IAB UE capabilities under AI 6.2.5

WID/SID:	NR_IAB - Release 16

Document for:	Discussion and Decision

# 1	Introduction

This is to provide a summary of TDocs submitted for IAB UE capabilities under AI 6.2.1 and 6.2.5 which include [1-5].

# 2	Summary

The discussion paper in [1] proposes to introduce the following IAB specific capabilities:

Proposal 1: Add the following parameter for Feature 4-1 in 38.306: Definitions for parameters; Per; M; FDD-TDD DIFF; FR1-FR2 DIFF. intraAndInterF-MeasAndReport-IAB-r16 Indicates whether the IAB-MT supports NR intra-frequency and inter-frequency measurements and at least periodical reporting. Note: It is up to the IAB node to set the capability bit; UE; Yes; Yes; No. Proposal 2: Add the following parameter for Feature 7-1, component 2) in 38.306: Definitions for parameters; Per; M; FDD-TDD DIFF; FR1-FR2 DIFF. handoverInterF-IAB-r16 Indicates whether the IAB-MT supports inter-frequency HO. It indicates the support for inter-frequency HO from the corresponding duplex mode if this capability is included in fdd-Add-UE-NR-Capabilities or tdd-Add-UE-NR-Capabilities. It indicates the support for inter-frequency HO in the corresponding frequency range if this capability is included in fr1-Add-UE-NR-Capabilities or fr2-Add-UE-NR-Capabilities.; IAB-MT; No; Yes; Yes.

Companies are invited to provide their views whether they agree with the proposals.

Q1: Do you agree with the proposals in [1]? If not, please provide comments.

| Company | Preference (Y/N) | Detailed Comments |
| --- | --- | --- |

The CR in [2] proposes changes to TS 38.306 based on the conclusions agreed by RAN Plenary and captured in RP-201292. The changes intend to clarify accordingly eventA-MeasAndReport and intraAndInterF-MeasAndReport features support.

Companies are invited to provide their views whether they agree with the changes.

Q2: Do you agree with the changes proposed in [2]? If not, please provide comments.

| Company | Preference (Y/N) | Detailed Comments |
| --- | --- | --- |

The CR in [3] proposes to introduce a new section into TS 38.300 to implement the following agreement:

- R2 to specify that IAB-MTs can make use of the UE capability signaling framework (including specification of minimum set). Whether it is actually used for e.g. Wide Area IAB-MTs may be up to implementation.

#### 4.7.4.5 IAB-node Capability Signaling IAB-MTs can make use of the UE capability signaling framework (including specification of minimum set). Whether it is actually used for e.g. Wide Area IAB-MTs may be up to implementation.

The companies are invited to provide their views for the above proposal and, especially, whether such section is needed to be introduced into Stage-2.

Q3: Do you agree to introduce a new section into TS 38.300 to implement the above agreement? If not, please provide comments.

| Company | Preference (Y/N) | Detailed Comments |
| --- | --- | --- |

The CR in [4] proposes to add MAC capability lcid-ExtensionIAB of IAB-MT defined in TS 38.306 into TS 38.331. Rapporteur notes that this parameter seems to have been mistakenly left out from TS 38.331.

Companies are invited to provide their views whether they agree with the change.

Q4: Do you agree with the change proposed in [4]? If not, please provide comments.

| Company | Preference (Y/N) | Detailed Comments |
| --- | --- | --- |

The CR in [5] proposes changes to TS 38.306. The changes intend to clarify:

- Optional features for IAB-MT are: multipleTCI, pdsch-MappingTypeA, pucch-F2-WithFH, pucch-F3-WithFH;

- Mandatory features for IAB-MT with capability signalling are: eventA-MeasAndReport, intraAndInterF-MeasAndReport

- Support at least one of the two features for IAB-MT is: drb-IAB-r16 or non-DRB-IAB-r16.

Companies are invited to provide their views whether they agree with the changes.

Q5: Do you agree with the changes proposed in [5]? If not, please provide comments.

| Company | Preference (Y/N) | Detailed Comments |
| --- | --- | --- |

# 3	Conclusion

TBD

# References

[1] R2-2006959, Remaining details of UE capabilities for IAB, AT&T

[2] R2-2007508, Update to IAB-MT capabilities, Nokia, Nokia Shanghai Bell

[3] R2-2007539, Corrections to capability signaling for IAB-MT, Samsung Electronics Romania

[4] R2-2007980, Correction on IAB-MT capability for TS 38.331, Huawei, HiSilicon

[5] R2-2007981, Correction on IAB-MT capability for TS 38.306, Huawei, HiSilicon