3GPP TSG-RAN WG2 Meeting #111 electronic	R2-2008364

Online, August 17th– 28th, 2020

Source:	CATT

Title:	Summary of Rel-16 power saving RRM issues

Agenda Item:	6.9.3

Document for:	Discussion and Decision

# Introduction

This contribution provides a summary of the documents posted on A.I. 6.9.3 “Control plane Corrections” and related to RRM [1]-[5].

# Discussion

[1]: 6685: Discussion on how to restructure the RRM relaxation

vivo and al propose to re-structure the text of the RRM relaxation procedure (with no functional change) so that there is no overlap between various cases and the legacy behaviors are removed since already addressed elsewhere:

“We should also follow the following principle to re-structure the RRM relaxation procedure part:

- Current procedure should correctly capture the RAN2/RAN4 conclusion, and nothing should be missing.

- For not relaxed measurement case (i.e. legacy behaviour cases), it should not be captured in this section. The UE behaviour should follow the legacy procedure in TS 38.304 and TS 38.133.

- For relaxed measurement cases, it should be captured only once (i.e.in one branch) in the specification, i.e. in order to avoid double captured problem.

- The principle for the UE behaviour in idle/inactive mode should be clear and unique based on TS 38.304 and TS 38.133.

Proposal 1: RAN2 to discuss the re-structure of RRM relaxation procedure based on the above principle”

Rapporteur’s view: it should be discussed on-line first if such restructuring is needed. If yes, since there is no functional change or correction, this is typically a CR finalization email discussion.

Proposal 1: It should be discussed on-line if a text re-structuring for 38.304 Section 5.2.4.9.0, as proposed in [1], is needed.

Proposal 2: If above text re-structuring is seen needed, discuss the new text in the 38.304 CR email discussion, based on [1].

[2][3][5]: 6686-7/7367: RRM relaxation for high priority frequency

[2]:

Observation 1: There is no decision and motivation in RAN2 to revert any previous agreements in RAN2#108, #109-e, and #109bis-e on highPriorityMeasRelax.

Observation 2: There is no corresponding discussion and conclusion in RAN4 on highPriorityMeasRelax for use cases (i.e. use cases C, G, E, F) other than the one in reply LS.

Proposal 1: The highPriorityMeasRelax should be appliable for all use cases for measurement relaxation of high priority frequency, i.e. rel-16 relaxation is not allowed for high priority frequencies when highPriorityMeasRelax is not configured.

Proposal 2: RAN2 to discuss whether an LS to RAN4 is needed based on the conclusion of the above issue.

Proposal 3: RAN2 to consider the following TP in Annex about the measurement relaxation for high priority frequencies.

[5]:

Based on a similar observation as above observation #2, it is proposed to limit the applicability of the parameter highPriorityMeasRelax to the case when Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, and the proposed CR is:

highPriorityMeasRelax

This indicates whether measurement on higher priority frequency is allowed to be relaxed beyond Thigher_priority_search (see clause 4.2.2.7 in TS 38.133 [8]) or not (in case Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ and the relaxed measurement criteria is fulfilled).

Vivo also provides a draft LS to RAN4 [3] to inform them about above change of UE behavior for high priority frequency cells.

Rapporteur’s view: it was captured in Chairlady’s report: “Update if RAN4 has concerns”. So in principle, if any issue with the last meeting agreement, it should come from RAN4. So it should be discussed on-line first if the agreement from last meeting should be re-opened independently of any RAN4 feedback.

Proposal 3: Discuss online if the last meeting agreement on RRM relaxation for high priority frequency should be re-opened independently of any RAN4 feedback.

[4]: 7063: Correction to UE behavior for RRM measurement relaxation

According to TS 38.304, if some conditions are fulfilled, UE may choose to relax the RRM measurement. One of the conditions described in TS 38.304 is “the UE has performed normal intra-frequency or inter-frequency measurements for at least TSearchDeltaP after (re-)selecting a new cell”. This condition was brought from LTE (i.e., TS 36.304), where RRM measurement relaxation can be applied to intra-frequency or inter-frequency measurement. However, in NR, UE can relax inter-RAT frequency measurement as well. Therefore, rather than using the exactly same condition from LTE, RAN2 needs to update this condition for NR to include inter-RAT frequency measurements as well.

As a result, the CR proposes to add the inter-RAT frequency cases to the conditions “if the UE has performed normal intra-frequency, NR inter-frequency, or inter-RAT frequency measurements for at least TSearchDeltaP after (re-)selecting a new cell; and”.

Rapporteur’s view: discuss this change in an email discussion.

Proposal 4: Discuss offline the proposed change in [4] about updating the condition “the UE has performed normal intra-frequency or inter-frequency measurements for at least TSearchDeltaP after (re-)selecting a new cell” to include inter-RAT frequency measurements as well.

# Conclusions

# Rapporteur’s suggestions from initial summary

Proposal 1: It should be discussed on-line if a text re-structuring for 38.304 Section 5.2.4.9.0, as proposed in [1], is needed.

Proposal 2: If above text re-structuring is seen needed, discuss the new text in the 38.304 CR email discussion, based on [1].

Proposal 3: Discuss online if the last meeting agreement on RRM relaxation for high priority frequency should be re-opened independently of any RAN4 feedback.

Proposal 4: Discuss offline the proposed change in [4] about updating the condition “the UE has performed normal intra-frequency or inter-frequency measurements for at least TSearchDeltaP after (re-)selecting a new cell” to include inter-RAT frequency measurements as well.

# Reference

- R2-2006685 Discussion on how to restructure the RRM relaxation; vivo, Samsung, LG Electronics Inc., MediaTek Inc., Panasonic.

- R2-2006686 RRM relaxation for high priority frequency; vivo, Samsung, ZTE, Intel, Panasonic.

- R2-2006687 [Draft] LS to RAN4 on RRM measurement relaxation in power saving; vivo.

- R2-2007063 Correction to UE behavior for RRM measurement relaxation; Samsung Electronics.

- R2-2007367 RRM relaxation for higher priority frequency; OPPO.