3GPP TSG-RAN WG2 Meeting #111-e	R2-2008399

# Electronic Meeting, 17th – 28th Aug 2020                   Revision of R2-2007708

| CR--Form--v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 1887 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Compliance check failure for CPC configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh-Core |  |  |  |  |  | Date: |  |  | 2020-08-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - According to the current RRC spec, all stored CPC configuration will be released upon PCell change. For the PCell change without SN involved, since the SN is not aware of the PCell change, the SN has no idea whether the stored CPC configuration has been released or not on the UE side, which may cause the configuration mismatching between the SN side and the UE side. - After PCell change without SN involved, if the SN wants to modify the previous CPC configuration, the SN may send the RRCReconfiguration message including an updated condExecutionCond but without condRRCReconfig or an updated condRRCReconfig without condExecutionCond to the UE. In such case, a reconfiguration failure may happen since the UE may consider the received CPC configuration as a new one but the condExecutionCond and the condRRCReconfig may not be carried simultaneously, which shall cause the undesired recovery procedure (e.g. SCG failure information procedure or the RRC re-establishment procedure). |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add a note in 5.3.5.13.3 to specify that the UE does not consider the message as erroneous if condRRCReconfig and condExecutionCond are not simultaneously included in an entry without the matching condReconfigId exists in the condReconfigToAddModList within the VarConditionalReconfig for CPC. And the UE ignores the entry. Impact analysis Impacted architecture options: NR, NR-DC, NE-DC Impacted functionality: CHO Inter-operability: - If the network is implemented according to the CR and the UE is not, in case the SN sends a RRCReconfiguration message including an updated condExecutionCond but without condRRCReconfig or an updated condRRCReconfig without condExecutionCond to the UE after PCell change, the UE might think the message as erroneous and then initiate the unnecessary SCG failure information procedure or RRC re-establishment procedure. - If the UE is implemented according to the CR and the network is not, there is no inter-operability problem. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The UE may think the received RRCReconfiguration message including an updated condExecutionCond but without condRRCReconfig or an updated condRRCReconfig without condExecutionCond as erroneous and then initiate the unnecessary SCG failure information procedure or RRC re-establishment procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.5.13.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

CHANGE START

5.3.5.13.3	Conditional reconfiguration addition/modification

For each condReconfigId received in the condReconfigToAddModList IE the UE shall:

1>	if an entry with the matching condReconfigId exists in the condReconfigToAddModList within the VarConditionalReconfig:

2>	if the entry in condReconfigToAddModList includes an condExecutionCond;

3>	replace the entry with the value received for this condReconfigId;

2>	else:

3>	keep the stored condExecutionCond as the target candidate configuration for this condReconfigId;

2>	if the entry in condReconfigToAddModList includes an condRRCReconfig;

2>	replace the entry with the value received for this condReconfigId;

2>	if the entry in condReconfigToAddModList does not include an condRRCReconfig;

3>	keep the stored condRRCReconfig as the target candidate configuration for this condReconfigId;

1>	else:

2>	add a new entry for this condReconfigId within the VarConditionalReconfig;

1>	perform conditional reconfiguration evaluation as specified in 5.3.5.13.4;

NOTE:  The UE does not consider the message as erroneous if condRRCReconfig and condExecutionCond are not simultaneously included in an entry without the matching condReconfigId exists in the condReconfigToAddModList within the VarConditionalReconfig for CPC. And the UE ignores the entry.

CHANGE END