3GPP TSG-RAN WG2 Meeting #111-e	R2-2008679

E-meeting, Aug 18 – Aug 29, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 4353 | rev | 2 | Current version: | 16.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corretion on the RLF for LTE DAPS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_feMob-Core |  |  |  |  |  | Date: |  |  | 2020-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to the following agreement made in the RAN2#107bis, the UE need to trigger RRC reestablishment at the target link failure and does not keep the source RLF detection after the successul RACH of the DAPS handover. - After the successful completion of the RACH to the target cell and before the release of the source link, the UE does not keep the source link failure detection of the source link. - After the target cell RACH completion and before the release of the source cell, when the target link fails, the UE triggers RRC connection re-establishment. According to the following agreement made in the RAN2#110e meeting, the DAPS bearer is still considered as configured before the release of the source link. - RAN2 understanding is that DAPS HO is considered successful after RA completion. DAPS HO is considered complete after source release. Update conclusion of H460 accordingly. However according to the Rel-16 RRC specification, when the DAPS bearer is configured and T304 is not running (i.e. before releasing the source connnection), the UE could receive the failure indication from PHY/MAC/RLC from either the source link or the target link. However the Rel-16 specification requires the UE to trigger the re-establishment even for the source link failure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1 In section 5.3.11.3, some editorial error is corrected by re-constructing the format of the last paragraph. 2 In section 5.3.11.3, it is clarifed that for all other cases (including the case “when the DAPS bearer is configured and T304 is running”) except for the case that the DAPS DRB is configured and T304 is running, the UE would follow the legacy procedures to only handle the target link failure and the source link failure is not handled. Impact Analysis Impacted 5G architecture options: No Impacted functionality: DAPS Inter-operability: 1. If the network is implemented according to the CR and the UE is not, the UE will trigger RRC re-establishment for the source link when the RACH to the target PCell is successful. 2. If the UE is implemented according to the CR and the network is not, no inter-operability issue is foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the CR is not approved, the source link failure after the successful RACH to the target link would strill trigger RRC re-establishment. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.11.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | [MCC]: Wrong rev number in the coversheet. Agenda item in the header |  |  |  |  |  |  |  |  |

Start of Change

#### 5.3.11.3	Detection of radio link failure

The UE shall:

1> in case any DAPS bearer is configured, only the target PCell is considered in the following;

1>	upon T310 expiry; or

1>	upon T312 expiry; or

1>	upon random access problem indication from MCG MAC while neither T300, T301, T304 nor T311 is running; or

1>	upon indication from MCG RLC, which is allowed to be send on PCell, that the maximum number of retransmissions has been reached for an SRB or DRB:

2>	consider radio link failure to be detected for the MCG i.e. RLF;

2>	discard any segments of segmented RRC messages received;

2>	if the UE is configured with (NG)EN-DC; and

2>	if T316 is configured; and

2>	if SCG transmission is not suspended; and

2>	if NR PSCell change is not ongoing (i.e. T304 for the NR PSCell is not running as specified in TS 38.331 [82], clause 5.3.5.5.2, in (NG)EN-DC):

3>	initiate the MCG failure information procedure as specified in 5.6.26 to report MCG radio link failure;

2>	else:

3>	store the following radio link failure information in the VarRLF-Report (VarRLF-Report-NB in NB-IoT) by setting its fields as follows:

4>	clear the information included in VarRLF-Report (VarRLF-Report-NB in NB-IoT), if any;

4>	set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. includes the RPLMN);

4>	set the measResultLastServCell to include the RSRP and RSRQ, if available, of the PCell based on measurements collected up to the moment the UE detected radio link failure;

4>	except for NB-IoT, set the measResultNeighCells to include the best measured cells, other than the PCell, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected radio link failure, and set its fields as follows;

5>	if the UE was configured to perform measurements for one or more EUTRA frequencies, include the measResultListEUTRA;

5>	if the UE was configured to perform measurement reporting for one or more neighbouring UTRA frequencies, include the measResultListUTRA;

5>	if the UE was configured to perform measurement reporting for one or more neighbouring GERAN frequencies, include the measResultListGERAN;

5>	if the UE was configured to perform measurement reporting for one or more neighbouring CDMA2000 frequencies, include the measResultsCDMA2000;

5>	if the UE was configured to perform measurement reporting for one or more neighbouring NR frequencies, include the measResultListNR;

5>	for each neighbour cell included, include the optional fields that are available;

NOTE 1:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Blacklisted cells are not required to be reported.

4>	except for NB-IoT, if available, set the logMeasResultListWLAN to include the WLAN measurement results, in order of decreasing RSSI for WLAN APs;

4>	except for NB-IoT, if available, set the logMeasResultListBT to include the Bluetooth measurement results, in order of decreasing RSSI for Bluetooth beacons;

4>	if detailed location information is available, set the content of the locationInfo as follows:

5>	include the locationCoordinates;

5>	include the horizontalVelocity, if available;

4>	set the failedPCellId to the global cell identity, if available, and otherwise , except for NB-IoT, to the physical cell identity and carrier frequency of the PCell where radio link failure is detected;

4>	except for NB-IoT, set the tac-FailedPCell to the tracking area code, if available, of the PCell where radio link failure is detected;

4>	except for NB-IoT, if an RRCConnectionReconfiguration message including the mobilityControlInfo was received before the connection failure:

5>	if the last RRCConnectionReconfiguration message including the mobilityControlInfo concerned an intra E-UTRA handover:

6>	include the previousPCellId and set it to the global cell identity of the PCell where the last RRCConnectionReconfiguration message including mobilityControlInfo was received;

6>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo;

5>	if the last RRCConnectionReconfiguration message including the mobilityControlInfo concerned a handover to E-UTRA from UTRA and if the UE supports Radio Link Failure Report for Inter-RAT MRO:

6>	include the previousUTRA-CellId and set it to the physical cell identity, the carrier frequency and the global cell identity, if available, of the UTRA Cell in which the last RRCConnectionReconfiguration message including mobilityControlInfo was received;

6>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo;

5>	if the last RRCConnectionReconfiguration message including the mobilityControlInfo concerned a handover to E-UTRA from NR and if the UE supports Radio Link Failure Report for Inter-RAT MRO:

6>	include the previousNR-PCellId and set it to the global cell identity of the PCell where the last RRCConnectionReconfiguration message including mobilityControlInfo was received embedded in NR RRC message MobilityFromNRCommand message as specified in TS 38.331 [82] clause 5.4.3.3;

6>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo embedded in NR RRC message MobilityFromNRCommand message as specified in TS 38.331 [82] clause 5.4.3.3.

4>	except for NB-IoT, if the UE supports QCI1 indication in Radio Link Failure Report and has a DRB for which QCI is 1:

5>	include the drb-EstablishedWithQCI-1;

4>	except for NB-IoT, set the connectionFailureType to rlf;

4>	except for NB-IoT, set the c-RNTI to the C-RNTI used in the PCell;

4>	except for NB-IoT, set the rlf-Cause to the trigger for detecting radio link failure;

3>	if AS security has not been activated:

4>	if the UE is a NB-IoT UE:

5>	if the UE is connected to EPC and the UE supports RRC connection re-establishment for the Control Plane CIoT EPS optimisation; or

5>	if the UE is connected to 5GC and the UE supports RRC connection re-establishment for the Control Plane CIoT 5GS optimisation:

6>	initiate the RRC connection re-establishment procedure as specified in 5.3.7;

5>	else:

6>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'RRC connection failure';

4>	else:

5>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other';

3>	else:

4>	initiate the connection re-establishment procedure as specified in 5.3.7;

In case of DC or NE-DC, the UE shall:

1>	upon T313 expiry; or

1>	upon random access problem indication from SCG MAC; or

1>	upon indication from SCG RLC, which is allowed to be sent on PSCell, that the maximum number of retransmissions has been reached for an SCG, for a split DRB or for a split SRB:

2>	consider radio link failure to be detected for the SCG i.e. SCG-RLF;

2>	if the UE is configured with DC; or

2>	if the UE is configured with NE-DC and MCG transmission is not suspended:

3>	initiate the SCG failure information procedure as specified in 5.6.13 to report SCG radio link failure;

2>	else:

3>	initiate the connection re-establishment procedure as specified in TS 38.331 [82], clause 5.3.7.

In case of CA PDCP duplication, the UE shall:

1>	upon indication from an RLC entity, which is restricted to be sent on SCell only, that the maximum number of retransmissions has been reached:

2>	initiate the failure information procedure as specified in 5.6.21 to report RLC failure of type duplication;

If any DAPS bearer is configured and T304 is running, the UE shall:

1>	upon T310 expiry; or

1>	upon T312 expiry; or

1>	upon random access problem indication from source MCG MAC; or

1>	upon indication from source MCG RLC, which is allowed to be sent on source PCell, that the maximum number of retransmissions has been reached for an DRB:

2>	consider radio link failure to be detected for the source MCG;

2>	suspend the transmission of all DRBs in the source MCG;

2>	reset MAC for the source MCG;

2>	release the source connection;

The UE may discard the radio link failure information, i.e. release the UE variable VarRLF-Report (VarRLF-Report-NB in NB-IoT), 48 hours after the radio link failure is detected, upon power off or upon detach, and for NB-IoT, upon entering another RAT.

End of Change