3GPP TSG RAN WG2 Meeting #112-e							R2-2008812

Electronic meeting, November 2nd – 13th, 2020

Source:	CATT

Title:	Discussion on error sources, threat models, occurrence rates and failure modes

Agenda item:	8.11.3.2

Document for:	Discussion and Decision

Introduction

At the RAN2#111-e meeting, the agreement on possible sources of feared events are applicable for RAT-independent positioning in 3GPP system as below [1].

Agreements:

Proposal 5: RAN2 confirms that 4 possible sources of feared events are applicable for RAT-independent positioning in 3GPP system.

1.	Faults in the correction data e.g.

a.	Incorrect computation by the provider

b.	External feared event impacting the provider

2.	Faults in transmitting the data to the UE, e.g.

a.	Data integrity faults

3.	External feared events, e.g.

a.	Satellite feared events

b.	Atmospheric feared events

c.	Multipath

4.	UE faults

In RAN #89-e meeting, the SID of Study on NR Positioning Enhancements was updated as below [2]:

- Study solutions necessary to support integrity and reliability of assistance data and position information: [RAN2]

  - Identify positioning integrity KPIs and relevant use cases.

  - Identify the error sources, threat models, occurrence rates and failure modes requiring positioning integrity validation and reporting.

  - Study methodologies for network-assisted and UE-assisted integrity.

NOTE 4:	Objective 2 is applicable to GNSS positioning methods.

In this contribution, we will focus on the error sources, threat models, occurrence rates and failure modes in GNSS positioning methods.

Error source in GNSS

In order to quantify the integrity risk of the position solution and its error bound, we need to have a trusted model of the nominal errors, and a model of the potential threats. Also we need to figure out the effect of these errors persistence.

RTCM (Radio Technical Commission for Maritime Services) SC-134 is working on the integrity message definition now.

Compared with agreement on high level events, the classification as below table specified the possible errors.

Table 2-1 Integrity Errors

| Feared events | Group Name | Errors sources |
| --- | --- | --- |
| Faults in transmitting the data | Signal In Space Integrity | Constellations and Satellites Integrity data |
|  |  | Fast Constellation and Satellites Health Status |
| External feared events | Global Integrity | Precise Orbit and Clock Integrity Parameters |
|  |  | Displacements error Integrity Parameters |
|  |  | Satellite bias Integrity Parameters |
| Faults in the correction data | Network Integrity | Reference Station Specific Integrity Monitoring parameters data and measurements variances |
| UE faults | UE Integrity | Pseudorange corrections Integrity Parameters |
|  |  | Carrier Phase Integrity Parameters Corrections |

Observation1: There are Integrity message groups defined by RTCM. The detail message definition will be finalized at the end of 2020 by RTCM SC-134.

Interfaces between 3GPP and integrity monitor systems

There is satellite segment system (SSS), Ground Based Augmentation Systems (GBAS), Satellite Based Augmentation System (SBAS), terrestrial augmentation system (TAS), and Airborne Based Augmentation System (ABAS) as integrity monitoring systems and FDE for RTK, NRTK and PPP. Some of message as above table come from SGS/SSS/SBAS/TAS systems, or UE.

3GPP network may help transfer the integrity message/data.

Here is the relationship between 3GPP and other integrity monitor systems.

Integrity assistant data

Non-3GPP

Figure 2-4 Interfaces between 3GPP and other integrity monitor systems

Observation2: Integrity data on GNSS are offered by the 3rd network, instead of 3GPP.

Proposal1: RAN2 to discuss the integrity assistant data in SI stage. Capture the TP as below in TR.

-------------------------------------------Start of Text Proposal1---------------------------------------------------------------------------

Table x-y GNSS integrity Assistant data error sources

| Feared events | Group Name | Assistant data on error sources |
| --- | --- | --- |
| Faults in transmitting the data | Signal In Space Integrity | Constellations and Satellites Integrity data |
|  |  | Fast Constellation and Satellites Health Status |
| External feared events | Global Integrity | Precise Orbit and Clock Integrity Parameters |
|  |  | Displacements error Integrity Parameters |
|  |  | Satellite bias Integrity Parameters |
| Faults in the correction data | Network Integrity | Reference Station Specific Integrity Monitoring parameters data and measurements variances |
| UE faults | UE Integrity | Pseudorange corrections Integrity Parameters |
|  |  | Carrier Phase Integrity Parameters Corrections |

-------------------------------------------End of Text Proposal1---------------------------------------------------------------------------

Conclusions

In this contribution, we focused on integrity monitor and the error sources analysis in GNSS.

Observation1: There are Integrity message groups defined by RTCM. The detail message definition will be finalized at the end of 2020 by RTCM SC-134.

Observation2: Integrity data on GNSS are offered by the 3rd network, instead of 3GPP.

Proposal1: RAN2 to discuss the integrity assistant data in SI stage. Capture the TP as below in TR.

References

- R2-20xxxxx, “Report of session on positioning and sidelink relay”. Session Chair (MediaTek)

- R1-2003638, “Revised SID: Study on NR Positioning Enhancements”. CATT, Intel Corporation

Text Proposals

-------------------------------------------Start of Text Proposal1---------------------------------------------------------------------------

Table x-y GNSS integrity error sources

| Feared events | Group Name | Errors sources |
| --- | --- | --- |
| Faults in transmitting the data | Signal In Space Integrity | Constellations and Satellites Integrity data |
|  |  | Fast Constellation and Satellites Health Status |
| External feared events | Global Integrity | Precise Orbit and Clock Integrity Parameters |
|  |  | Displacements error Integrity Parameters |
|  |  | Satellite bias Integrity Parameters |
| Faults in the correction data | Network Integrity | Reference Station Specific Integrity Monitoring parameters data and measurements variances |
| UE faults | UE Integrity | Pseudorange corrections Integrity Parameters |
|  |  | Carrier Phase Integrity Parameters Corrections |

-------------------------------------------End of Text Proposal1---------------------------------------------------------------------------