3GPP TSG-RAN WG2 Meeting #112 electronic                                                            	R2-2008844

Online, November 2nd - 13th, 2020

Source:	CATT

Title:	Discussion on CHO and DAPS Mobility Enhancement

Agenda Item:	8.13.2.1

Document for:	Discussion and Decision

- Introduction

In RAN2#111-e meeting, it was agreed to consider the SON aspects of CHO and DAPS HO as part of the RAN2 SON/MDT WI. Agreement about CHO and DAPS mobility enhancement has been made in last meeting as following and some FFS needs further discuss [1]:

Agreements: => RAN2 to consider the SON aspects of CHO and SON aspects of 2-step RA as part of the WI. => RAN2 to consider the SON aspects of DAPS HO as part of the WI. => The following scenarios are considered: - 1) Successful CHO and HO (i.e. no failure happens). FFS consideration in RAN2/3 - 2) Unsuccessful CHO due to late CHO execution. - 3) Unsuccessful CHO after CHO execution. - 4) Successful or Unsuccessful CHO after unsuccessful CHO or handover failure. - Note: other scenarios are not ruled out… => RAN2 should study what CHO failure information can be stored in RLF report. => RAN 2 to discuss the method for distinguishing between different handover types in RLF report. FFS the details, e.g., explicitly way or not. => RAN2 to agree studying the RLF report and/or FailureInformation message contents in the DAPS failure scenarios.

In this contribution, we will continue to discuss the CHO and DAPS mobility enhancement issue.

- Discussion

For SON feature to introduce the mobility enhancement optimization, the most important thing is to distinguish the different HO scenarios in our opinion. In R16 the DAPS and the CHO/CPC cannot be configured simultaneously, so for network side how to make the improvement for mobility enhancement could be separately discussed for DAPS and for CHO/CPC.

2.1 CHO mobility enhancement

CHO/CPC is used to pre-configure a set of HO configurations to the UE side, each one is linked to a specific target cell. If the CHO triggered condition is fulfilled, the UE will directly perform random access to the corresponding target cell. CHO mechanism is beneficial for HO robustness, but the signaling overhead in Uu-interface, S1/NG, X2/Xn and the occupied resource may be increased by preparing more pre-configured target cells.

CHO scenario

In last meeting, some scenario was considered for CHO, it includes successful CHO and HO, unsuccessful CHO due to late CHO execution, unsuccessful CHO after CHO execution, successful or unsuccessful CHO after unsuccessful CHO or handover failure. There are some scenarios that are not included in the agreement and all of these do not consider the DC situation. As the conditional PSCell change is supported in release 16, we think it is essential to record the CPC related failure information for mobility enhancement in SN. We would like to add some supplementary CHO scenarios as follows:

  - Unsuccessful CHO/HO due to late CHO/HO execution

  - Unsuccessful CHO/HO after CHO/HO execution

  - Successful CPC and PSCell change (i.e. no failure happens)

  - Unsuccessful CPC/PSCell change due to late CPC/PSCell change execution

  - Unsuccessful CPC/PSCell change after CPC/PSCell change execution

  - Successful or Unsuccessful CPC after unsuccessful CPC or PSCell change failure

Proposal 1:  We propose to add some supplementary CHO scenarios:

  - Unsuccessful CHO/HO due to late CHO/HO execution

  - Unsuccessful CHO/HO after CHO/HO execution

  - Successful CPC and PSCell change (i.e. no failure happens)

  - Unsuccessful CPC/PSCell change due to late CPC/PSCell change execution

  - Unsuccessful CPC/PSCell change after CPC/PSCell change execution

  - Successful or Unsuccessful CPC after unsuccessful CPC or PSCell change failure

Since CPC is the conditional change for PSCell, before we discuss the optimization, whether the SN related information reporting could be introduced for MRO should be discussed first. And then based on the SN information report, the optimization for CPC could be considered.

Proposal 2: The SN related information reporting (e.g. RLF or PSCell change failure) should be discussed first, then optimization for CPC could be considered.

Proposal 3: For Mobility enhancement optimization, the conditional PSCell change relevant records can be considered in RLF report.

In CHO scenario, the first access the target failed and the UE cell selection to CHO candidate cell, UE may continue CHO and access successful. How the first CHO failure information and the second successful CHO information are recorded need to be discussed. In our opinion, we prefer to record and report the failure and successful information to network together for network optimization.

In CHO scenario, the first access the target failed and the UE cell selection to CHO candidate cell, UE may continue CHO procedures and may lead to second access failure, how to record the two CHO failure information need to be discussed. In our opinion, we prefer to record and report the two failure information to network together for network optimization.

Proposal 4: In Successful or Unsuccessful  CHO after unsuccessful CHO or handover failure scenario, we prefer to record and report the two failure information or failure and successful information to network together for network optimization.

CHO failure parameter in RLF

In RAN2#111-e meeting, many companies proposed to study what CHO failure information can be stored in RLF report. Considering that there are some differences between traditional HO and CHO in handover execution and handover failure handle, traditional HO failure parameter and their definition in RLF report may not be used directly for CHO failure scenario, we need to revise the definition of some traditional HO failure parameter and based on the existing parameter, we may introduce the new parameter for CHO scenario if necessary.

- Reuse traditional parameters

Review the traditional parameters in RLF report, there are two parameters can be reuse for CHO scenario with some modifications in field description, other parameters can be reused directly. The two parameters are as follows:

timeConnFailure This field is used to indicate the time elapsed since the last HO initialization until connection failure. Actual value = field value * 100ms. The maximum value 1023 means 102.3s or longer. failedPCellId This field is used to indicate the PCell in which RLF is detected or the target PCell of the failed handover. For intra-NR handover nrFailedPCellId is included and for the handover from NR to EUTRA eutraFailedPCellId is included. The UE sets the ARFCN according to the frequency band used for transmission/ reception when the failure occurred.

The field timeConnFailure indicate the time elapsed since the last HO initialization until connection failure, it means the start point is the UE received the handover command from network and execute handover. But for CHO scenario, when the UE receive the conditional handover command from network with some conditional configurations, the UE will not start the handover execution procedure immediately, it will only perform the execution procedure when the condition of CHO are met. So for CHO scenario, it may take relatively long time between HO initialization and HO execution, the timeConnFailure for CHO should indicate the time elapsed since the last HO execution until connection failure.

Proposal 5: It is proposed to use the HO execution instead of HO initialization in timeConnFailure field description.

For field failedPCellId, it is used to indicate the PCell in which RLF is detected or the target PCell of the failed handover. In traditional handover, when the UE fails to access the target cell for the first time, the UE will perform the re-connection procedure, the failedPCellId field records the target PCell of the failed handover. But for CHO scenario, when the first access the target PCell failed and cell selection to CHO candidate cell, UE may continue CHO procedures and may lead to second access failure, and there will be two target PCells of the failed handover. To be aligned with the traditional HO, it is propose to define the target PCell of the failed handover only for the first access.

Proposal 6: It is proposed to define the target PCell of the failed handover only for the first access.

- Introduce new parameters

CHO access success or failure indicator: For CHO scenario, the first access the target failed and the UE cell selection to CHO candidate cell, UE may continue CHO and access successful. Although, the procedure may be regarded as successful CHO, but further optimization is necessary for the type of CHO. A CHO access success or failure indicator is introduced to remind the NW to continue CHO optimization for the type of successful CHO.

CHO handover type indicator: The indicator is used to indicate the last handover failure type is conditional handover failure, it is benefit for network to optimize the CHO configuration.

CHO candidate cell list: If the CHO candidate cell list or the execute condition configuration is not reasonable, it will lead to handover to wrong cell, too early handover or too late handover. It is necessary to optimize the CHO candidate cell list and the execute condition configured to UE. CHO candidate cell list can be included in RLF report to assist the network optimization.

Multiple RLF Report:  According to current implementation in TS38.331, if the UE failed to perform the CHO for the first access, the UE will record the failure information in RLF report, when the UE cell selection to CHO candidate cell, UE may continue CHO procedures and may lead to second access failure, the second access failure information will also be recorded in RLF report which will override the first failure information. For complex scenario such as CHO/CPC/DAPS, the relationship between each failure may be useful for network to judge the node and configured parameters needs to be improved. For mobility enhancement optimization, more than one RLF report entries could be considered to reflect the relationship of different scenarios.

To improve network optimization performance for CHO scenario, new parameters (CHO access success or failure indicator, CHO handover type indicator, CHO candidate cell list, multiple RLF Report) are suggested to be introduced in RLF report.

Proposal 7: New parameters (CHO access success or failure indicator, CHO handover type indicator, CHO candidate cell list, multiple RLF Report) are suggested to be introduced in RLF report for network optimization.

2.2 DAPS mobility enhancement

The UE will connect to both the target cell and the source cell. Only after the UE successfully access the target cell, the connection with the source cell could be released. If RLF occurs immediately in the target cell after UE HO or UE HO procedure is failed, and the RRC connection with the source cell has not been released yet, a fallback mechanism could be used. The UE will send the FailureInformation message to the source cell with a failure type of “daps-failure”, and then the RRC connection will be maintained for a while. Therefore DAPS is beneficial to reduce the delay of the handover procedure, and to give a chance for the UE to fallback to the source cell to maintain the RRC connection.

DAPS scenario

For DAPS HO, there is no related discussion about the DAPS scenario in RAN2#111-e meeting, in our opinion, the following scenarios may happen:

  - DAPS HO failure with/without fallback to source

  - DAPS HO successful with target RLF, and then fallback to source

    - Fallback successful

    - Fallback failure

Proposal 8:  We propose the following DAPS scenario:

  - DAPS HO failure with/without fallback to source

  - DAPS HO successful with target RLF, and then fallback to source

    - Fallback successful

    - Fallback failure

In DAPS scenario, the UE can keep the connection with source cell and target cell simultaneously during DAPS handover execution, UE may occur RLF in source cell and also failed to access the target cell, how to record the RLF information in source cell and the handover failure information need to be discussed. In our opinion, we prefer to record and report the two failure information to network together for network optimization.

Proposal 9: We prefer to record and report the two failure information to network together for network optimization when the RLF occurred in source cell and also failed to access the target cell.

DAPS failure parameter in RLF

- Reuse traditional parameters

Similar to CHO, we need to consider if the traditional HO failure parameter in RLF can be reused for DAPS scenario as some differences between traditional HO and DAPS in handover execution and handover failure handle. We need to revise the definition of some traditional HO failure parameter and based on the existing parameter, we may introduce the new parameter for DAPS scenario if necessary.

Review the traditional parameters in RLF report, there are five parameters in RLF report can be reuse for DAPS scenario with some modifications in field description, other parameters can be reused directly. The five parameters are as follows:

connectionFailureType This field is used to indicate whether the connection failure is due to radio link failure or handover failure. failedPCellId This field is used to indicate the PCell in which RLF is detected or the target PCell of the failed handover. For intra-NR handover nrFailedPCellId is included and for the handover from NR to EUTRA eutraFailedPCellId is included. The UE sets the ARFCN according to the frequency band used for transmission/ reception when the failure occurred. timeConnFailure This field is used to indicate the time elapsed since the last HO initialization until connection failure. Actual value = field value * 100ms. The maximum value 1023 means 102.3s or longer. timeSinceFailure This field is used to indicate the time that elapsed since the connection (radio link or handover) failure. Value in seconds. The maximum value 172800 means 172800s or longer. timeUntilReconnection This field is used to indicate the time that elapsed between the connection (radio link or handover) failure and the next time the UE comes to RRC CONNECTED in an NR or EUTRA cell. Value in seconds. The maximum value 172800 means 172800s or longer.

For DAPS HO, the UE can keep the connection with source cell and target cell simultaneously during DAPS handover execution, UE may occur RLF in source cell and also failed to access the target cell, in this case, the connection failure type parameter connectionFailureType may be RLF or HOF, similarly, there will also be two failedPCellId. To align with traditional handover, we prefer to redefine the parameter connectionFailureType and failedPCellId only to record failure of handover target cell, the RLF occurred in source cell will may be recorded in new parameter.

Proposal 10: We prefer to redefine the field connectionFailureType and failedPCellId only to record failure of handover target cell, the RLF occurred in source cell will may be recorded in new parameter.

The field timeConnFailure is used to indicate the time elapsed since the last HO initialization until connection failure. For DAPS HO, the connection failure may be the RLF in source cell or the handover access target cell failure, the definition of timeConnFailure field is not clear for DAPS HO scenario.

The field timeSinceFailure is used to indicate the time that elapsed since the connection (radio link or handover) failure. Similarly as timeConnFailure, the connection failure may be the RLF in source cell or the handover access target cell failure, the definition of timeSinceFailure field is not clear for DAPS HO scenario.

The field timeUntilReconnection is used to indicate the time that elapsed between the connection (radio link or handover) failure and the next time the UE comes to RRC CONNECTED in an NR or EUTRA cell. Similarly as timeConnFailure and timeSinceFailure, the definition of timeUntilReconnection field is not clear for DAPS HO scenario.

For the three time related parameter, we suggest to keep original definition excluding the case that RLF occurs in source cell during DAPS handover.

Proposal 11: For the three time related field, we suggest to keep original definition excluding the case that RLF occurs in source cell during DAPS handover.

- Introduce new parameters

DAPS access success or failure indicator: For DAPS HO, UE may suffer the RLF in source cell but access to the target cell successful. Although, the procedure may be regarded as successful DAPS HO, but further optimization is necessary for the type of DAPS HO. A DAPS access success or failure indicator is introduced to remind the NW to continue DAPS HO optimization for the type of successful DAPS HO.

DAPS handover type indicator: The indicator is used to indicate the last handover failure type is DAPS handover failure, it is benefit for network to optimize the DAPS configuration.

Source cell timer: As we mentioned above, the timeConnFailure field is used to indicate the time elapsed since the last HO initialization until connection failure. For DAPS scenario, the connection failure may be RLF in source cell or handover failure to the target cell. We suggest to introduce one parameter to record the time elapsed since the last HO initialization until RLF in source cell. The parameter can be used to judge whether UE handover to wrong cell, too early handover or too late handover.

Proposal 12: New parameters (DAPS access success or failure indicator, DAPS handover type indicator, Source cell timer) are suggested to be introduced in RLF report for network optimization.

- Conclusion

In conclusion, we propose the followings:

Proposal 1: We propose to add some supplementary CHO scenarios:

  - Unsuccessful CHO/HO due to late CHO/HO execution

  - Unsuccessful CHO/HO after CHO/HO execution

  - Successful CPC and PSCell change (i.e. no failure happens)

  - Unsuccessful CPC/PSCell change due to late CPC/PSCell change execution

  - Unsuccessful CPC/PSCell change after CPC/PSCell change execution

  - Successful or Unsuccessful CPC after unsuccessful CPC or PSCell change failure

Proposal 2: The SN related information reporting (e.g. RLF or PSCell change failure) should be discussed first, then optimization for CPC could be considered.

Proposal 3: For Mobility enhancement optimization, the conditional PSCell change relevant records can be considered in RLF report.

Proposal 4: In Successful or Unsuccessful  CHO after unsuccessful CHO or handover failure scenario, we prefer to record and report the two failure information or failure and successful information to network together for network optimization.

Proposal 5: It is proposed to use the HO execution instead of HO initialization in timeConnFailure field description.

Proposal 6: It is proposed to define the target PCell of the failed handover only for the first access.

Proposal 7: New parameters (CHO access success or failure indicator, CHO handover type indicator, CHO candidate cell list, multiple RLF Report) are suggested to be introduced in RLF report for network optimization.

Proposal 8:  We propose the following DAPS scenario:

  - DAPS HO failure with/without fallback to source

  - DAPS HO successful with target RLF, and then fallback to source

    - Fallback successful

    - Fallback failure

Proposal 9: We prefer to record and report the two failure information to network together for network optimization when the RLF occurred in source cell and also failed to access the target cell.

Proposal 10: We prefer to redefine the field connectionFailureType and failedPCellId only to record failure of handover target cell, the RLF occurred in source cell will may be recorded in new parameter.

Proposal 11: For the three time related field, we suggest to keep original definition excluding the case that RLF occurs in source cell during DAPS handover.

Proposal 12: New parameters (DAPS access success or failure indicator, DAPS handover type indicator, Source cell timer) are suggested to be introduced in RLF report for network optimization.

- References

[1] RAN2-111-e SONMDT Notes HuNan 2020-08-27-1030 UTC