3GPP TSG-RAN WG2 Meeting #112-e                                                    R2-2008875

Electronic, 2nd - 13th Nov, 2020

Source:	CATT

Title:	Discussion on left issue of 38.304 and 36.304

Agenda Item:	6.4.2

Document for:	Discussion and Decision

- Introduction

The email discussion of TS 38.304 and TS 36.304 corrections after RAN2#111-e Meeting [1] was discussed and the issue of cell selection and intra-frequency reselection‎ procedure is postponed to RAN2#112-e.

This contribution will further discuss this issue in Section 2 and the conclusions are summarized in Section 3.

- Discussion

2.1	Intra-frequency restriction for cell reselection

In TS 38.304 clause 8.2, the description of V2X measurements is as below:

When UE is interested to perform NR sidelink communication on non-serving frequency, it may perform measurements on that frequency or the frequencies which can provide inter carrier NR sidelink configuration for that frequency for cell selection and intra-frequency reselection purpose in accordance with TS 38.133[8]. When UE is interested to perform V2X sidelink communication on non-serving frequency, it may perform measurements on that frequency or the frequencies which can provide inter carrier V2X sidelink configuration for that frequency for cell selection and intra-frequency reselection purpose in accordance with TS 38.133[8].

Considering on the case that UE camps on F1 and the frequency which can provide inter carrier NR sidelink configuration for F2 is on F3, the UE should perform measurements on F3 for inter-frequency purpose. Thus, when the UE performs measurement on the concerned frequencies, the measurements may be used for inter-frequency as well as intra-frequency cell reselection.

In [1], 9 of 11 companies agreed to remove intra-frequency restriction for cell reselection in both 38.304 and 36.304

Proposal 1: Remove intra-frequency restriction for cell reselection when UE is ‎interested to perform NR sidelink ‎communication ‎or V2X sidelink ‎communication on non-serving frequency‎ in TS 38.304 and TS 36.304.

2.2 Cell selection

In [1], OPPO wondered why the measurement for sidelink operation is related to cell selection. Since the cell selection has been already done before the UE camping on F1 (for attach/registration), by scanning all RF and done as legacy cell selection. UE performs measurements on the non-serving frequency which provides inter or intra carrier V2X sidelink configuration only for cell reselection. Cell selection should be removed.

Proposal 2: Remove cell selection purpose in the measurement for sidelink operation in TS 38.304 and TS 36.304.

Proposal 3: Agree the CRs in [2] and [3].

- Conclusion

According to the analysis in section 2, it is proposed:

Proposal 1: Remove intra-frequency restriction for cell reselection when UE is ‎interested to perform NR sidelink ‎communication ‎or V2X sidelink ‎communication on non-serving frequency‎ in TS 38.304 and TS 36.304.

Proposal 2: Remove cell selection purpose in the measurement for sidelink operation in TS 38.304 and TS 36.304.

Proposal 3: Agree the CRs in [2] and [3].

- Reference

- Discussion paper on [Post111-e][701][V2X] 38.304 and 36.304 corrections‎

- R2-2008876	Correction to TS 38.304		CATT

- R2-2008877	Correction to TS 36.304		CATT