3GPP TSG-RAN WG2 Meeting #112-e	R2-2008907

# Electronic, 2 – 13 November 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.306 | CR | 1789 | rev |  | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to UE capabilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo, Motorola Mobility (Rapporteur) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IIOT-Core, LTE_DL_MIMO_EE-Core, LTE_eMTC5-Core, TEI16 |  |  |  |  |  | Date: |  |  | 2020-10-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - In ASN.1 the capability jointEHC-ROHC-Config-r16 has been defined instead of jointEHC-ROHC-r16. - In the description of addSRS-AntennaSwitching-r16 there is a reference to value useLegacy that does not exist in ASN.1. In TS 36.331 the value useBasic has been defined for addSRS-AntennaSwitching-r16. - Grouping of capability altFreqPriority-r16 is not aligned with TS 36.331. In 36.306 it has been grouped under “General parameters”, whereas in TS 36.331 it has been grouped under IE MeasParameters. - In ASN.1 the capability ce-InactiveState-r16 has been defined instead of ce-RRC-INACTIVE-r16. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The capability name jointEHC-ROHC-r16 has been corrected to jointEHC-ROHC-Config-r16. - In the description of addSRS-AntennaSwitching-r16 the value useLegacy has been replaced with useBasic. - The capability altFreqPriority-r16 has been moved to “Measurement parameters”. - The capability name ce-RRC-INACTIVE-r16 has been corrected to ce-InactiveState-r16. Impact analysis Impacted functionality: UE capability signaling Inter-operability: There are no interoperability issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Grouping and description of UE capabilities remains misaligned with TS 36.331. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.15, 4.3.4.221.5, 4.3.6.x (new), 4.3.8.15, 4.3.36.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

### 4.3.1	PDCP Parameters

<Text omitted>

#### 4.3.1.15	jointEHC-ROHC-Config-r16

Indicates whether the UE supports simultaneous configuration of EHC and ROHC protocols for the same DRB.

Next change

### 4.3.4	Physical layer parameters

<Text omitted>

##### 4.3.4.221.5	addSRS-AntennaSwitching-r16

Indicates the antenna switching capabilities for additional SRS symbol(s). This field can be included only if addSRS-r16 is included.

If signalled in addSRS, value useBasic indicates the antenna switching capabilities for additional SRS symbol(s) for a band of band combination for which the capability is not signalled in bandParameterList-v1610 is the same as indicated by bandParameterList-v1380 and/or bandParameterList-v1530 for the concerned band of band combination.

If signalled in bandParameterList-v1610, the field indicates the antenna switching capabilities for additional SRS symbol(s) for the concerned band of band combination.

Next change

### 4.3.6	Measurement parameters

<Text omitted>

#### 4.3.6.45	measGapPatterns-NRonly-ENDC-r16

This field indicates whether the UE supports gap patterns 2, 3 and 11 in (NG)EN-DC when the frequencies to be measured within this measurement gap are all NR frequencies.

#### 4.3.6.x	altFreqPriority-r16

This field defines whether the UE supports alternative cell reselection priority as defined in TS 36.331 [5].

Next change

### 4.3.8	General parameters

#### 4.3.8.14	dl-DedicatedMessageSegmentation-r16

Indicates whether the UE supports reception of segmented DL RRC messages.

#### 4.3.8.15	Void

Next change

### 4.3.36	E-UTRA/5GC Parameters

<Text omitted>

#### 4.3.36.10	ce-InactiveState-r16

This field indicates whether the UE supports RRC_INACTIVE state with extended DRX cycles up to 10.24s without PTW when the UE is operating in coverage enhancement mode A or B as specified in TS 36.331 [5] . A UE indicating support of ce-InactiveState-r16 shall also indicate support of ce-ModeA-r13.

End of changes