3GPP TSG RAN WG2 Meeting #112-e	R2-2008933

Electronic meeting, Nov. 2nd – Nov. 13th, 2020

Agenda item:	8.1.3

Source:	Chengdu TD Tech

Title:	NR MBS for RRC_IDLE/RRC_INACTIVE UE

Document for:	Discussion and Decision

- Introduction

The work item “NR Multicast and Broadcast Services” [1] was agreed in RAN#88-e. This work item focuses on providing both the NR MBS solution for RRC_CONNECTED UEs and the NR MBS solution for RRC_IDLE/RRC_INACTIVE UEs. In the contribution, the NR MBS solution for RRC_IDLE/ RRC_INACTIVE UEs is proposed and the related proposals are made. In detail, the following content is discussed in the contribution.

- user plane protocol stack

- control plane protocol stack

- BWP configuration

- User plane protocol stack

The user plane protocol stack of an MBS is described in [2]. The related proposals in [2] are given below for the further discussion.

Proposal 1: The unitary G-RNTI and SPS G-RNTI are allocated to an MBS to identify the MBS over Uu in the cells in the same gNB and in the cells in the different gNBs, where G-RNTI is used for the dynamic scheduling of the MBS and SPS G-RNTI is used for the SPS scheduling of the MBS.

Proposal 2: The PTM bearers of an MBS share the same SDAP entity and the same PDCP entities.

Proposal 3: The PTM bearers of an MBS have or support the same RLC configuration, which means that all PTM bearers have the same RLC parameters for the same RB. Each RLC entity of the PTM bearer takes UM or TM mode.

Proposal 4: SC-MTCH is used to carry the RLC PDUs of a RB of an MBS.

- Control plane protocol stack

In order to provide an MBS to RRC_IDLE/RRC_INACTIVE UEs, a new control channel SC-MCCH is defined for carrying the SC-PTM configuration information. The SC-PTM configuration information is the set of the configuration information of each MBS broadcast with the PTM bearer for RRC_IDLE/RRC_INACTIVE UEs in the NR cell.

In the NR cell, SC-MCCH is broadcast in the same manner as SC-MCCH in the LTE cell. The broadcast parameters for SC-MCCH in the NR cell is sent with a new SIB. Similar to each existing physical channel in the NR cell, SC-MCCH is transmitted in one BWP. In order to identify SC-MCCH in Uu in any NR cell, a fixed SC-RNTI is configured for SC-MCCH. For SC-MCCH on a BWP, the CORESET and common search space on the same BWP need to be configured to carry the dynastic scheduling information of SC-MCCH.

Based on the above discussion, the following proposals are made.

Proposal 5: A new control channel SC-MCCH is used to carry the SC-PTM configuration information.

Proposal 6: The broadcast parameters for SC-MCCH in the NR cell is sent with a new SIB.

Proposal 7: A fixed SC-RNTI is configured for identifying SC-MCCH in Uu in any NR cell.

Proposal 8：Configure the CORESET and common search space for SC-MCCH to carry the dynastic scheduling information of SC-MCCH on the BWP broadcasting SC-MCCH.

The control plane protocol stack as shown in the following figure can be used for SC-MCCH.

Figure 1: NR MBS control plane stack for SC-MCCH

According to the above control plane stack for SC-MCCH, a PDCP entity and a RLC entity are configured for SC-MCCH. RRC generates the SC-PTM configuration information for SC-MCCH. The corresponding information is sent to the PDCP entity. The RLC entity takes UM mode. In the physical layer, SC-MCCH is mapped onto an independent DL-SCH and the DL-SCH is mapped onto an independent PDSCH. The dynamic scheduling information of the SC-MCCH specific PDSCH is sent on the PDCCH with the CRC of the PDCCH scrambled with SC-RNTI.

Based on the above discussion, the following proposal is made.

Proposal 9: Configure a PDCP entity and a RLC entity with UM mode for SC-MCCH.

- BWP configuration

As described in [3], in order to provide an MBS for RRC_IDLE/RRC_INACTIVE UEs with no negative influence on their camping on the cell, the frequency range [F1, F2] with the initial DL BWP contained can be configured. The frequency range [F1, F2] can be divided into several MBS specific BWPs for RRC_IDLE/RRC_INACTIVE UEs with each BWP denoted by II-UE BWP. Each II-UE BWP contains the initial DL BWP. Due to the fact that the RF channel bandwidth of a UE is limited, the bandwidth of the frequency range [F1,F2] may not satisfy the bandwidth requirement of the MBS for RRC_IDLE/RRC_INACTIVE UEs. In order to satisfy the bandwidth requirement of the MBS for RRC_IDLE/RRC_INACTIVE UEs, the secondary II-UE BWP can be configured. Each secondary II-UE BWP is configured outside the above frequency range. Each II-UE BWP can provide the same information for a RRC_IDLE/RRC_INACTIVE UE as the initial DL BWP does. Therefore, a RRC_IDLE/RRC_INACTIVE UE can receive an MBS on a secondary II-UE BWP without affecting its camping on the cell. The information provided by a secondary II-UE BWP includes: the system information and the aging information. In order for a RRC_IDLE/RRC_INACTIVE UE to know a new MBS on SC-MCCH in time, it’s better to transmit SC-MCCH on each secondary II-UE BWP.

Based on the above discussion, the following proposals are made：

Proposal 10: The frequency range [F1, F2] with the initial DL BWP contained can be divided into several MBS specific BWPs for RRC_IDLE/RRC_INACTIVE UEs (II-UE BWP). Each II-UE BWP contains the initial DL BWP.

Proposal 11: One or more secondary II-UE BWPs can be configured with each secondary II-UE BWP outside the frequency range [F1, F2]. Each secondary II-UE BWP can provide the same information for a RRC_IDLE/RRC_INACTIVE UE as the initial DL BWP does.

Proposal 12: SC-MCCH is transmitted on each secondary II-UE BWP.

Proposal 13: Configure the CORESET and search space groups for SC-MCCH on the initial DL BWP and each secondary II-UE BWP.

- Conclusion

Based on the discussion in the above sections, the following proposals for NR MBS for RRC_IDLE/RRC_INACTIVE UEs are made and expected to be agreed.

On the user plane protocol stack:

Proposal 1: The unitary G-RNTI and SPS G-RNTI are allocated to an MBS to identify the MBS over Uu in the cells in the same gNB and in the cells in the different gNBs, where G-RNTI is used for the dynamic scheduling of the MBS and SPS G-RNTI is used for the SPS scheduling of the MBS.

Proposal 2: The PTM bearers of an MBS share the same SDAP entity and the same PDCP entities.

Proposal 3: The PTM bearers of an MBS have or support the same RLC configuration, which means that all PTM bearers have the same RLC parameters for the same RB. Each RLC entity of the PTM bearer takes UM or TM mode.

Proposal 4: SC-MTCH is used to carry the RLC PDUs of a RB of an MBS.

On the control plane protocol stack:

Proposal 5: A new control channel SC-MCCH is used to carry the SC-PTM configuration information.

Proposal 6: The broadcast parameters for SC-MCCH in the NR cell is sent with a new SIB.

Proposal 7: A fixed SC-RNTI is configured for identifying SC-MCCH in Uu in any NR cell.

Proposal 8：Configure the CORESET and common search space for SC-MCCH to carry the dynastic scheduling information of SC-MCCH on the BWP broadcasting SC-MCCH.

Proposal 9: Configure a PDCP entity and a RLC entity with UM mode for SC-MCCH.

On the BWP configuration:

Proposal 10: The frequency range [F1, F2] with the initial DL BWP contained can be divided into several MBS specific BWPs for RRC_IDLE/RRC_INACTIVE UEs (II-UE BWP). Each II-UE BWP contains the initial DL BWP.

Proposal 11: One or more secondary II-UE BWPs can be configured with each secondary II-UE BWP outside the frequency range [F1, F2]. Each secondary II-UE BWP can provide the same information for a RRC_IDLE/RRC_INACTIVE UE as the initial DL BWP does.

Proposal 12: SC-MCCH is transmitted on each secondary II-UE BWP.

Proposal 13: Configure the CORESET and search space groups for SC-MCCH on the initial DL BWP and each secondary II-UE BWP.

Reference

- RP-201038, Revised Work Item on NR Multicast and Broadcast Services, RAN Meeting #88-e.

- R2-2008929, Discussion on the protocol stack for NR MBS, RAN2 Meeting #112-e

- R1-2007639, Basic functions for MBS for RRC_IDLE/RRC_INACTIVE UEs, RAN1 Meeting#103-e