3GPP TSG-RAN WG2 #112-e	R2-2008938

E-meeting, November 2020

Agenda Item:	6.4.4

Source:	OPPO, Huawei, HiSilicon, Samsung

Title:	Correction on LTE V2X UE capability

Document for:	Discussion, Decision

Introduction

This is to correct one left issue in LTE V2X UE capability.

Discussion

In TS 36.331 V16.2.0, the capability for mixed LTE-PC5 and NR-PC5 BC lists was not correctly defined [1], i.e., the dimension of band within a BC is missing

SL-Parameters-v1610 ::=					SEQUENCE {

sl-ParameterNR-r16							OCTET STRING					OPTIONAL,

v2x-SupportedBandCombinationListNR-r16	V2X-SupportedBandCombinationNR-r16	OPTIONAL

}

V2X-SupportedBandCombinationNR-r16	::=	SEQUENCE (SIZE (1..maxBandCombSidelinkNR-r16)) OF V2X-BandCombinationParametersNR-r16

V2X-BandCombinationParametersNR-r16 ::=	CHOICE {

eutra									SEQUENCE {

v2x-BandParameters1-r16				V2X-BandParameters-r14		OPTIONAL,

v2x-BandParameters2-r16				V2X-BandParameters-v1530		OPTIONAL

},

nr										SEQUENCE {

v2x-BandParametersNR-r16					OCTET STRING				OPTIONAL

}

}

In order to correct this error, there are two alternatives: NBC change and BC change. Considering Rel-16 specification has been frozen, backwards compatibility is preferred.

With BC change, the wrong IE can be dummied

SL-Parameters-v1610 ::=					SEQUENCE {

sl-ParameterNR-r16							OCTET STRING					OPTIONAL,

dummy	V2X-SupportedBandCombinationNR-r16	OPTIONAL

}

SL-Parameters-v16xy ::=					SEQUENCE {

v2x-SupportedBandCombinationListNR-v16xy	V2X-SupportedBandCombinationNR-v16xy	OPTIONAL

}

V2X-SupportedBandCombinationNR-r16	::=	SEQUENCE (SIZE (1..maxBandCombSidelinkNR-r16)) OF V2X-BandParametersNR-r16

V2X-SupportedBandCombinationNR-v16xy	::=	SEQUENCE (SIZE (1..maxBandCombSidelinkNR-r16)) OF V2X-BandCombinationParametersNR-v16xy

V2X-BandCombinationParametersNR-v16xy	::=	SEQUENCE (SIZE (1.. maxSimultaneousBands-r10)) OF V2X-BandParametersNR-r16

V2X-BandParametersNR-r16 ::=	CHOICE {

eutra									SEQUENCE {

v2x-BandParameters1-r16				V2X-BandParameters-r14		OPTIONAL,

v2x-BandParameters2-r16				V2X-BandParameters-v1530		OPTIONAL

},

nr										SEQUENCE {

v2x-BandParametersNR-r16					OCTET STRING				OPTIONAL

}

}

RAN2 needs to fix this issue as soon as possible.

RAN2 solves the error in V2X BC list in LTE spec in a backwards compatible way.

Conclusion

We have the following proposal:

Proposal 1	RAN2 solves the error in V2X BC list in LTE spec in a backwards compatible way.

Reference

- R2-2008345   CR for V2X UE capability     OPPO     CR   Rel-16    36.331 16.1.1     4349       1     B     5G_V2X_NRSL-Core