3GPP TSG-RAN WG2 Meeting #112                                                    	 R2-2008999

E-Meeting, USA, Nov 2nd - 13th, 2020

Agenda item: 	8.13.2.1

Source:	Intel Corporation

Title: 	CHO support for MDT/SON

Document for:	Discussion and Decision

# Introduction

During last meeting, it has been agreed:

=>	RAN2 to consider the SON aspects of CHO and SON aspects of 2-step RA as part of the WI.

=>	RAN2 to consider the SON aspects of DAPS HO as part of the WI.

=>	The following scenarios are considered:

- 1) Successful CHO and HO (i.e. no failure happens). FFS consideration in RAN2/3

- 2) Unsuccessful CHO due to late CHO execution.

- 3) Unsuccessful CHO after CHO execution.

- 4) Successful or Unsuccessful  CHO after unsuccessful CHO or handover failure.

- Note: other scenarios are not ruled out…

=>	RAN2 should study what CHO failure information can be stored in RLF report.

=>	RAN 2 to discuss the method for distinguishing between different handover types in RLF report. FFS the details, e.g., explicitly way or not.

=>	RAN2 to agree studying the RLF report and/or FailureInformation message contents in the DAPS failure scenarios.

=>	New logged content for 2-step RA is introduced in:

- RA report

- RLF report

- CEF report

=>	Study the necessity of introducing new method for more precise identification of the DL coverage quality during the UL coverage outage.

In this contribution, we would like to discuss further detail of CHO support for MDT/SON.

# Discussion

In Rel16, NR introduced conditional handover (CHO) where the network will configure conditional handover condition to the UE. The UE will elevate the condition during measurement, if one or more condition is met, the UE will perform conditional handover execution where the UE will handover (performing RACH) to the target cell. However, there may be cases where the UE HO will fail, below shows the cases and the information the UE should store for MDT/SON reporting. The follow cases and information for each case should be reported to the network:

- Case 1: RLF in source cell before CHO execution:

  - In this case, the UE faces RLF in source cell before CHO is triggered

  - The reason could be poor execution condition configuration (for example, the condition configured RSRP to high or “AND” condition is configured but one of the conditions never met or different target cell is configured)

  - Case 1-1: RLF in source cell before CHO execution but recovery successful using CHO

    - In this case, the UE RLF at source cell but then perform recovery using CHO and successful

    - Information UE should store for reporting:

      - Indication of RLF at source before CHO execution and recovery successful with CHO,

      - Corresponding measurements for candidate cells

      - Corresponding cell ID

      - Time of receiving condition configuration

      - Time of RLF at source

      - Recovery cell information (includes cell ID, measurement, RACH information, time etc)

  - Case 1-2: RLF in source cell before CHO execution but recovery failure using CHO and then perform reestablishment

    - Information UE should store for reporting:

      - Indication of RLF at source before CHO execution and recovery failure with CHO,

      - The execution condition that is configured to the UE (per candidate cell),

      - Corresponding measurements for candidate cells

      - Corresponding cell ID

      - Time of receiving condition configuration

      - Time of RLF at source

      - Recovery cell information (includes cell ID, measurement, RACH information, time etc)

  - Case 1-3: Reestablishment

    - In this case, there is no candidate cells are good enough during cell selection after failure

    - Information UE should store for reporting:

      - Indication of reestablishment after reestablishment failure

      - The execution condition that is configured to the UE (per candidate cell),

      - Corresponding measurements for candidate cells

      - Corresponding cell ID

      - Time of receiving condition configuration

      - Time of RLF at source

      - Cell information during cell selection (includes cell ID, measurement, RACH information, time etc)

- Case 2: RLF does not triggered in source before CHO is triggered

  - In this case, RLF does not triggered in source before CHO is triggered

  - Then CHO is triggered but fail

  - The reason in this case may be due to poor execution condition configuration for candidate cells

  - Case 2-1: CHO failure and then recovery successfully based on CHO

    - In this case, the CHO failure and UE perform recovery using CHO and successful

    - Information UE should store for reporting:

      - Indication of CHO failure and CHO recovery success,

      - Source cell measurement when CHO failure happens

      - The execution condition that is configured to the UE (per candidate cell),

      - Corresponding measurements for candidate cells

      - Corresponding cell ID

      - Time of receiving condition configuration

      - CHO Execution cell information (includes cell ID, measurement, RACH information, time perform CHO execution etc

      - Recovery cell information (includes cell ID, measurement, RACH information, time etc)

  - Case 2-2: CHO failure and then perform reestablishment

    - In this case, the CHO failure and UE perform recovery using CHO and fail as well

    - Information UE should store for reporting:

      - Indication of CHO failure and CHO recovery failure,

      - Source cell measurement when CHO failure happens

      - The execution condition that is configured to the UE (per candidate cell),

      - Corresponding measurements for candidate cells

      - Corresponding cell ID

      - Time of receiving condition configuration

      - CHO Execution cell information (includes cell ID, measurement, RACH information, time perform CHO execution etc

      - Recovery cell information (includes cell ID, measurement, RACH information, time etc)

  - Case 2-3: reestablishment (no candidate cells are good during cell selection)

    - In this case, after case 2-2, and restablishment triggered and no good cell for cell selection

    - Information UE should store for reporting:

      - Indication of CHO failure and reestablishment failure

      - Source cell measurement when CHO failure happens

      - The execution condition that is configured to the UE (per candidate cell)

      - Corresponding measurements for candidate cells

      - Corresponding cell ID

      - Time of receiving condition configuration

      - CHO Execution cell information (includes cell ID, measurement, RACH information, time perform CHO execution etc

      - Cell information during cell selection (includes cell ID, measurement, RACH information, time etc)

Proposal: RAN2 consider the information to reported in each case in this contribution.

# Conclusion

Proposal: RAN2 considers the information to reported to the network in each case below:

  - Case 1-1: RLF in source cell before CHO execution but recovery successful using CHO

    - Information UE should store for reporting:

      - Indication of RLF at source before CHO execution and recovery successful with CHO

      - The execution condition that is configured to the UE (per candidate cell)

      - Corresponding measurements for candidate cells

      - Corresponding cell ID

      - Time of receiving condition configuration

      - Time of RLF at source

      - Recovery cell information (includes cell ID, measurement, RACH information, time etc)

  - Case 1-2: RLF in source cell before CHO execution but recovery failure using CHO

    - Information UE should store for reporting:

      - Indication of RLF at source before CHO execution and recovery failure with CHO

      - The execution condition that is configured to the UE (per candidate cell)

      - Corresponding measurements for candidate cells

      - Corresponding cell ID

      - Time of receiving condition configuration

      - Time of RLF at source

      - Recovery cell information (includes cell ID, measurement, RACH information, time etc)

  - Case 1-3: Reestablishment

    - In this case, there is no candidate cells are good enough during cell selection after failure

    - Information UE should store for reporting:

      - Indication of reestablishment after CHO recovery failure

      - The execution condition that is configured to the UE (per candidate cell)

      - Corresponding measurements for candidate cells

      - Corresponding cell ID

      - Time of receiving condition configuration

      - Time of RLF at source

      - Recovery cell information (includes cell ID, measurement, RACH information, time etc)

      - Cell information during cell selection (includes cell ID, measurement, RACH information, time etc)

  - Case 2-1: CHO failure and then recovery successfully based on CHO

    - In this case, the CHO failure and UE perform recovery using CHO and successful

    - Information UE should store for reporting:

      - Indication of CHO failure and CHO recovery success

      - Source cell measurement when CHO failure happens

      - The execution condition that is configured to the UE (per candidate cell)

      - Corresponding measurements for candidate cells

      - Corresponding cell ID

      - Time of receiving condition configuration

      - CHO Execution cell information (includes cell ID, measurement, RACH information, time perform CHO execution etc

      - Recovery cell information (includes cell ID, measurement, RACH information, time etc)

  - Case 2-2: CHO failure and then recovery failure based on CHO

    - Information UE should store for reporting:

      - Indication of CHO failure and CHO recovery failure

      - Source cell measurement when CHO failure happens

      - The execution condition that is configured to the UE (per candidate cell)

      - Corresponding measurements for candidate cells

      - Corresponding cell ID

      - Time of receiving condition configuration

      - CHO Execution cell information (includes cell ID, measurement, RACH information, time perform CHO execution etc

      - Recovery cell information (includes cell ID, measurement, RACH information, time etc)

  - Case 2-3: reestablishment (no candidate cells are good during cell selection)

    - Information UE should store for reporting:

      - Indication of CHO failure and CHO recovery failure,

      - Source cell measurement when CHO failure happens

      - The execution condition that is configured to the UE (per candidate cell),

      - Corresponding measurements for candidate cells

      - Corresponding cell ID

      - Time of receiving condition configuration

      - CHO Execution cell information (includes cell ID, measurement, RACH information, time perform CHO execution etc

      - Recovery cell information (includes cell ID, measurement, RACH information, time etc)

      - Cell information during cell selection (includes cell ID, measurement, RACH information, time etc)

# Reference

[1] “New WID on enhancement of data collection for SON/MDT in NR”, RP-201281