3GPP TSG-RAN WG2 Meeting #112 electronic				R2-2009002

Elbonia, Nov 2nd – 13th 2020

Agenda item:		8.11.2

Source:			Intel Corporation

Title:		Support of positioning in idle/inactive mode

Document for:	 	Discussion and decision

# Introduction

In last meeting, RAN2 agreed to study positioning in idle/inactive mode, on-demand PRS and latency analysis in the study phase.

In this contribution, we discuss how to support the positioning in IDLE/INACTIVE mode.

# Discussion

The support of positioning in IDLE/INACTIVE was discussed in [1], and got lots of supports. The benefit to support positioning in IDLE/INACTIVE without state transmission are:

- Can allow higher number of UEs to support positioning simultaneously, esp. for IIoT scenario;

- Low power consumption;

- Low signaling overhead;

- Low latency;

Therefore in Rel-17, the support of positioning in IDLE/INACTIVE is to support positioning in IDLE/INACTIVE without state transition.

Proposal 1: Support positioning in IDLE/INACTIVE without state transmission;

According to [2], below positioning methods and versions are supported in NR.

Table 4.3.1-1: Supported versions of UE positioning methods

| Method | UE-based | UE-assisted, LMF-based | NG-RAN node assisted | SUPL |
| --- | --- | --- | --- | --- |
| A-GNSS | Yes | Yes | No | Yes |
| OTDOA Note1, Note 2 | No | Yes | No | Yes |
| E-CID Note 4 | No | Yes | Yes | Yes for E-UTRA |
| Sensor | Yes | Yes | No | No |
| WLAN | Yes | Yes | No | Yes |
| Bluetooth | No | Yes | No | No |
| TBS Note 5 | Yes | Yes | No | Yes (MBS) |
| DL-TDOA | Yes | Yes | No | Yes |
| DL-AoD | Yes | Yes | No | Yes |
| Multi-RTT | No | Yes | Yes | Yes |
| NR E-CID | No | Yes | Yes | Yes (DL NR E-CID) |
| UL-TDOA | No | No | Yes | Yes |
| UL-AoA | No | No | Yes | Yes |

In [Post111-e][625][POS] End-to-end latency analysis (Intel), a related issue was discussed for UE based positioning:

Scenario: For UE based positioning, the UE does not need to have NAS signaling connection. The UE could be in IDLE mode, and obtain the assistance data via system information, and perform the measurements and provide the results to internal APP.

However companies have different view on the assumption. A company commented that the scenario can work if the assumption is, the LCS client is internal to the UE, and only DL positioning is used, but that is not exactly same as the definition of UE based positioning.  Therefore from implementation perspective, it is possible, i.e. if the location is used inside the UE, the UE can obtain the location without the interaction with LMF, and without the NAS connection with CoreNetwork.

In addition, RAN2 introduced on demand SI request for positioning assistance data in rel-16, and one of the solution is dedicated for IDLE/INACTIVE UE. To our understanding, the motivation is to support it  for IDLE/INACTIVE UE.

Proposal 2: To confirm, in Rel-16, IDLE/INACTIVE UE can obtain UE location without  the need to have NAS signalling connection with Core network, i.e. without state transition;

For MT-LR, MO-LR, the positioning procedure is triggered by the Core network and the positioning method is selected by the LMF, in addition, for MT-LR, the UE needs to report the location back to the LMF. Anyway the Core Network will page the UE if there is no NAS connection for MT-LR case. Therefore it does not work for IDLE mode UE, i.e. IDLE mode UE has to transit to CONNECTED mode in order to receive the request from the LMF and report the location back to the LMF (for MT-LR) even if the UE can get the assistance data from broadcast signalling.

For INACTIVE UE, it can save UE power and reduce the latency if the UE do not need state transition when performing the positioning. And then the UE needs to receive the DL signalling, measure DL PRS and transfer UL signalling in INACTIVE.

Proposal 3: In Rel-17,  only consider INACTIVE UE in the discussion on how to support UE based positioning without state transition.

Proposal 4: To support UE based positioning without state transition, the INACTIVE UE should be able to:

- Receive the DL request from the LMF;

- Obtain the assistance data via broadcast signalling;

- Measure PRS and calculate the location;

- Transmit the location back to the LMF;

For UE assisted positioning, even if the UE can get the assistance data via broadcast signalling. We still cannot avoid the signalling exchange between the UE and the LMF for at least location response regardless of MO-LR, MT-LR, etc since the final calculation  is done in the LMF. Therefore it is impossible to avoid the state transmission for IDLE mode UE.

Proposal 5: In Rel-17, only consider INACTIVE UE in the discussion on how to support UE assisted positioning without state transition.

Same as UE based positioning, it can save UE power and reduce the latency if the UE based positioning can be done without the state transition. But to support all positioning methods, DL-TDOA, DL-AoD, UL-TDOA, UL-AoA, E-CID, Multi-RTT, the INACTIVE UE should be able to:

- Transfer SRS in INACTIVE;

- Measure PRS in INACTIVE;

- Receive DL signalling in INACTIVE;

- Transmit UL signalling in INACTIVE

Proposal 6: To support UE assisted positioning without state transition, the INACTIVE UE should be able to:

- Transfer SRS;

- Measure PRS;

- Receive DL signalling;

- Transmit UL signalling;

There is ongoing Rel-17 SI SDT, the UE may transfer the data to the network without state transition. But the problem is, for positioning LPP message is carried via NAS/RRC, i.e. signalling, but SDT so far  only covers data in their discussion. Therefore we may need to ask SDT SI/WI to revise their scope if we conclude the need of this in positioning SI.

Proposal 7:  How to enable the reception  of DL signalling and the transmission of SRS/UL signalling over SDT in INACTIVE can be discussed under Rel-17 SDT WI.

# Conclusion

Based on the discussion, we have following proposals:

Proposal 1: Support positioning in IDLE/INACTIVE without state transmission;

Proposal 2: To confirm, in Rel-16, IDLE/INACTIVE UE can obtain UE location without  the need to have NAS signalling connection with Core network, i.e. without state transition;

Proposal 3: In Rel-17, only consider INACTIVE UE in the discussion on how to support UE based positioning without state transition.

Proposal 4: To support UE based positioning without state transition, the INACTIVE UE should be able to:

- Receive the DL request from the LMF;

- Obtain the assistance data via broadcast signalling;

- Measure PRS and calculate the location;

- Transmit the location back to the LMF;

Proposal 5: In Rel-17, only consider INACTIVE UE in the discussion on how to support UE assisted positioning without state transition.

Proposal 6: To support UE assisted positioning without state transition, the INACTIVE UE should be able to:

- Transfer SRS;

- Measure PRS;

- Receive DL signalling;

- Transmit UL signalling;

Proposal 7:  How to enable the reception  of DL signalling and the transmission of SRS/UL signalling over SDT in INACTIVE can be discussed under Rel-17 SDT WI.

# Reference

[1] RP-192412, Email Summary on NR Positioning, Qualcomm

[2] TS38.305