3GPP TSG RAN WG2 Meeting #112-e                                                         R2-2009022

# Electronic meeting, Nov 2nd - 13th 2020

Agenda item:		8.12.3

Source:			Intel Corporation

Title:		Relax measurement for stationary and low mobility devices

Document for:	 	Discussion and decision

# Introduction

During last meeting, the following agreements have been made:

Agreements via email - from offline 108:

- RAN2 studies, and provides input to TR 38.875, on whether and how it can be ensured RedCap UEs are used only for intended use cases. This may require coordination with other WGs (e.g. RAN3 / SA / CT).

- RAN2 studies, and provides input to TR 38.875, on how and when to identify RedCap UEs and how to control RedCap UE access in RAN. Before concluding the identification discussion, further progress is needed in RAN1.

Agreements:

- For power saving, for now RAN2 studies extended DRX for idle and inactive modes and RRM relaxation for stationary RedCap devices, and input to be provided to TR 38.875.

- Depending on RAN1 input, discussion is expected at least on the following impacts on RAN2 procedures:

a.	Impact on cell (re)selection

b.	Impact on initial access

c.	Impact on other idle mode procedures (i.e. SI acquisition, paging)

FFS:

- Whether reduction of upper layer capabilities should be considered is FFS (in any case no email discussion until the next meeting on this)

During email discussion [915], majority of the companies prefer to support up to device moving slowly for REDCAP UE and 2 levels of relaxation based on the mobility. In this contribution, we discuss how the mobility level can be defined as well as the corresponding RRM relaxation.

# Discussion

## Email discussion

During the email discussion, companies were asked preference on up to which of the following mobility level should be supported in REDCAP UE:

- Level 1: still device at fixed location (e.g. fixed static sensor)

- Level 2: moving (e.g. rotary) device at a fixed location (e.g. camera, robot) [12]

- Level 3: temporarily fixed device (e.g. smart watch at night) [RAN2#111-e on-line comment]

- Level 4: device is moving around slowly (e.g. medical wearables)

Majority of the companies prefer to support up to level 4 where the UE is moving slowly.

Observation 1: Majority of the companies prefer to support up to level 4 where the UE is moving slowly for RECAP UE.

Then, the companies were asked to indicate how many relation levels should be considered for REDCAP UE. Majority of the companies prefer 2 levels. In this case, the 2 level of the RRM relaxation can be mapped to 2 mobility levels.

Observation 2: Majority of the companies prefer to support 2 RRM relaxation levels.

Combining observation 1 and 2, we can map 2 mobility state (fixed, slow mobility) to 2 RRM relaxation levels.

Proposal 1: RAN2 defines 2 RRM relaxation levels for stationary and low mobility state UE.

Two options to identify stationary during the email discussion below:

- Option 1: a UE non-mobility attribution (subscription information)

- Option 2: evaluated by criterions based on measurements

Majority of the companies prefer option 2 which is similar to current Rel16 measurement relaxation feature. In Rel-16, it allows UEs in RRC_IDLE and RRC_INACTIVE to relax measurements on neighbour cells when it meets the rules and criteria determining that UE is in “low mobility” and/or “not at cell edge”. The target scenarios aim for UEs with a very deterministic usage and may benefit of being identified as stationary from subscription (without having apply/meet the Rel-16 criteria for “low mobility” and/or “not at cell edge”. Below is the related specification in TS38.304:

##### 5.2.4.9.1              Relaxed measurement criterion for UE with low mobility

The relaxed measurement criterion for UE with low mobility is fulfilled when:

-    (SrxlevRef – Srxlev) < SSearchDeltaP,

Where:

-    Srxlev = current Srxlev value of the serving cell (dB).

-    SrxlevRef = reference Srxlev value of the serving cell (dB), set as follows:

-     After selecting or reselecting a new cell, or

-     If (Srxlev - SrxlevRef) > 0, or

-     If the relaxed measurement criterion has not been met for TSearchDeltaP:

-     The UE shall set the value of SrxlevRef to the current Srxlev value of the serving cell.

However, in order to support 2 mobility states (stationary and low mobility states) and apply the corresponding RRM relaxations, some modification may be needed. There are following options based on modification of the current relaxed measurement criterion in TS38.304:

- Option 1: allow 2 level of configurations based on RSRP measurement of serving cell, for example:

  - Stationary: (SrxlevRef – Srxlev) < SSearchDeltaP_stationary

  - Low mobility: SSearchDeltaP_stationary <= (SrxlevRef – Srxlev) < SSearchDeltaP_low_mobility

- Option 2: take into account of beam switching, for example:

  - Stationary:

    - number of beam switch < N1 or

    - no beam switch and (SrxlevRef – Srxlev) < SSearchDeltaP_stationary

  - Low mobility:

    - number of beam switch < N2 or

    - SSearchDeltaP_stationary <= (SrxlevRef – Srxlev) < SSearchDeltaP_low_mobility

Option 1 is a simple extension from the current specification and has the smallest impact to the specification. However, it doesn’t consider the beam operation hence the measurement is not accurate when beam switch happens. Option 2 considers the beam operation using the number of beam switch. However, in the case where the UE is stationary, the number of beam switch can still be very high (e.g. the UE is spinning). In that case, the relax of RRM measurement will not be available. On the other hand, if the number of beam switch is small, the chance of a high-speed UE is very low. For this reason, the method still seems reasonable.

Proposal 2: RAN2 to agree enhance rel-16 criteria by taking into account beam switching.

Proposal 3: FFS on how to modify existing relaxed measurement taken into account of beam switching and to support 2 levels of RRM relaxation.

In addition, measurements of the serving cell are currently not relaxed as part of Rel-16 measurement relaxation feature due to the concerns raised on the performance impact. However, at least for stationary devices, it is reasonable to also relax measurement for serving cell since the UE is not moving.

Proposal 4: RAN2 to allow measurement relaxation of the serving cell for stationary UEs.

# Conclusion

Observation 1: Majority of the companies prefer to support up to level 4 where the UE is moving slowly for RECAP UE.

Observation 2: Majority of the companies prefer to support 2 RRM relaxation levels.

Proposal 1: RAN2 defines 2 RRM relaxation levels for stationary and low mobility state UE.

Proposal 2: RAN2 to agree enhance rel-16 criteria by taking into account beam switching.

Proposal 3: FFS on how to modify existing relaxed measurement taken into account of beam switching and to support 2 levels of RRM relaxation.

Proposal 4: RAN2 to allow measurement relaxation of the serving cell for stationary UEs.

# Reference

- RP-201386, Rel-17 Study on support of reduced capability NR devices, July 2020.

- R2-152171, Performance evaluation of extended DRX cycle in idle mode, Rel-13 LTE, Intel Corporation, May 2015.

- C1-191389, Reply LS on eDRX cycles for CM-CONNECTED with RRC inactive, Rel-16 5G_CIoT, From CT1, To SA2, Ccing RAN2 and CT4, March 2019.