3GPP TSG-RAN WG2 Meeting #112 electronic				R2-2009023

Elbonia, Nov 2nd – 13th 2020

Agenda item:		8.11.2

Source:			Intel Corporation

Title:		Solution directions to reduce end-to-end latency

Document for:	 	Discussion and decision

# Introduction

In last meeting, RAN2 agreed to study positioning in idle/inactive mode, on-demand PRS and latency analysis in the study phase.

In addition, in the email discussion [Post111-e][625][POS] End-to-end latency analysis (Intel), the E2E latency was discussed.

In this contribution, we discuss the potential solution directions on how to reduce the E2E latency.

# Discussion

Based on [1], using Multi-RTT as example:

The E2E latency for Multi-RTT is

| Positioning technique [Multi-RTT] [UE-A] Figure 3 |  |  |
| --- | --- | --- |
| Latency Component | Value Range (ms) | Description of Latency Component |
| Step 1 LPP Request capabilities | 18-34 | TLMFProc+ TAMF-LMF+ TAMFProc +TgNB-AMF + TgNBProc-NAS/LPP +TUE-gNB+ TUEProc-RRCDLInfo Processing delays: 14ms - UE: TUEProc-RRCDLInfo= 5ms - gNB: TgNBProc-NAS/LPP= 3ms - AMF: TAMFProc= 3ms - LMF: TLMFProc= 3ms Signalling delay:4-20.5ms - UE-gNB: TUE-gNB= 0-0.5ms - gNB-AMF: TgNB-AMF= 3-10ms - AMF-LMF: TAMF-LMF= 1-10ms Note, the LPP capability processing delay is counted together in response message. |
| Step 2 LPP Provide Capabilities | 25-54.5 | TLMFProc+ TAMF-LMF+ TAMFProc +TgNB-AMF + TgNBProc-NAS/LPP +TUE-gNB+ TUEProc-RRCULInfo+ TUEProc-LPPCapab Processing delays: 21-34ms - UE: TUEProc-RRCULInfo= 2-5ms TUEProc-LPPCapab= 10-20ms - gNB: TgNBProc-NAS/LPP= 3ms - AMF: TAMFProc= 3ms - LMF: TLMFProc= 3ms Signalling delay:4-20.5ms - UE-gNB: TUE-gNB= 0-0.5ms - gNB-AMF: TgNB-AMF= 3-10ms - AMF-LMF: TAMF-LMF= 1-10ms |
| Step 3 NRPPa POSITIONING INFORMATION REQUEST | 13-29 | TLMFProc+ TAMF-LMF+ TAMFProc +TgNB-AMF + TgNBProc-NRPPa Processing delays: 9 ms - gNB: TgNBProc-NRPPa= 3ms - AMF: TAMFProc= 3ms - LMF: TLMFProc= 3ms Signalling delay:4-20ms - gNB-AMF: TgNB-AMF= 3-10ms - AMF-LMF: TAMF-LMF= 1-10ms |
| Step 4 RRC SRS configuration | 13-13.5 | TgNBProc-RRC+ TUE-gNB+ TUEProc-RRCReconf Processing delays: 13ms - UE: TUEProc-RRCReconf = 10ms - gNB: TgNBProc-RRC= 3ms Signalling delay:0-0.5ms - UE-gNB: TUE-gNB= 0-0.5ms |
| Step 5 NRPPa POSITIONING INFORMATION RESPONSE | 13-29 | TLMFProc+ TAMF-LMF+ TAMFProc +TgNB-AMF + TgNBProc-NRPPa Processing delays: 9 ms - gNB: TgNBProc-NRPPa= 3ms - AMF: TAMFProc= 3ms - LMF: TLMFProc= 3ms Signalling delay:4-20ms - gNB-AMF: TgNB-AMF= 3-10ms - AMF-LMF: TAMF-LMF= 1-10ms |
| Step 6 NRPPa Request UE SRS activation | 13-29 | TLMFProc+ TAMF-LMF+ TAMFProc +TgNB-AMF + TgNBProc-NRPPa Processing delays: 9 ms - gNB: TgNBProc-NRPPa= 3ms - AMF: TAMFProc= 3ms - LMF: TLMFProc= 3ms Signalling delay:4-20ms - gNB-AMF: TgNB-AMF= 3-10ms - AMF-LMF: TAMF-LMF= 1-10ms |
| Step 7 MAC Activate UE SRS transmission | 1-3.5 | TUE-gNB +TUEProc-MAC-SRSAct Processing delays: 13ms - UE: TUEProc-MAC-SRSAct = 1-3ms Signalling delay:0-0.5ms - UE-gNB: TUE-gNB= 0-0.5ms |
| Step 8 NRPPa Request UE SRS activate Response | 13-29 | TLMFProc+ TAMF-LMF+ TAMFProc +TgNB-AMF + TgNBProc-NRPPa Processing delays: 9 ms - gNB: TgNBProc-NRPPa= 3ms - AMF: TAMFProc= 3ms - LMF: TLMFProc= 3ms Signalling delay:4-20ms - gNB-AMF: TgNB-AMF= 3-10ms - AMF-LMF: TAMF-LMF= 1-10ms |
| Step 9 NRPPa MEASUREMENT REQUEST | 13-29 | TLMFProc+ TAMF-LMF+ TAMFProc +TgNB-AMF + TgNBProc-NRPPa Processing delays: 9 ms - gNB: TgNBProc-NRPPa= 3ms - AMF: TAMFProc= 3ms - LMF: TLMFProc= 3ms Signalling delay:4-20ms - gNB-AMF: TgNB-AMF= 3-10ms - AMF-LMF: TAMF-LMF= 1-10ms |
| Step 10 LPP Provide Assistance Data | 28-44.5 | TLMFProc+ TAMF-LMF+ TAMFProc +TgNB-AMF + TgNBProc-NAS/LPP +TUE-gNB+ TUEProc-RRCDLInfo+ TUEProc-LPPAssi Processing delays: 24ms - UE: TUEProc-RRCDLInfo= 5ms TUEProc-LPPAssi= 10ms - gNB: TgNBProc-NAS/LPP= 3ms - AMF: TAMFProc= 3ms - LMF: TLMFProc= 3ms Signalling delay:4-20.5ms - UE-gNB: TUE-gNB= 0-0.5ms - gNB-AMF: TgNB-AMF= 3-10ms - AMF-LMF: TAMF-LMF= 1-10ms |
| Step 11 LPP Request Location Information | 23-39.5 | TLMFProc+ TAMF-LMF+ TAMFProc +TgNB-AMF + TgNBProc-NAS/LPP +TUE-gNB+ TUEProc-RRCDLInfo+ TUEProc-LPPLocationRe Processing delays: 19ms - UE: TUEProc-RRCDLInfo= 5ms TUEProc-LPPLocationRe = 5ms - gNB: TgNBProc-NAS/LPP= 3ms - AMF: TAMFProc= 3ms - LMF: TLMFProc= 3ms Signalling delay:4-20.5ms - UE-gNB: TUE-gNB= 0-0.5ms - gNB-AMF: TgNB-AMF= 3-10ms - AMF-LMF: TAMF-LMF= 1-10ms |
| Step 12 RRC Location Measurement Indication | 5-8.5 | TUEProc-RRCLocationMeas + TUE-gNB+ TgNBProc-RRC Processing delays: 5-8ms - UE: TUEProc-RRCLocationMeas = 2-5ms - gNB: TgNBProc-RRC= 3ms Signalling delay:0-0.5ms - UE-gNB: TUE-gNB= 0-0.5ms |
| Step 13 RRC Measurement Gap configuration | 13-13.5 | TgNBProc-RRC+ TUE-gNB+ TUEProc-RRCReconf Processing delays: 13ms - UE: TUEProc-RRCReconf = 10ms - gNB: TgNBProc-RRC= 3ms Signalling delay:0-0.5ms - UE-gNB: TUE-gNB= 0-0.5ms |
| Step 14 a DL PRS measurement | TUE-measc | RAN1 inputs |
| Step 14 b UL SRS measurement | TgNB-measc | RAN1 inputs |
| Step 15 LPP Provide Location Information | 20-39.5 | TLMFProc+ TAMF-LMF+ TAMFProc +TgNB-AMF + TgNBProc-NAS/LPP +TUE-gNB+ TUEProc-RRCULInfo+ TUEProc-LPPLocationRe Processing delays: 16-19ms - UE: TUEProc-RRCULInfo= 2-5ms TUEProc-LPPLocationRe= 5ms - gNB: TgNBProc-NAS/LPP= 3ms - AMF: TAMFProc= 3ms - LMF: TLMFProc= 3ms Signalling delay:4-20.5ms - UE-gNB: TUE-gNB= 0-0.5ms - gNB-AMF: TgNB-AMF= 3-10ms - AMF-LMF: TAMF-LMF= 1-10ms |
| Step 16 NRPPa MEASUREMENT RESPONSE | 13-29 | TLMFProc+ TAMF-LMF+ TAMFProc +TgNB-AMF + TgNBProc-NRPPa Processing delays: 9 ms - gNB: TgNBProc-NRPPa= 3ms - AMF: TAMFProc= 3ms - LMF: TLMFProc= 3ms Signalling delay:4-20ms - gNB-AMF: TgNB-AMF= 3-10ms - AMF-LMF: TAMF-LMF= 1-10ms |
| Step 17 LMF calculation | 2-30 | TLMF-Calc LMF calculation/estimation delay: 2-30 TLMF-Calc |
| Total values | 246-455 | Note 1: TUE-measc and TgNB-measc are not counted; Note 2: DL PRS related procedure may be performed in parallel with UL SRS related procedure, and therefore we may only count the delay caused by DL PRS related procedure, i.e. similar to the latency of DL-TDOA/DL-AoD |

Based on above table, we could see:

1 LPP capability exchange (step 1, 2): 33-88.5 ms

2 SRS configuration+activation (step 3-8): 66- 133ms

3 SRS measurement request (step 9): 13-29 ms

- Processing delays: 9 ms

  - gNB: TgNBProc-NRPPa= 3ms

  - AMF: TAMFProc= 3ms

  - LMF: TLMFProc= 3ms

- Signalling delay:4-20ms

  - gNB-AMF: TgNB-AMF= 3-10ms

  - AMF-LMF: TAMF-LMF= 1-10ms

4 LPP assistance data (step 11): 28-44.5 ms

5 LPP location request (step 12):23-39.5 ms

- Processing delays: 19ms

  - UE:

    - TUEProc-RRCDLInfo= 5ms

    - TUEProc-LPPLocationRe = 5ms

  - gNB: TgNBProc-NAS/LPP= 3ms

  - AMF: TAMFProc= 3ms

  - LMF: TLMFProc= 3ms

- Signalling delay:4-20.5ms

  - UE-gNB: TUE-gNB= 0-0.5ms

  - gNB-AMF: TgNB-AMF= 3-10ms

  - AMF-LMF: TAMF-LMF= 1-10ms

6 Measurement Gap (step 13-14): 18-22ms

7 LPP UE provide measurements (step 16):20-39.5ms

- Processing delays: 16-19ms

  - UE:

  - TUEProc-RRCULInfo= 2-5ms

  - TUEProc-LPPLocationRe= 5ms

  - gNB: TgNBProc-NAS/LPP= 3ms

  - AMF: TAMFProc= 3ms

  - LMF: TLMFProc= 3ms

- Signalling delay:4-20.5ms

  - UE-gNB: TUE-gNB= 0-0.5ms

  - gNB-AMF: TgNB-AMF= 3-10ms

  - AMF-LMF: TAMF-LMF= 1-10ms

8 gNB provide measurements (step 17): 13-29 ms

- Processing delays: 9 ms

  - gNB: TgNBProc-NRPPa= 3ms

  - AMF: TAMFProc= 3ms

  - LMF: TLMFProc= 3ms

- Signalling delay:4-20ms

  - gNB-AMF: TgNB-AMF= 3-10ms

  - AMF-LMF: TAMF-LMF= 1-10ms

9 LMF calculation: 2-30ms

Obviously, the total E2E latency cannot meet the Rel-17 requirement. To reduce the latency, we need to consider to reduce the procedure, and also skip nodes in order to reduce the processing delay/transmission delay.

The following solutions can be considered for latency reduction of NR positioning solutions from RAN2 perspective (some of them are also proposed in [2]):

            - Skip the capability procedure (can reduce the latency caused by 1 as above)

            - Pre-configuration and minimization of DL/UL transactions (can reduce the latency caused by 2, 4 as above)

              - DL PRS assistance information can be pre-configured to UE. Multiple DL PRS configurations can be associated with DL PRS configuration ID and activated when necessary

              - SRS for positioning configuration information can be pre-configured to UE. Multiple configurations of SRS for positioning can be associated with SRS for positioning configuration ID and activated when necessary

            - Measurement gaps (MG) optimizations (can reduce the latency caused by 6 as above)

              - MG-less operation - UE may operate w/o measurement gaps to process DL PRS

              - Support of semi-persistent a-periodic MGs, their pre-configuration and association with MG configuration ID

            - Reduce the number of hops between gNB, AMF and LMF, e.g. Local NR positioning in NG-RAN (To reduce the latency caused by the transmission/processing from AMF/LMF, i.e. only gNB is shown in the positioning);

Some of listed above enhancements (2, 3) have overlap in terms of scope with RAN WG1 and thus need to be jointly considered with RAN WG1. The updated procedure is shown as below:

| DL-TDOA | UL-TDOA | Multi-RTT |
| --- | --- | --- |

Figure 1: Low latency procedures for NR UE-assisted positioning (DL-TDOA, UL-TDOA, Multi-RTT)

In order to perform latency analysis for enhanced NR positioning solutions, we use the following assumptions:

- gNB is collocated with LMF, and SRS measurement is done in the gNB, and therefore SRS measurement (step 9/17) is not needed, and the processing delay/transmission delay related to AMF/LMF are not needed;

3 SRS measurement request (step 9): 13-29 ms

- Processing delays: 9 ms

  - gNB: TgNBProc-NRPPa= 3ms

  - AMF: TAMFProc= 3ms

  - LMF: TLMFProc= 3ms

- Signalling delay:4-20ms

  - gNB-AMF: TgNB-AMF= 3-10ms

  - AMF-LMF: TAMF-LMF= 1-10ms

5 LPP location request (step 12):13-13.5 ms

- Processing delays: 13ms

  - UE:

    - TUEProc-RRCDLInfo= 5ms

    - TUEProc-LPPLocationRe = 5ms

  - gNB: TgNBProc-NAS/LPP= 3ms

  - AMF: TAMFProc= 3ms

  - LMF: TLMFProc= 3ms

- Signalling delay:0-0.5ms

  - UE-gNB: TUE-gNB= 0-0.5ms

  - gNB-AMF: TgNB-AMF= 3-10ms

  - AMF-LMF: TAMF-LMF= 1-10ms

7 LPP UE provide measurements (step 16):10-13.5ms

- Processing delays: 10-13ms

  - UE:

    - TUEProc-RRCULInfo= 2-5ms

    - TUEProc-LPPLocationRe= 5ms

  - gNB: TgNBProc-NAS/LPP= 3ms

  - AMF: TAMFProc= 3ms

  - LMF: TLMFProc= 3ms

- Signalling delay:0-0.5ms

  - UE-gNB: TUE-gNB= 0-0.5ms

  - gNB-AMF: TgNB-AMF= 3-10ms

  - AMF-LMF: TAMF-LMF= 1-10ms

8 gNB provide measurements (step 17): 13-29 ms

- Processing delays: 9 ms

  - gNB: TgNBProc-NRPPa= 3ms

  - AMF: TAMFProc= 3ms

  - LMF: TLMFProc= 3ms

- Signalling delay:4-20ms

  - gNB-AMF: TgNB-AMF= 3-10ms

  - AMF-LMF: TAMF-LMF= 1-10ms

9 LMF calculation: 2-30ms

Then the minimum delay could be 25+ UE measurement time (UE measurement time is not counted).

Proposal: To reduce the latency, following enhancement directions are considered in WI phase:

- Skip the capability procedure (can reduce the latency caused by exchange of capability as above)

- Pre-configuration and minimization of DL/UL transactions (can reduce the latency caused by SRS/PRS configuration)

  - DL PRS assistance information can be pre-configured to UE. Multiple DL PRS configurations can be associated with DL PRS configuration ID and activated when necessary

  - SRS for positioning configuration information can be pre-configured to UE. Multiple configurations of SRS for positioning can be associated with SRS for positioning configuration ID and activated when necessary

- Measurement gaps (MG) optimizations (can reduce the latency caused by measurement gap request procedure)

  - MG-less operation - UE may operate w/o measurement gaps to process DL PRS

  - Support of semi-persistent a-periodic MGs, their pre-configuration and association with MG configuration ID

- Reduce the number of hops between gNB, AMF and LMF, e.g. Local NR positioning in NG-RAN (To reduce the latency caused by the transmission/processing from AMF/LMF, i.e. only gNB is shown in the positioning);

# Conclusion

Based on the discussion, we have following proposals:

Proposal: To reduce the latency, following enhancement directions are considered in WI phase:

- Skip the capability procedure (can reduce the latency caused by exchange of capability as above)

- Pre-configuration and minimization of DL/UL transactions (can reduce the latency caused by SRS/PRS configuration)

  - DL PRS assistance information can be pre-configured to UE. Multiple DL PRS configurations can be associated with DL PRS configuration ID and activated when necessary

  - SRS for positioning configuration information can be pre-configured to UE. Multiple configurations of SRS for positioning can be associated with SRS for positioning configuration ID and activated when necessary

- Measurement gaps (MG) optimizations (can reduce the latency caused by measurement gap request procedure)

  - MG-less operation - UE may operate w/o measurement gaps to process DL PRS

  - Support of semi-persistent a-periodic MGs, their pre-configuration and association with MG configuration ID

- Reduce the number of hops between gNB, AMF and LMF, e.g. Local NR positioning in NG-RAN (To reduce the latency caused by the transmission/processing from AMF/LMF, i.e. only gNB is shown in the positioning);

# Reference

[1] R2-2009001, [Post111-e][625][POS] End-to-end latency analysis (Intel)

[2] R1-2007945 Evaluation Results for NR Positioning Performance in I-IoT Scenarios, Intel