3GPP TSG-RAN WG2 Meeting #112 Electronic	R2-2009065

02 – 13 November 2020

Agenda item:	6.12

Source:	Nokia, Nokia Shanghai Bell

Title:	Considerations on parameter selection for shared cells

WID/SID:	NG_RAN_PRN-Core - Release 16

Document for:	Discussion and Decision

# 1	Introduction

At RAN2#111e, when the proposals of R2-2006853 were discussed (partially within emails discussion [AT111e][104][PRN] Stage 3 Corrections, see details in R2-2008184 and in R2-2008209), there was no conclusion on how to specify the selection of PNI-NPNs related parameter especially for the case when a cell is shared between a PLMN and PNI-NPNs of the PLMN.

This paper discusses the problems and proposes solutions.

# 2	Discussion

When a cell is shared between a PLMN and PNI-NPNs of the PLMN then:

- there are two occurrences of the PLMN ID of the PLMN in CellAccessRelatedInfo: (i) in the PLMN-IdentityInfoList; (ii) in the NPN-IdentityInfoList;

- there may be two set of UAC parameters: (i) one set is assigned to the case when the cell is used as cell of the PLMN, and (ii) another set when the cell is used as a cell of a PNI-NPN of the PLMN.

If it is left to UE implementation whether the parameters assigned to the selected PLMN or the parameters assigned to the PNI-NPNs of the selected PLMN are used, then the network cannot know which parameters the UE used. This may cause problems when the location of the UE is reported, as the UE may report a different cell identifier than the network. An example of such a case when UE inserts its serving cell identifier in IMS signalling when initiates an emergency session. Another problem is that the differentiation in access control between PNI-NPNs and PLMN does not work, as it is left for UE implementation which set of UAC parameters the UE uses. E.g., the network cannot effectively decrease the PNI-NPN traffic in shared cell, as a UE that selects the cell as PNI-NPN may use the UAC parameters of the PLMN.

Observation 1: If it is left for UE implementation whether the parameters assigned to the PLMN or PNI-NPN of the PLMN are used when a cell is shared between a PLMN and PNI-NPNs of the PLMN then there can be mismatch between the UE behaviour and assumption in gNB about the UE behaviour.

Proposal 1: RAN2 specifies how the UE selects between the parameters assigned to the PLMN or PNI-NPN are used when a cell is shared between a PLMN and the PNI-NPNs of that PLMN.

In the specification of initiation of UAC, 38.331 refers to the refers to "selected PNI-NPN". The selected CAG ID (or PNI-NPN) is not defined in automatic selection mode, as it was removed from TS 23.122 (see details in C1-203966, CR0548 " Removal of selected CAG-ID in automatic selection mode"). Therefore, referring the selected PNI-NPN means that AS will never follow that behaviour when automatic network selection is performed.

Observation 2: The selected PNI-NPN can only be used to cover the manual selection case, as selected PNI-NPN does not exist in automatic selection mode.

The following cases could be differentiated when the cell that is shared between the selected PLMN and the PNI-NPNs of the selected PLMN is selected by a UE:

- There is a manually selected CAG ID that the cell supports: in this case the UE can only select the cell as PNI-NPN cell.

- The UE is only allowed to access PNI-NPN cells of that PLMN and there is at least one CAG ID in the allowed CAG ID list that the cell supports: in this case the UE can only select the cell as PNI-NPN cell.

- The UE is allowed to access non PNI-NPN cells of that PLMN and there is at least one CAG ID in the allowed CAG ID list that the cell supports: in this case the UE can select the cell as PNI-NPN or as a PLMN cell.

- The UE is allowed to access non PNI-NPN cells of that PLMN and there is no CAG ID in the allowed CAG ID list that the cell supports: in this case the UE can only select the cell as a PLMN cell. (Note this case also covers the case when the UE does not support NPNs, as in this case the UE is allowed to access non PNI-NPN cells and the allowed CAG ID should be considered empty.)

In case 1) and 2) the UE can only select the cell as PNI-NPN cell. Therefore, the UE shall use the parameters assigned to the PNI-NPN(s) of the PLMN in AS level procedures.

In case of 3) the UE can select the cell as PNI-NPN or as a PLMN cell. However, if it is left for implementation which set of parameters are used, the problem that the UE reports about different cell identifier than gNB when the location of the UE is reported may cause some issues and it remains unspecified which set of UAC parameters are used. To prevent problems that the network cannot know the UE behaviour, it is suggested to specify that UE uses the PNI-NPN parameters also in this case.

As in case 4) the UE can only select the cell as PLMN cell. Therefore, the UE shall use the parameters assigned to the PNI-NPN(s) of the PLMN in AS level procedures. If this is not followed, then similar issues arise as in case 1) and 2).

Proposal 2: RAN2 specifies the following UE behaviour:

- When there is a manually selected CAG ID that the cell supports with the selected PLMN: parameters assigned to PNI-NPN(s) of that PLMN shall be used.

- When there is at least one CAG ID in the allowed CAG ID list that the cell supports with the selected PLMN: the parameters assigned to PNI-NPN(s) of the selected PLMN shall be used.

- When there is no CAG ID in the allowed CAG ID list with the selected PLMN and the UE is allowed to use non PNI-NPN cells of the selected PLMN: the parameters assigned to the selected PLMN shall be used.

The following AS level procedures have been identified where this may have impacts to the UE behaviour:

- Actions upon reception of the SIB1 (clause 5.2.2.4.2 of TS 38.331)

- Reception of the RRCSetup by the UE (clause 5.3.3.4 of TS 38.331)

- Reception of the RRCResume by the UE (clause 5.3.3.14 of TS 38.331)

- Initiation of Unified Access Control (clause 5.3.14.2 of TS 38.331)

The following subsections investigate those procedures considering the above observations and proposals.

## 2.1	Actions upon reception of the SIB1

In the "Actions upon reception of the SIB1" (clause 5.2.2.4.2 of TS 38.331) procedure AS layer selects which set of parameters (TAC, Cell ID etc. from PLMN-IdentityInfoList; or from NPN-IdentityInfoList) is forwarded to upper layers.

The current specification leaves unspecified the case when the selected PLMN can be found in the PLMN-IdentityInfoList and in the NPN-IdentityInfoList:

************ 5.2.2.4.2 of TS 38.331************

1>	if the cellAccessRelatedInfo contains an entry with the PLMN-Identity of the selected PLMN:

2>	in the remainder of the procedures use plmn-IdentityList, trackingAreaCode, and cellIdentity for the cell as received in the corresponding PLMN-IdentityInfo containing the selected PLMN;

1>	if the cellAccessRelatedInfo contains an entry of npn-IdentityInfoList with the NPN identity of the selected PLMN or SNPN:

2>	in the remainder of the procedures use npn-IdentityList, trackingAreaCode, and cellIdentity for the cell as received in the corresponding entry of npn-IdentityInfoList containing the selected PLMN or SNPN;

************************

Due to observation 1, this does not assure that UE will forward the correct set of parameters to upper layer, and thus modifications are needed:

Proposal 3: Modify the "Actions upon reception of the SIB1" (clause 5.2.2.4.2 of TS 38.331) procedure to remove any ambiguity on the parameters forwarded to forwarded to the upper layers based on Proposal 2.

Corresponding changes are presented in a companion contribution R2-2009066 where:

- When there is a manually selected CAG ID that the cell supports with the selected PLMN, the parameters assigned to the selected PLMN from the npn-IdentityInfoList shall be used.

- When there is at least one CAG ID in the allowed CAG ID list that the cell supports with the selected PLMN, the parameters assigned to the selected PLMN from the npn-IdentityInfoList shall be used.

- When the UE is allowed to access non PNI-NPN cells of that PLMN and there is no CAG ID in the allowed CAG ID list that the cell supports with the selected PLMN, the parameters assigned to the selected PLMN from the plmn-IdentityInfo shall be used.

## 2.2	Reception of the RRCSetup by the UE

In the "Reception of the RRCSetup by the UE" procedure the AS layer selects which network entry is referred in the RRCSetupComplete message. The current specification leaves this unspecified the case when the selected PLMN can be found in the PLMN-IdentityInfoList and in the NPN-IdentityInfoList:

************ 5.2.3.4 of TS 38.331************

2>	set the selectedPLMN-Identity to the PLMN or SNPN selected by upper layers (TS 24.501 [23]) from the PLMN(s) included in the plmn-IdentityList or the PLMN(s) or SNPN(s) included in the npn-IdentityInfoList in SIB1;

************************

Due to observation 1 this does not assure that UE will use the correct network reference, and thus modifications are needed:

Proposal 4: Modify the "Reception of the RRCSetup by the UE" (clause 5.3.3.4 of TS 38.331) procedure to remove any ambiguity on selectedPLMN-Identity based on Proposal 2.

Corresponding changes are presented in a companion contribution R2-2009066 where:

- When there is a manually selected CAG ID that the cell supports with the selected PLMN, the selectedPLMN-Identity shall refer to the PNI-NPN(s) of the selected PLMN from the npn-IdentityInfoList.

- When there is at least one CAG ID in the allowed CAG ID list that the cell supports with the selected PLMN, the selectedPLMN-Identity shall refer to the PNI-NPN(s) of the selected PLMN from the npn-IdentityInfoList.

- Otherwise (when the UE is allowed to access non PNI-NPN cells of that PLMN and there is no CAG ID in the allowed CAG ID list that the cell supports with the selected PLMN) the selectedPLMN-Identity shall refer to the selected PLMN from the plmn-IdentityInfo.

## 2.3	Reception of the RRCResume by the UE

In the "Reception of the RRCResume by the UE" (clause 5.3.3.14 of TS 38.331) procedure the AS layer selects which network entry is referred in the RRCResumeComplete message. The current specification leaves this unspecified the case when the selected PLMN can be found in the PLMN-IdentityInfoList and in the NPN-IdentityInfoList:

************ 5.2.3.14 of TS 38.331************

2>	if the upper layer provides a PLMN, set the selectedPLMN-Identity to PLMN selected by upper layers (TS 24.501 [23]) from the PLMN(s) included in the plmn-IdentityList or in the npn-IdentityInfoList in SIB1;

************************

Due to observation 1 this does not assure that UE will use the correct network reference, and thus modifications are needed. As CAG ID can only be selected manually at initial cell selection when a UE initially registers to a network, the manually selected CAG ID case cannot happen during RRC Resume procedure.

Proposal 5: Modify the "Reception of the RRCResume by the UE" (clause 5.3.3.14 of TS 38.331) procedure to remove any ambiguity on selectedPLMN-Identity based on Proposal 2.

Corresponding changes are presented in a companion contribution R2-2009066 where:

- When there is at least one CAG ID in the allowed CAG ID list that the cell supports with the selected PLMN, the selectedPLMN-Identity shall refer to the PNI-NPN(s) of the selected PLMN from the npn-IdentityInfoList.

- Otherwise (when the UE is allowed to access non PNI-NPN cells of that PLMN and there is no CAG ID in the allowed CAG ID list that the cell supports with the selected PLMN) the selectedPLMN-Identity shall refer to the selected PLMN from the plmn-IdentityInfo.

## 2.4	Initiation of Unified Access Control

In the "Initiation of Unified Access Control" (clause 5.3.14.2 of TS 38.331) procedure the AS layer selects which set of UAC parameters are used for UAC when there are network specific UAC parameters. The current specification clarifies that PNI-NPN specific parameters are used when a PNI-NPN is selected, but does not specify which set of UAC parameters are used when automatic network selection is performed and the selected PLMN can be found in the PLMN-IdentityInfoList and in the NPN-IdentityInfoList:

************ 5.2.14.2 of TS 38.331************

3>	if SIB1 includes uac-BarringPerPLMN-List and the uac-BarringPerPLMN-List contains an UAC-BarringPerPLMN entry with the plmn-IdentityIndex corresponding to the PLMN or SNPN selected by upper layers (see TS 24.501 [23]):

4>	if the upper layers selected a PNI-NPN (identified by a PLMN identity and a CAG-ID) and UAC-BarringPerPLMN has an entry with the plmn-IdentityIndex corresponding to the PNI-NPN selected by upper layers:

5>	select the UAC-BarringPerPLMN entry with the plmn-IdentityIndex corresponding to the PNI-NPN selected by upper layers;

4>	else:

54>	select the UAC-BarringPerPLMN entry with the plmn-IdentityIndex corresponding to the PLMN or to the SNPN selected by upper layers;

4>	in the remainder of this procedure, use the selected UAC-BarringPerPLMN entry (i.e. presence or absence of access barring parameters in this entry) irrespective of the uac-BarringForCommon included in SIB1;

************************

Due to observation 1 and 2 this does not assure that in case of automatic network selection case the UE will use the correct set of UAC parameters when different network specific parameters are provided for the PLMN and the PNI-NPNs for that PLMN, and thus modifications are needed:

Proposal 6: Modify the "Initiation of Unified Access Control" (clause 5.3.14.2 of TS 38.331) procedure to cover the automatic network selection case when PLMN or PNI-NPN specific UAC parameters are selected based on Proposal 2.

Corresponding changes are presented in a companion contribution R2-2009066 where:

- When there is a manually selected CAG ID that the cell supports and PNI-NPN specific UAC parameters are provided for PNI-NPN(s) of selected PLMN, the UE shall select the UAC parameters provided for the PNI-NPN(s) of the selected PLMN.

- When there is at least one CAG ID in the allowed CAG ID list that the cell supports and PNI-NPN specific UAC parameters are provided for PNI-NPN(s) of the selected PLMN, the UE shall select the UAC parameters provided for the PNI-NPN(s) of the selected PLMN.

- Otherwise (when the UE is allowed to access non PNI-NPN cells of the selected PLMN and there is no CAG ID in the allowed CAG ID list that the cell supports) if PLMN specific UAC parameters are provided for the selected PLMN, the UE shall select the UAC parameters provided for the selected PLMN.

# 3	Conclusions

This paper has the following observations and proposals:

Observation 1: If it is left for UE implementation whether the parameters assigned to the PLMN or PNI-NPN of the PLMN are used when a cell is shared between a PLMN and PNI-NPNs of the PLMN then there can be mismatch between the UE behaviour and assumption in gNB about the UE behaviour.

Proposal 1: RAN2 specifies how the UE selects between the parameters assigned to the PLMN or PNI-NPN are used when a cell is shared between a PLMN and the PNI-NPNs of that PLMN.

Observation 2: The selected PNI-NPN can only be used to cover the manual selection case, as selected PNI-NPN does not exist in automatic selection mode.

Proposal 2: RAN2 specifies the following UE behaviour:

- When there is a manually selected CAG ID that the cell supports with the selected PLMN: parameters assigned to PNI-NPN(s) of that PLMN shall be used.

- When there is at least one CAG ID in the allowed CAG ID list that the cell supports with the selected PLMN: the parameters assigned to PNI-NPN(s) of the selected PLMN shall be used.

- When there is no CAG ID in the allowed CAG ID list with the selected PLMN and the UE is allowed to use non PNI-NPN cells of the selected PLMN: the parameters assigned to the selected PLMN shall be used.

Proposal 3: Modify the "Actions upon reception of the SIB1" (clause 5.2.2.4.2 of TS 38.331) procedure to remove any ambiguity on the parameters forwarded to forwarded to the upper layers based on Proposal 2.

Proposal 4: Modify the "Reception of the RRCSetup by the UE" (clause 5.3.3.4 of TS 38.331) procedure to remove any ambiguity on selectedPLMN-Identity based on Proposal 2.

Proposal 5: Modify the "Reception of the RRCResume by the UE" (clause 5.3.3.14 of TS 38.331) procedure to remove any ambiguity on selectedPLMN-Identity based on Proposal 2.

Proposal 6: Modify the "Initiation of Unified Access Control" (clause 5.3.14.2 of TS 38.331) procedure to cover the automatic network selection case when PLMN or PNI-NPN specific UAC parameters are selected based on Proposal 2.

The corresponding changes of TS 38.331 can be found in R2-2009066.