3GPP TSG-RAN WG2 Meeting #112 Electronic	R2-2009066

# 02 – 13 November 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 2028 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for PNI-NPN related parameter selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RAN_PRN-Core |  |  |  |  |  | Date: |  |  | 2020-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There are unclear and incorrect procedure descriptions how the UE selects the PNI-NPNs related parameters: - In 5.4.2.4.2 (Actions upon reception of the SIB1): The current specification leaves unspecified which set of parameters (TAC, Cell ID etc.) is forwarded to upper layers when the selected PLMN ID is listed both in the plmn-IdentityList and in the npn-IdentityInfoList in SIB1. - In 5.3.3.4 (Reception of the RRCSetup by the UE) it is not clarified how PLMN index value is set in the RRCsetupComplete message when the selected PLMN ID is listed both in the plmn-IdentityList and in the npn-IdentityInfoList in SIB1. - In 5.3.13.4 (Reception of the RRCResume by the UE) it is not clarified how PLMN index value is set in the RRCResumeComplete message when the selected PLMN ID is listed both in the plmn-IdentityList and in the npn-IdentityInfoList in SIB1. - In 5.3.14.2 (Initiation of UAC) the selection of the UAC parameters per PLMN is not specified when automatic network selection has been performed and the selected PLMN ID is listed both in the plmn-IdentityList and in the npn-IdentityInfoList in SIB1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In 5.2.2.4.2 it is clarified how the UE selects the relevant cell access information: - When there is a selected CAG ID that the cell supports, the parameters assigned to the selected PLMN from the npn-IdentityInfoList shall be used. - When there is at least one CAG ID in the allowed CAG ID list that the cell supports with the selected PLMN, the parameters assigned to the selected PLMN from the npn-IdentityInfoList shall be used. - When the UE is allowed to access non PNI-NPN cells of the selected PLMN and there is no CAG ID in the allowed CAG ID list that the cell supports, the parameters assigned to the selected PLMN from the plmn-IdentityInfo shall be used. - In 5.3.3.4 it is clarified how selectedPLMN-Identity is set in the RRCSetupComplete message: - When there is a selected CAG ID or there is at least one CAG ID in the allowed CAG ID list that the cell supports with the selected PLMN, the selectedPLMN-Identity shall refer to the PNI-NPN(s) of the selected PLMN from the npn-IdentityInfoList. - Otherwise, the selectedPLMN-Identity shall refer to the selected PLMN from the plmn-IdentityInfo. - In 5.3.13.4 it is clarified how selectedPLMN-Identity is set in the RRCResumeComplete message: - When there is at least one CAG ID in the allowed CAG ID list that the cell supports with the selected PLMN, the selectedPLMN-Identity shall refer to the PNI-NPN(s) of the selected PLMN from the npn-IdentityInfoList. - Otherwise, the selectedPLMN-Identity shall refer to the selected PLMN from the plmn-IdentityInfo. - In 5.3.14.2 it is clarified how the PLMN and NPN-NPN specific UAC parameters are selected: - When there is a selected CAG ID that the cell supports or there is at least one CAG ID in the allowed CAG ID list that the cell supports with the selected PLMN, then the UE shall select the UAC parameters provided for the PNI-NPN(s) of the selected PLMN (if available). - Otherwise, the UE shall select the UAC parameters provided for the selected PLMN (if available). Impact analysis Impacted functionality: PNI-NPN Inter-operability: As this CR has no network impact, implementation of this CR by a UE will not cause compatibility issues |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is ambiguous how the UE selects the used network parameters for PNI-NPNs in shared cells, and thus the UE may not follow the desired behaviour (e.g. not using the appropriate UAC parameters) |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.4.2, 5.3.3.4, 5.3.13.4, 5.3.14.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Modified Subclause

##### 5.2.2.4.2	Actions upon reception of the SIB1

Upon receiving the SIB1 the UE shall:

1>	store the acquired SIB1;

1>	if the upper layers selected a PNI-NPN (identified by a PLMN identity and a CAG-ID) and the cellAccessRelatedInfo contains an entry for that PNI-NPN:

2>	in the remainder of the procedures use npn-IdentityList, trackingAreaCode, and cellIdentity from that entry for the cell;

1>	else if at least one of the CAG-IDs of the Allowed CAG list provided by upper layers is listed with the selected PLMN in the npn-IdentityInfoList:

2>	in the remainder of the procedures use npn-IdentityList, trackingAreaCode, and cellIdentity from that entry for the cell;

1>	else if the cellAccessRelatedInfo contains an entry with the PLMN-Identity of the selected PLMN and the CAG-only indication is not provided by NAS:

2>	in the remainder of the procedures use plmn-IdentityList, trackingAreaCode, and cellIdentity from that entry for the cell;

1>	if the cellAccessRelatedInfo contains an entry of npn-IdentityInfoList with the NPN identity of the selected SNPN:

2>	in the remainder of the procedures use npn-IdentityList, trackingAreaCode, and cellIdentity from that entry for the cell;

- if in RRC_CONNECTED while T311 is not running:

<********** OMITTED TEXT ************>

Next Modified Subclause

#### 5.3.3.4	Reception of the RRCSetup by the UE

<********** OMITTED TEXT ************>

1>	set the content of RRCSetupComplete message as follows:

2>	if upper layers provide a 5G-S-TMSI:

3>	if the RRCSetup is received in response to an RRCSetupRequest:

4>	set the ng-5G-S-TMSI-Value to ng-5G-S-TMSI-Part2;

3>	else:

4>	set the ng-5G-S-TMSI-Value to ng-5G-S-TMSI;

2>	if the upper layers selected a PNI-NPN (identified by a PLMN identity and a CAG-ID) or at least one of the CAG-IDs of the Allowed CAG list provided by upper layers is listed with the selected PLMN in the npn-IdentityInfoList:

3> set the selectedPLMN-Identity to the PNI-NPN(s) of the PLMN selected by upper layers (TS 24.501 [23]) from the npn-IdentityInfoList included in SIB1;

2>	else:

3>	set the selectedPLMN-Identity to the PLMN or SNPN selected by upper layers (TS 24.501 [23]) from the PLMN(s) included in the plmn-IdentityList or the PLMN(s) or SNPN(s) included in the npn-IdentityInfoList in SIB1;

2>	if upper layers provide the 'Registered AMF':

<********** OMITTED TEXT ************>

Next Modified Subclause

#### 5.3.13.4	Reception of the RRCResume by the UE

The UE shall:

<********** OMITTED TEXT ************>

1>	set the content of the of RRCResumeComplete message as follows:

2>	if the upper layer provides NAS PDU, set the dedicatedNAS-Message to include the information received from upper layers;

2>	if the upper layer provides a PLMN:

3>	if at least one of the CAG-IDs of the Allowed CAG list provided by upper layers is listed with the selected PLMN in the npn-IdentityInfoList:

4>	set the selectedPLMN-Identity to the PNI-NPN(s) of the PLMN selected by upper layers (TS 24.501 [23]) from the npn-IdentityInfoList included in SIB1;

3>	else:

4> set the selectedPLMN-Identity to PLMN selected by upper layers (TS 24.501 [23]) from the PLMN(s) included in the plmn-IdentityList or in the npn-IdentityInfoList in SIB1;

2>	if the masterCellGroup contains the reportUplinkTxDirectCurrent:

<********** OMITTED TEXT ************>

Next Modified Subclause

#### 5.3.14.2	Initiation

Upon initiation of the procedure, the UE shall:

1>	if timer T390 is running for the Access Category:

2>	consider the access attempt as barred;

1>	else if timer T302 is running and the Access Category is neither '2' nor '0':

2>	consider the access attempt as barred;

1>	else:

2>	if the Access Category is '0':

3>	consider the access attempt as allowed;

2>	else:

3>	if the upper layers selected a PNI-NPN (identified by a PLMN identity and a CAG-ID) at least one of the CAG-IDs of the Allowed CAG list provided by upper layers is listed with the selected PLMN in the npn-IdentityInfoList:

4>	if SIB1 includes uac-BarringPerPLMN-List and UAC-BarringPerPLMN has an entry with the plmn-IdentityIndex corresponding to the PNI-NPN selected by upper layers:

5>	select the UAC-BarringPerPLMN entry with the plmn-IdentityIndex corresponding to the PNI-NPN(s) of the PLMN selected by upper layers;

3>	else:

4> if SIB1 includes uac-BarringPerPLMN-List and the uac-BarringPerPLMN-List contains an UAC-BarringPerPLMN entry with the plmn-IdentityIndex corresponding to the PLMN or SNPN selected by upper layers (see TS 24.501 [23]):

5>	select the UAC-BarringPerPLMN entry with the plmn-IdentityIndex corresponding to the PLMN or to the SNPN selected by upper layers;

3>	if a UAC-BarringPerPLMN entry is selected:

4>	in the remainder of this procedure, use the selected UAC-BarringPerPLMN entry (i.e. presence or absence of access barring parameters in this entry) irrespective of the uac-BarringForCommon included in SIB1;

3>	else if SIB1 includes uac-BarringForCommon:

<********** OMITTED TEXT ************>

End of Changres