3GPP TSG-RAN WG2 Meeting #112-e							              R2-2009086

Electronic, 2nd – 13th Nov. 2020

Source: 	vivo, Guangdong Genius

Title:  	Identification and Access Restrictions for RedCap UEs

Agenda Item:	8.12.2.2

Document for:	Discussion and Decision

Introduction

In 3GPP TSG RAN #86, a new SID on support of reduced capability NR devices (RedCap) was revised and agreed in [1]. One of the objectives in the SID is following:

Study functionality that will allow devices with reduced capabilities to be explicitly identifiable to networks and network operators, and allow operators to restrict their access, if desired [RAN2, RAN1].

In RAN2#111-e meeting, the following agreements have been reached for UE identification and access restrictions:

Agreements:

- An indication in system information is needed to indicate whether a REDCAP UE can camp on the cell. FFS whether the indication is explicit or implicit.

- UAC mechanism also apply to REDCAP UEs.

- System information indicates whether REDCAP operation is allowed/barred on a frequency. FFS reuse the legacy intraFreqReselection or introduce separate flag

- Further discuss enhancement of UAC for REDCAP UEs, including e.g.:

a. define new Access Identity for REDCAP UEs

b. define new Access Categories for REDCAP UEs

(for any final decision we need to check with SA1 and/or CT1)

After the meeting, an email discussion was assigned to discuss the UE identification and access restrictions [2]. In this paper, we will make some further analysis on enhancement of UAC for RedCap UEs, and share our views on early indication and SI enhancement for RedCap UEs.

Discussion

2.1 Enhancement of UAC for RedCap UEs

2.1.1 Groups of RedCap Devices for Access Restriction

In the SID of RedCap, three use cases are elaborated. The requirements for each use case are summarized in the following table.

Table I: RedCap use cases and requirements

| Use cases | reference bit rate | end-to-end latency | reliability /availability | peak bit rate | Battery |
| --- | --- | --- | --- | --- | --- |
| Industrial sensor | <2Mbps (UL heavy) | <100ms; 5-10ms for safety related sensors | Availability:99.99% | N/A | few years |
| Video surveillance | 2-4 Mbps for economic video; 7.5-25 Mbps for High-end video | < 500 ms | Reliability: 99%-99.9%. | N/A | N/A |
| Wearable | 5-50 Mbps in DL and 2-5 Mbps in UL | N/A | N/A | Up to 150 Mbps for DL and up to 50 Mbps for UL | Multiple days (up to 1-2 weeks) |

In general, it is preferred to define less UE types considering the economics of scale and to avoid market fragmentation. However, according to the above description, it can be noticed that even for RedCap devices the range of bitrate for different devices may vary greatly, e.g. from less than 2 Mbps to up to 150 Mbps. In [3], we discussed the UE type or category based on the requirements(e.g. maximum bitrate supported) for RedCap UEs. We have the proposal that: two UE types should be defined for RedCap devices to cover various use cases: high-end(i.e. high bitrate) and low-end(i.e. low bitrate) devices. As different bitrate implies different requirements on the amount of radio resource, hence the UE type should be considered while performing the access control.

It’s worth noting that the services provided by the low-end RedCap devices are NOT always low priority services. For example, a wearable eHealth related device needs to transfer collected medical data of the user to the hospital when acute disease occurs. Another example is some smart watches can detect that the user has fallen. When an incident like this occurs, a hard fall alert is delivered, and the user or the watch itself may initiate a call for emergency services.

Besides, there are of course other low-end RedCap devices only used for delay tolerant service, such as a thermometers senor in a fish tank. Hence:

Observation 1: Low-end RedCap devices can be further divided into 2 use cases based on the provided service considering different access requirements:

- Low-end wearable devices: devices may be used for important or delay sensitive service, e.g. transfer collected medical data with a wearable eHealth related device when acute disease occurs;

- Low-end industry devices: devices only used for delay tolerant service, e.g. a thermometers senor in a fish tank.

According to the above analysis, the access initiated by the RedCap devices has quite different requirements on the amount of radio resource, success probability and access latency etc. To simply the access control mechanism, it is reasonable to group the RedCap devices which have the similar requirements for access control. According to this guideline, the access restriction for low-end RedCap should not only based on the UE type, but also the use case (e.g. wearable/industrial sensor). In the way, the RedCap devices can be grouped according to both UE type and use cases (e.g. wearable/industry sensor). For example, 3 groups of RedCap devices, which require different access control treatments, can be defined:

- High-end RedCap devices: the RedCap devices with higher data rate requirement, e.g. high-end wearables and high-end video surveillance;

- Low-end wearable devices: the RedCap devices with lower data rate requirement, and could be used for important or delay sensitive service, e.g. a wearable eHealth related device;

- Low-end industry devices: the RedCap devices with lower data rate requirement, and mainly used for delay tolerant service, e.g. a thermometers senor in a fish tank.

Proposal 1: UAC should be able to treat access initiated by the following 3 groups of RedCap devices differently: High-end Redcap devices, Low-end wearable devices and Low-end industrial sensors.

2.1.2 Access Control solution for RedCap UEs

In Rel-15, Unified Access Control (UAC) mechanism is developed for NR. UAC is based on the use of Access Identity and Access Category. The Access Identity is determined by the user’s subscriber and the Access Category is determined by the type of service initiated. When initiating a service request, the UE NAS provides one or more Access Identities and one Access Category to UE RRC. If a cell intends to partially or fully bar the access request triggered by certain Access Category, the cell can broadcast UAC-BarringInfoSet for the Access Category.

For each Access Identity, the uac-BarringForAccessIdentity of UAC-BarringInfoSet indicates whether the access request is 100% allowed. For the Access Identities which access requests are not 100% allowed, UAC-BarringInfoSet further indicates the rate of barring and barring timer which can be used for backoff when the access is barred.

Observation 2: Unified Access Control mechanism, which is introduced in Rel-15 for NR, can perform access control based on Access Identity (determined by user subscriber) and Access Category (determined by type of service).

The RedCap devices and normal (i.e. non-RedCap) UE may be treated differently in some aspects, such as charging policy and QoS, hence it is very likely that the operator would like to differentiate RedCap devices from normal UE via user subscriber. Given Access Identity is associated with user subscriber, it is reasonable to use new Access Identities for RedCap devices. What’s more, it is better to allocated different Access Identities for RedCap devices belonging to different groups to differentiate the above 3 groups of RedCap devices.

Using new Access Identities for RedCap devices enables UAC to bar RedCap devices. Sometimes, the congestion is not very serious, one cell may only want to bar portion (e.g. 20%) of access requests from the low-end industry RedCap devices, to reduce the load of the network. According to the UAC mechanism, the cell can indicate UAC-BarringInfoSet only for the Access Identity allocated to low-end industry RedCap device to achieve this.

Proposal 2: Different Access Identities can be used in UAC for High-end, Low-end wearable and Low-end industry RedCap devices to enable applying different access control strategies on RedCap devices belonging to different groups.

In some other cases, due to the congestion is serious or operator strategy, the network wants to bar portion of the access requests triggered by a certain service, e.g. MO SMS, with different probability for different devices groups, e.g. bar 100% requests from the Low-end industry RedCap devices, 80% requests from the low-end wearable RedCap devices and 20% requests from high-end RedCap devices.

According to the UAC mechanism, a common Access Category specific barring probability is applied to all the Access Identities if the access restriction is applied to the Access Identity. Hence, the above requirement can only be achieved by allocating different Access Categories for RedCap devices belonging to different groups.

Proposal 3: RAN2 to discuss whether there is a requirement to apply different access probability among different groups of RedCap devices when the accesses are triggered by the same type of service.

Proposal 4: If the answer to Proposal 3 is yes, different Access Categories can be used in UAC to differentiate accesses from high-end RedCap devices, low-end wearable RedCap devices and low-end industry RedCap devices in case the access is triggered by the same type of service.

2.2 Early indication of RedCap devices

Early indication solutions during RACH procedure for UE identification was also discussed in RAN1#102-e, the following agreements were made [4]:

Agreements: - Further study the options for identification of RedCap UEs, including at least the following indication methods: - Opt. 1: During Msg1 transmission, e.g., via separate initial UL BWP, separate PRACH resource, or PRACH preamble partitioning. - Opt. 2: During Msg3 transmission. - Opt. 3: Post Msg4 acknowledgment. - E.g., during Msg5 transmission or part of UE capability reporting. - Opt. 4: During MsgA transmission (subject to support of if 2-step RACH) - Other options are not precluded. - Note: This study intends to establish feasibility of, and pros and cons for the identified options from RAN1 perspective, without any intention of down-selection without guidance from RAN2. Conclusion: - RAN1 to wait for further progress in RAN2 on the issues of temporary access barring and congestion control.

In our understanding, we need to first agree the intended use case or motivation for this early indication considering RAN1 inputs and RAN2 requirements before making any decision. Given it is being discussion in RAN1 and there is no consensus on RAN2 requirement for early indication, we cannot see a strong need for RAN to identify RedCap UEs in MSG1/3/5/A by now. Thus, we can also wait RAN1 for more progress on this.

Proposal 5: Early indication of RedCap devices in MSG1/3/5/A can be considered later if clear use cases or motivation are identified.

2.3 System information enhancement for RedCap devices

The following was agreed on System information enhancement for RedCap devices in RAN2#111e meering:

1. An indication in system information is needed to indicate whether a REDCAP UE can camp on the cell. FFS whether the indication is explicit or implicit.

3. System information indicates whether REDCAP operation is allowed/barred on a frequency. FFS reuse the legacy intraFreqReselection or introduce separate flag

In our understanding, the FFSs are related to:

- some other design of system information such as new parameters defined for UAC which may enable the implicit indication; and

- the requirements from RAN1, e.g. whether the initial BWP for RedCap UEs is different from non-RedCap UEs.

Hence, it is too early to handle these details in study item phase. During the email discussion [2], majority companies also share the same view.

Proposal 6: The details of camping and intraFreqReselection indicator for RedCap UEs are discussed in the WI phase.

Conclusions

In this paper, we discussed the characters of different types of RedCap UEs and analysed how to introduce the access restriction for RedCap UEs based on the UAC mechanism developed for NR. And we also shared our views on early indication and SI enhancement for RedCap UEs. According to the discussion, we have the following proposals:

Proposal 1: UAC should be able to treat access initiated by the following 3 groups of RedCap devices differently: High-end Redcap devices, Low-end wearable devices and Low-end industrial sensors.

Proposal 2: Different Access Identities can be used in UAC for High-end, Low-end wearable and Low-end industry RedCap devices to enable applying different access control strategies on RedCap devices belonging to different groups.

Proposal 3: RAN2 to discuss whether there is a requirement to apply different access probability among different groups of RedCap devices when the accesses are triggered by the same type of service.

Proposal 4: If the answer to Proposal 3 is yes, different Access Categories can be used in UAC to differentiate accesses from high-end RedCap devices, low-end wearable RedCap devices and low-end industry RedCap devices in case the access is triggered by the same type of service.

Proposal 5: Early indication of RedCap devices in MSG1/3/5/A can be considered later if clear use cases or motivation are identified.

Proposal 6: The details of camping and intraFreqReselection indicator for RedCap UEs are discussed in the WI phase.

Reference

- RP-201386, Revised SID on Study on support of reduced capability NR devices, Ericsson

- R2-2009936, Summary of email discussion 914 on UE identification and access restriction, Huawei

- R2-2009085, UE type definition and constraining for RedCap UEs, vivo

- RAN1 minutes for RAN1#102-e