3GPP TSG-RAN WG2 Meeting #112-e	R2-2009137

# Online, Nov 2nd – 13th, 2020                                   revised R2-2007049

Agenda Item:	8.11.2

Source:	Spreadtrum Communications

Title:	Discussion on positioning enhancements for commercial use cases

Document for:	Discussion and Decision

Introduction

In the SID, one of the objective is to study enhancements and solutions of Rel-16 positioning techniques. The details are as follows:

- Study enhancements and solutions necessary to support the high accuracy (horizontal and vertical), low latency, network efficiency (scalability, RS overhead, etc.), and device efficiency (power consumption, complexity, etc.) requirements for commercial uses cases (incl. general commercial use cases and specifically (I)IoT use cases as exemplified in section 3 above (Justification)):

  - Define additional scenarios (e.g. (I)IoT) based on TR 38.901 to evaluate the performance for the use cases (e.g. (I)IoT). [RAN1]

  - Evaluate the achievable positioning accuracy and latency with the Rel-16 positioning solutions in (I)IoT scenarios and identify any performance gaps. [RAN1]

  - Identify and evaluate positioning techniques, DL/UL positioning reference signals, signalling and procedures for improved accuracy, reduced latency, network efficiency, and device efficiency.
Enhancements to Rel-16 positioning techniques, if they meet the requirements, will be prioritized, and new techniques will not be considered in this case. [RAN1, RAN2]

NOTE 1:	Sidelink is not part of this objective.

NOTE 2:	Involve RAN4 for validating assumptions for the systems evaluations where appropriate.

NOTE 3:	The commercial use cases and requirements are applicable to a limited geographic area.

In this contribution, we discuss the positioning enhancements of Rel-16 positioning techniques about RRC IDLE/INACTIVE positioning and express our opinions.

Discussion

In Rel-16, only RRC_CONNECTED positioning is supported. For example, when a UE is periodically localized, it needs to keep in connected state even when it has no other services, or transfer into connected state with the periodicity of the positioning service. In some scenarios such as factory or shopping mall, it is important to enabling poisoning service for UEs in RRC_IDLE/INACTIVE states, as those UEs may be mainly used for positioning and have limited date service.

In addition, power consumption is a foremost parameter since positioning devices are expected to operate for a long time, especially for (I)IoT devices. Nowadays, in [1], the following positioning use cases are defined. Among all the use cases, the use cases requiring low energy or power saving mode include AR, wearables, waste management and UAV. For all these four use cases, it is clear that the devices need to report its position periodically to the network. Take wearable devices as an example, in order to obtain minor or elderly people’s position, wearable devices, such as smart watches, can provide this services, such as tracking, activity monitoring and emergency messages.

Table 1 – Use cases synthesis

| Use cases |  | Potential requirements per use cases |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Environment of Use | Position Accuracy | Velocity | Avail. | Update rate or interval | TTFF | Latency | Other KPI |
| 5.2.1 | Bike sharing | 5G positioning service area - Outdoor | 2m Horizontal |  | 90 % |  | 10s | 1s |  |
|  |  | Enhanced positioning area - Outdoor | 0.2m Horizontal |  | 99 % |  | 10s | 1s |  |
| 5.2.2 | Augmented Reality | Outdoor - 5G positioning service area | 1-3m Horizontal 0.1-3m Vertical | 2 m/s 10deg. | 80 % | 1 - 10 Hz | 10s | 1s | Low Energy |
| 5.2.3 | Wearables | 5G positioning service area - -Outdoor/Indoor | 2m Horizontal 1-3m Vertical |  | 90 % | 30s - 300s | 10s |  | Power saving mode |
|  |  | 5G positioning service area - -Outdoor/Indoor | 2m Horizontal 1-3m Vertical |  | 99 % | 1s - 30s | 10s | 1s | Normal mode |
| 5.2.4 | Advertisement push | 5G positioning service area - -Outdoor/Indoor | 3m Horizontal 3m Vertical |  | 90 % |  |  | 60s |  |
| 5.2.5 | Flow management | Enhanced positioning- Outdoor/Indoor | 10m Horizontal |  | 80 % | 10s | 10s |  |  |
| 5.3.1 | Person and medical equipment location in Hospital | Enhanced positioning- Outdoor/Indoor | 3m Horizontal 2m Vertical |  | 99 % |  |  | 60s |  |
| 5.3.2 | Patient location (outside Hospital) | 5G positioning service area Outdoor/Indoor | 10m Horizontal 3m Vertical (floor) |  | 99 % |  |  |  |  |
| 5.3.3 | Trolley | Enhanced positioning- Outdoor/Indoor | 0.5m Horizontal 1-3m Vertical |  | 99 % |  |  | 20ms |  |
| 5.3.4 | Waste management | 5G positioning service area - Outdoor | 3m Horizontal |  | 99 % | 2h - 1 day |  | 60s | Very low energy (15 years) |
| 5.4.1 | Emergency call | 5G positioning service area Outdoor/Indoor | 50m Horizontal 3m Vertical |  | 95 % |  | 30s | 60s | Reliability/ Confidence |
| 5.4.2 | Accurate Positioning for First Responders | Outdoor | 1m Horizontal, 0.3 m Vertical |  | 98 % |  | 10s | 5s | MCX Confidence Event-triggered report |
|  |  | Indoor | 1m Horizontal, 2 m Vertical |  | 95 % |  | 10s | 1s |  |
| 5.4.3 | Alerting nearby emergency responders | 5G positioning service area Outdoor/Indoor | 50m Horizontal 3m Vertical (floor) |  | 99% |  | 10s |  | Privacy, scalability, cross operator |
| 5.4.4 | Emergency equipment loc. outside hospitals | 5G positioning service area Outdoor/Indoor | 10m Horizontal 3m Vertical (floor) |  | 95% |  | 10s |  | Extended sleep periods |
| 5.5.1 | Traffic Monitoring & Control | 5G positioning service area - Outdoor | 1-3m Horizontal 2.5m Vertical |  | 95 % | 10 Hz | 10s | 30ms | Antispoofing Antitampering |
| 5.5.2 | Road User Charging | 5G positioning service area - Outdoor Enhanced positioning-Tunnels | <1m (across track) 3m (along track) | 2 m/s | 99 % | 1 Hz | 10s |  | Antispoofing Antitampering |
| 5.6.1 | Asset tracking and management | 5G positioning service area - Outdoor | 10-30m Horizontal | 5 m/s | 99 % | 300s-1day |  |  | 20 mJ/fix (average), Antispoofing, Antitampering, support for "out of coverage" positioning |
|  |  | Enhanced positioning - Outdoor | 1m Horizontal |  | 99 % | 1s | 1s in enhanced positioning area |  |  |
| 5.7.1 | UAV (Data analysis) | 5G positioning service area - Outdoor | 0.1m Horizontal 0.1m Vertical | 0.5 m/s 2 deg. | 99 % |  | 10s |  | Low Energy, Antispoofing, Antitampering |
| 5.7.2 | UAV (Remote control) | 5G positioning service area - Outdoor | 0.5m Horizontal 0.3m Vertical |  | 99 % |  |  | 150ms | Antispoofing Antitampering |
|  |  | Enhanced positioning area - Outdoor | 0.5m Horizontal 0.1m Vertical |  | 99.9 % |  |  | 150ms | Antispoofing Antitampering |
| 5.8.1 | Support multiple different location service | 5G positioning service area - Outdoor | 2m Horizontal |  | 90 % |  | 10s | 1s | Management of different KPI and positioning services |
|  |  | Enhanced positioning area - Indoor | 0.1m Horizontal |  | 99 % |  | 10s | 1s |  |
| 5.8.2 | Support location method negotiation | 5G positioning service area Outdoor/Indoor |  |  |  |  |  |  | Support + negotiation of positioning methods (incl. hybrid) |
| Note: most use cases also feature potential requirements on modes of operation, intended for the UE, the Network or for the 5G system. |  |  |  |  |  |  |  |  |  |

Without supporting idle/Inactive mode positioning, UE needs to switch between connected and idle states frequently, which leads to lots of signalling overhead. If positioning can be performed without entering the connected mode, both the UE power consumption and the signalling overhead could be reduced.

Observation 1: Positioning for UEs in RRC IDLE/INACTIVE state can improve device efficiency and reduce signaling overhead of state switch.

In order to support positioning a RRC IDLE/INACTIVE devices, there are two ways: 1) UE-based positioning method; and 2) UE-assisted positioning method with UE measurement result reporting.

For UE-based positioning, The UE does not need to report measurement result to network, instead calculates the positioning result by himself. Take DL-TDoA as an example, upon the reception of assistance data including PRS configuration, UE can start to measure the DL reference signals (e.g. PRS) from the serving and neighbouring cells to obtain the RSTD. And then UE can calculate the positioning result by itself. Moreover, in Rel-16., it has been support broadcasting assistance data. Thus, this method has weedy change to the spec, and complexity is also very low. In addition, if location request is initiated by the UE – i.e. mobile originated location request (MO-LR) [1], it is usually not needed to report location result to the network.

If location request is initiated by AMF or location client – i.e. Mobile Terminated Location Request (MT-LR) and Network Induced Location Request (NI-LR) [2], the location results should be report to the network.

For UE-assisted positioning method, UE needs to report its measurement result of the DL reference signals to calculate positioning results in RRC IDLE/INACTIVE state.

Observation 2：For positioning of RRC IDLE/INACTIVE UE, only when location request is Mo-LR and the positioning method is UE-based, neither measurement result nor report location result is needed to be reported to the network.

From the above analysis, we know that it is necessary to transmit measurement results or location results to LMF in RRC IDLE/INACTIVE state. At present, Rel-17 small data topic is studying how to support small data transmission in RRC INACTIVE state, which can be used as the baseline for positioning data report. However, small data topic is studying and there is no mature solutions for data transmission. So RRC inactive positioning cannot use the small data transmission solutions as the baseline.

For RRC IDLE UE, the early data transmission (EDT) procedure of NB-IoT can be used as the baseline. However, EDT procedure is not supported in NR. If we want to extend EDT to NR, there is amount of work. And we do not have much time to do this.

Proposal 1: Only when location request is MO-LR and the positioning method is UE-based, positioning for UEs in RRC IDLE/INACTIVE state is supported in R17.

Conclusions

In this contribution, we discuss the positioning enhancements of Rel-16 positioning techniques. Proposals are given as follows:

Observation 1: RRC IDLE/INACTIVE positioning can improve device efficiency and signaling overhead

Observation 2：For positioning of RRC IDLE/INACTIVE UE, only when location request is Mo-LR and the positioning method is UE-based, neither measurement result nor report location result is needed to be reported to the network.

Proposal 1: Only when location request is MO-LR and the positioning method is UE-based, positioning for UEs in RRC IDLE/INACTIVE state is supported in R17.

References

- TR 22.872 “Study on positioning use cases” v16.1.0, 2019-09

- TS 23.273 “5G system (5GS) Location Services (LCS)” v16.4.0, 2020-07