3GPP TSG-RAN WG2 #112e	R2-2009191

eMeeting, 02-13 November 2020

Agenda item:		8.6.4

Source:	ZTE Corporation, Sanechips

Title:	Consideration on the RACH based SDT

Document for:	Discussion and Decision

# Introduction

The intention of this contribution is to discuss the details of the RACH based solution for SDT (RA-SDT).

# Stage2 procedure for RACH based SDT

In the previous meeting, the following agreements have been made for SDT

- Small data transmission with RRC message is supported as baseline for RA-based and CG based schemes

- The 2-step RACH or 4-step RACH should be applied to RACH based uplink small data transmission in RRC_INACTIVE

Based on the two agreements above, the figure below illustrates the stage 2 procedure for RACH based SDT.

Figure x: Stage 2 procedure for RACH based SDT

Based on the procedure shown above, in case the UE determines to initiate RACH based SDT, the following steps will be performed:

- Step 1: Once RACH procedure is initiated, the UE should transmit RRC message through MSG3 (4-step RACH) or MSGA (2-step RACH). In addition to the RRC message, MAC CE (e.g. BSR) and DRB data packet can be included in MSG3/MSGA as well

- Step 2: Once the MSG3/MSGA is transmitted, the UE will monitor RA-RNTI/MSGB-RNTI for MSG4/MSGB, in which the contention resolution ID will be carried.

- Step 3: Once RACH procedure is successfully completed, the UE should monitor C-RNTI for the subsequent UL/DL transmission.

- Step 4: Once the RRC release message is received, the UE will stop the monitoring the C-RNTI and enter normal INACTIVE state.

Based on the steps above, we propose to adopt the baseline of RACH based SDT procedure as follow:

Proposal 1: The following procedure shall be taken as baseline for the RACH based SDT.

- Step 1: Once RA-SDT is initiated, the UE should transmit RRC message through MSG3 (4-step RACH) or MSGA (2-step RACH). In addition to the RRC message, MAC CEs (e.g. BSR) and DRB data packet can be included in MSG3/MSGA as well

- Step 2: Once the MSG3/MSGA is transmitted, the UE will monitor RA-RNTI/MSGB-RNTI for MSG2/MSGB.

- Step 3: Once RACH procedure is successfully completed, the UE should monitor C-RNTI for subsequent UL/DL transmission (details of the coreset/searchspace for monitoring this can be left to RAN1).

- Step 4: Once the RRC release message is received, the UE will stop the monitoring of C-RNTI and enter normal INACTIVE state.

One question for step 1 is which RRC message shall be used in RACH based SDT, reuse the RRCResumeRequest or define a new CCCH message?

In RACH based solution, the following information will be required:

- I-RNTI: which will be used to identify the UE

- Some kind of MAC-I, which will be used to verify the UE

- Resume Cause, which will be used to indicate the cause for initiating RESUME

Based on the information above, it seems the content of the expected RRC message is anyway the same as the RRCResumeRequest, thus it is quite straight forward to reuse the RRCResumeRequest message in RACH based SDT.

Proposal 2: RRCResumeRequest message will be reused in the RACH based SDT.

In step 4, besides the RRC release message, if NW determines to switch UE to CONNECTED mode, similar as the legacy Resume procedure, the RRCResume can be sent to UE. In addition, in case no UE context can be found based on the I-RNTI, the RRCSetup message can be sent to UE as a fallback solution to switch UE to connected mode.

Proposal 3: After the reception of RRCResumeRequest, gNB can send RRCResume or RRCSetup message to switch UE to CONNECTED mode.

# RACH resource configuration for SDT

When 2-step RACH was specified, both common and separate RACH resources were supported to avoid the negative impacts on the legacy RACH resources (e.g. congestion). One question is whether, for the RACH based SDT, we allow both shared and separate RACH resources. The following options are possible then for SDT.

- Alt1: Shared ROs/POs and shared preambles for SDT and normal case

- Alt2: Shared ROs/POs but different preambles for SDT and normal case

- Alt3: Separate ROs/POs for SDT and normal case.

If only Alt2 and Alt3 are supported for SDT, then, the network will know the SDT cause already from MSG1 and hence the MSG3/MSGA payload size can be tailored to SDT and there is no need for separate SDT indication in MSG3/MSGA payload. On the other hand, if Alt1 is not supported then separate RACH resources will be needed for SDT and this may increase the load on the overall RACH resources in the cell. So, first RAN2 should decide whether Alt2+Alt3 alone is sufficient.

Observation 1: RAN2 shall decide whether the following alternatives are sufficient for SDT.

- Shared ROs but different preambles for SDT and normal case

- Separate ROs for SDT and normal case.

If Alt1 (i.e. shared ROs and preambles) is also to be supported, then there will be ambiguity at the network whether the access attempt is for SDT or not. If MSG3/MSGA payload size is large enough then the contents of the MSG3/MSGA payload (e.g. BSR) can indicate already that the attempt is for SDT. However, if the MSG3/MSGA payload can only accommodate CCCH payload like today, then the network doesn’t know if the access attempt is for SDT. Thus, it seems Alt1 would need further discussion on how to indicate SDT cause (e.g. in MSG3/MSGA payload – but then this indication may come too late to tailor the MSG3/MSGA size).

Considering the above, we propose the following:

Proposal 4: For RA-SDT only the following options are supported:

- Shared ROs but different preambles for SDT and normal case

- Separate ROs for SDT and normal case

For alternative3, since separate ROs are used for SDT and normal RA, based on the formula of RA-RNTI, in case the ROs from separate resource pool are overlapped in time domain and have the same FID (i.e. different frequency but the same FID), then the same RA-RNTI/MSGB-RNTI will be used, in which case, some ambiguity may exist in the MSG2/MSGB reception.

Observation 2: In case separate ROs are used for SDT and normal RA, there will be some ambiguity in MSG2/MSGB reception

With the ambiguity, since UE cannot distinguish the MSG2/MSGB for different resource pool, the early MSG2/MSGB will be used by both of the UEs using RA resource from different resource pool and the benefit of having separate RACH resource pool is gone (i.e. the consequence is the same as the two UEs selecting the same preamble from one RACH resource pool). Therefore, we think a solution is preferable to solve this ambiguity.

Proposal 5: In case separate ROs are configured for SDT and normal RA attempts, UE should be able to distinguish the MSG2/MSGB for SDT from the MSG2/MSGB for other normal RA attempts.

To avoid the ambiguity and distinguish the RA for SDT from the normal RA attempt, the following alternatives can be considered:

- Alt1: RA-RNTI based solution (e.g. a special RA-RNTI can be used to indicate the MSG2/MSGB for SDT, and more information can be included in MAC RAR to identify UE)

- Alt2: Separate SearchSpace/CORESET (needs RAN1 input)

In 2-step RACH, one fixed offset (i.e. 14 × 80 × 8 × 2) is added in the formula for MSGB-RNTI to avoid the overlapping between RA-RNTI and MSGB-RNTI. However, it seems we don’t have enough RNTI space to introducce one additional fixed offset for the SDT. Although alternative 1 is still possible by using a special RA-RNTI/MSGB-RNTI and some additiaonl inforatmion in MAC RAR/MSGB-RAR, it seems considerable compelxity will be introdeuced as well.

In addition, also considering the alternativce 2 can be used to avoid the congestion on the legacy RA SearchSpace/CORESET (i.e. by using separate configuration of SearchSpace/CORESET for SDT), we prefer to endorse alternaitve 2 as working assumption in RAN2, and ask RAN1 to confirm the feasibility.

Proposal 6: To solve the ambiguity related to two ROs overlapping in time but different in frequency, a new searchSpace/CORESET for SDT is preferred from RAN2 perspective, and ask RAN1 to confirm the feasibility.

# Conclusion and proposals

Based on the analysis above, we give our proposals as follow:

Stage2 procedure for RACH based SDT

Proposal 1: The following procedure shall be taken as baseline for the RACH based SDT.

- Step 1: Once RA-SDT is initiated, the UE should transmit RRC message through MSG3 (4-step RACH) or MSGA (2-step RACH). In addition to the RRC message, MAC CEs (e.g. BSR) and DRB data packet can be included in MSG3/MSGA as well

- Step 2: Once the MSG3/MSGA is transmitted, the UE will monitor RA-RNTI/MSGB-RNTI for MSG2/MSGB.

- Step 3: Once RACH procedure is successfully completed, the UE should monitor C-RNTI for subsequent UL/DL transmission (details of the coreset/searchspace for monitoring this can be left to RAN1).

- Step 4: Once the RRC release message is received, the UE will stop the monitoring of C-RNTI and enter normal INACTIVE state.

Proposal 2: RRCResumeRequest message will be reused in the RACH based SDT.

Proposal 3: After the reception of RRCResumeRequest, gNB can send RRCResume or RRCSetup message to switch UE to CONNECTED mode.

RACH resource configuration for SDT

Observation 1: RAN2 shall decide whether the following alternatives are sufficient for SDT.

- Shared ROs but different preambles for SDT and normal case

- Separate ROs for SDT and normal case.

Proposal 4: For RA-SDT only the following options are supported:

- Shared ROs but different preambles for SDT and normal case

- Separate ROs for SDT and normal case

Observation 2: In case separate ROs are used for SDT and normal RA, there will be some ambiguity in MSG2/MSGB reception

Proposal 5: In case separate ROs are configured for SDT and normal RA attempts, UE should be able to distinguish the MSG2/MSGB for SDT from the MSG2/MSGB for other normal RA attempts.

Proposal 6: To solve the ambiguity related to two ROs overlapping in time but different in frequency, a new searchSpace/CORESET for SDT is preferred from RAN2 perspective, and ask RAN1 to confirm the feasibility.

# References

- xxx