3GPP TSG-RAN WG2 #112e	R2-2009219

Electronic meeting, November 2nd – 13th 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 0914 | rev | - | Current version: | 16.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to SL grant terminology |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2020-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current MAC spec, a SL grant may indicate resources for one or more TBs, which is different from the existing definition. Therefore, a clearer definition on the term “SL grant” is necessary. In addition, a SL grant is considered as a dynamic SL grant in two cases. - Case 1: a SL grant received on the PDCCH addressed to SL-RNTI - Case 2: a SL grant received on the PDCCH addressed to SLCS-RNTI with NDI = 1 However, such definition is missing in the current MAC spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Included definitions in clause 5.22.1.1 - Definition for the terminology “SL grant” - Definition for the terminology “dynamic SL grant” Impact analysis Impacted functionality Sidelink grant reception Inter-operability: If UE implements the changes but not the network, gNB may schedule a grant which is not expected by UE. If UE doesn’t implement the changes but the network does, UE may have misinterpretation of a SL grant issued by a gNB. If one UE implements the changes, but the other UE doesn’t implement the changes, there will be different UE behaviors regarding SL grant reception. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE actions regarding SL grant reception would be unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.22.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of the first change

#### 5.22.1.1	SL Grant reception and SCI transmission

Sidelink grant is received dynamically on the PDCCH, configured semi-persistently by RRC or autonomously selected by the MAC entity. A SL grant received in Sidelink resource allocation mode 1 can indicate resources corresponding to transmissions of one MAC PDU. A SL grant created in Sidelink resource allocation mode 2 can indicate resources corresponding to transmissions of one or multiple MAC PDUs. The indicated resources are used for both initial transmissions and retransmissions. The MAC entity shall have a sidelink grant on an active SL BWP to determine a set of PSCCH duration(s) in which transmission of SCI occurs and a set of PSSCH duration(s) in which transmission of SL-SCH associated with the SCI occurs. A sidelink grant is considered as a dynamic sidelink grant in the below two cases

Case 1: the grant is received on the PDCCH addressed to SL-RNTI;

Case 2: the grant is received on the PDCCH addressed to SLCS-RNTI with NDI = 1.

Irrelevant texts are skipped

End of the first change