3GPP TSG-RAN WG2 Meeting #112-e		R2-2009331

Webinar, November 02 – 13, 2020

Agenda item:	8.11.3.2

Title:		Discussion on GNSS Integrity Errors

Source:	Swift Navigation, Ericsson, Intel Corporation, u-blox

Document for:	Discussion, Decision

# Introduction

This TP addresses Objective 2b for the Study on NR Positioning Enhancements [1]:

2. Study solutions necessary to support integrity and reliability of assistance data and position information: [RAN2] - Identify positioning integrity KPIs and relevant use cases. - Identify the error sources, threat models, occurrence rates and failure modes requiring positioning integrity validation and reporting. - Study methodologies for network-assisted and UE-assisted integrity.

The accompanying submission R2-2009333 [2] (Agenda Item 8.11.3.3) examines the corresponding methods for detecting the error sources. Additional references and prior agreements are also available in [3][4][5][6].

# Concepts and Terminology

To fully address Objective 2b it is useful to first describe the terms within this objective (and their related concepts).

## 2.1	Feared Events

Feared Events are considered to be all possible threats and failure modes (i.e. of natural, systemic or operational nature) that can cause the computed position to deviate from the true position, regardless of whether a specific fault can be identified in one of the positioning systems or not [7]. Therefore, a feared event with a probability of occurrence greater than the TIR may threaten the system’s positioning integrity (if the event is undetected), i.e. leading to Misleading Information (MI) or Hazardously Misleading Information (HMI).

Observation 1: Feared events are threats that can lead to MI or HMI in the positioning system.

## 2.2	Threat Models

Threat modelling is the process of identifying potential feared events and modelling their impacts (e.g. position error amplitude and occurrence rate). For example, to avoid HMI occurring in the system, Fault-Tree Analysis (FTA) can be used to determine all potential feared events that can lead to HMI, based on their probability of occurrence. Feared events that occur with negligible rate compared to the TIR can be excluded from the analysis, however if the occurrence rate is significant then the feared event must be mitigated. For example if the TIR is specified at 1x10-7/hr, and the threat of a satellite failure is modeled to be 1x10-5/hr, the satellite failure has a probability of occurrence greater than the TIR; it needs to be monitored as a feared event. Figure 2.2 shows a simplified example of FTA for potential feared events that may contribute to a misleading position at the UE (for UE-Based GNSS positioning).

Figure 2.2: Simplified FTA of feared events contributing to a misleading position using GNSS.

GNSS threat models are defined outside of 3GPP and the most relevant examples from the literature are those supporting Advanced Receiver Autonomous Integrity Monitoring (ARAIM) [7].

Observation 2: Threat models are an input to determining the probability of occurrence of feared events. GNSS threat models are implementation-defined.

2.3	Monitor

A monitor is used to detect the feared events that occur more frequently than is acceptable to meet the TIR, i.e. the monitor’s purpose is to reduce the likelihood that feared events go undetected. If the feared events are detected, the system is safe and functioning as intended (i.e. with no MI or HMI). CRC checks are a type of monitor, for example.

Observation 3:	Monitors are used to detect feared events which occur more frequently than is acceptable to meet the TIR.

2.4	Occurrence Rates

Different feared events have different probabilities of occurrence per unit of time. The UE is typically responsible for monitoring localized events which need to be detected in the shortest time possible, i.e. high occurrence events in the case of GNSS, such as multipath, cycle slips and high dynamic satellite events. The network is typically responsible for monitoring the low occurrence events, which are less time-critical but still depend on a reliable communication channel with the UE (e.g. atmospheric gradients, low dynamic satellite events). The rate at which the feared event information needs to be indicated to the UE will influence the required bandwidth.

Observation 4:	The rate at which feared events are indicated to the UE will influence the required bandwidth.

2.6	Error Sources

It has been agreed in [5][6] that the feared events for UE-Based GNSS positioning in the SI are categorized as follows:

- Faults in the correction data

- Faults in transmitting the data to the UE

- External feared events

- UE faults

In the case of External feared events (3), this can include the well-known GNSS error sources (e.g. SSR orbit, clock, biases, ionosphere, troposphere etc), which can be reduced through the use of corrections (e.g. LPP SSR messages in 3GPP Release 16 [9]). These corrections improve positioning accuracy, but do not ensure positioning integrity, given the remaining errors after correction remain unbounded - i.e. the GNSS correction assistance data removes the quantified errors, whereas integrity focuses on bounding the unknown or undetected errors which can otherwise lead to MI or HMI. Further consideration must therefore be given to the possibility of external feared events occurring beyond what is corrected in the existing GNSS assistance data alone. Additionally, potential faults in computing the GNSS correction data (1) also need to be considered as feared events. Further analysis of the four error sources is provided in the following section.

# GNSS Integrity in 3GPP

When A-GNSS support was introduced to 3GPP LPP Release 9, some initial IEs related to integrity were also included to be able to indicate faulty satellite signals to the device, specifically the IE GNSS-RealTimeIntegrity in the GNSS-GenericAssistDataElement IE (see Figure 3 below). However, these IEs alone are not sufficient to support a full integrity implementation and more need to be proposed through the SI/WI.

Figure 3: Example of GNSS-RealTimeIntegrity indicator within the LPP assistance data [9].

– GNSS-RealTimeIntegrity The IE GNSS-RealTimeIntegrity is used by the location server to provide parameters that describe the real-time status of the GNSS constellations. GNSS-RealTimeIntegrity data communicates the health of the GNSS signals to the mobile in real-time. The location server shall always transmit the GNSS-RealTimeIntegrity with the current list of unhealthy signals (i.e., not only for signals/SVs currently visible at the reference location), for any GNSS positioning attempt and whenever GNSS assistance data are sent. If the number of bad signals is zero, then the GNSS-RealTimeIntegrity IE shall be omitted. -- ASN1START GNSS-RealTimeIntegrity ::= SEQUENCE { gnss-BadSignalList GNSS-BadSignalList, ... } GNSS-BadSignalList ::= SEQUENCE (SIZE(1..64)) OF BadSignalElement

BadSignalElement ::= SEQUENCE { badSVID SV-ID, badSignalID GNSS-SignalIDs OPTIONAL, -- Need OP ... } -- ASN1STOP GNSS-RealTimeIntegrity field descriptions. gnss-BadSignalList This field specifies a list of satellites with bad signal or signals.. badSVID This field specifies the GNSS SV-ID of the satellite with bad signal or signals.. badSignalID This field identifies the bad signal or signals of a satellite. This is represented by a bit string in GNSS-SignalIDs, with a one-value at a bit position means the particular GNSS signal type of the SV is unhealthy; a zero-value means healthy. Absence of this field means that all signals on the specific SV are bad..

Observation 5:	New assistance data is required in LPP to indicate feared events.

## 3.1	Feared Events (UE-Based GNSS Positioning)

The four categories of integrity feared events for UE-Based GNSS positioning are illustrated in Figure 3-A using a simplified diagram of the UE positioning architecture (see TS 38.305 [8] for a complete description of the current UE positioning architecture in NG-RAN).

A detailed description of each feared event was provided by ESA in RP-2007647 [10] and should be used as a reference for Figure 3.1 and Table 3.2 below.

Figure 3.1: High-level relationship between the GNSS Integrity feared events and the 3GPP UE positioning architecture (GNSS).

## 3.2	Indicating Feared Events

Table 3.2 describes the feared events for each category and provides examples of the types of assistance data to be indicated to the UE. It is anticipated that the work to define and specify new IEs will be undertaken in the WI phase.

Table 3.2: GNSS feared event categories for UE-based GNSS positioning integrity.

| Feared Event Category | Feared Event Sub-Category | Integrity Indicator Examples | New IEs required? |
| --- | --- | --- | --- |
| 1. Correction Data | Incorrect computation by provider, e.g. software bug, corrupt or lost data | Validity or quality flags for existing assistance data IEs | Yes |
|  | External feared event impacting provider, e.g. station outages, or other external feared event as per (3) |  |  |
| 2. Transmission to the UE | Data corruption | Data corruption check, e.g. CRC | Maybe* |
|  | Malicious attack | Data Authentication / Signature | Maybe* |
| 3. External Feared Events | Satellite feared events | Bad Signal in Space | Maybe* (possible to re-use GNSS- RealTimeIntegrity) |
|  |  | Bad Broadcast Navigation Data | Yes |
|  | Atmospheric feared events | Ionosphere disturbance | Yes |
|  |  | Troposphere disturbance | Yes |
|  | Multipath | N/A | No** |
|  | Spoofing | FFS, research topic in the GNSS literature | FFS |
|  | Jamming | FFS, research topic in the GNSS literature | FFS |
| 4. UE Feared Events | GNSS receiver design | N/A | No** |
|  | GNSS receiver noise | N/A | No** |
|  | Reception and decoding of GNSS assistance data | N/A, except to the extent specified in (2) | No** |

*Maybe means the parameters require further study to determine whether existing IEs can be utilized or extended.

**No means it is not possible to mitigate with assistance data from the network, the UE is responsible for mitigating these feared events locally on implementation.

Proposal 1: Agree to include Table 3.2 in Section 9.3.1.1 of the Skeleton TR.

# Conclusions

Observation 1: Feared events are threats that can lead to MI or HMI in the positioning system.

Observation 2: Threat models are an input to determining the probability of occurrence of feared events. GNSS threat models are implementation-defined.

Observation 3:	Monitors are used to detect feared events which occur more frequently than is acceptable to meet the TIR.

Observation 4:	The rate at which feared events are indicated to the UE will influence the required bandwidth.

Observation 5:	New assistance data is required in LPP to indicate feared events.

Proposal 1: Agree to include Table 3.2 in Section 9.3.1.1 of the Skeleton TR.

# References

- 3GPP RP-193237, “New SID on NR Positioning Enhancements”, Qualcomm Incorporated, December 2019.

- R2-2009333, TP for GNSS Integrity Methodologies, RAN2#112-e, Swift Navigation, Ericsson, Intel Corporation.

- R2-2009129, Summary of [Post111-e][626][POS] Email Discussion on integrity use cases and specification impacts, Swift Navigation.

- R2-2008262, [AT111-e][607][POS] Summary of email discussion on Integrity definitions, KPIs, and use cases (Swift), RAN2#111-e, Swift Navigation.

- R2-2008125, Report from session on positioning and sidelink relay, RAN2#111-e, MediaTek.

- R2-2008263, [AT111-e][613][POS] Integrity Error Sources (Huawei), RAN2#111-e, Huawei, HiSilicon.

- Working Group C (WG-C), “EU-U.S. Cooperation on Satellite Navigation”, ARAIM Technical Subgroup, Interim Report, Issue 1, December 2012.

- 3GPP TS 38.305, “Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN”.

- 3GPP TS 37.355, “LTE Positioning Protocol (LPP)”.

- RP-2007647, “Discussion on GNSS position integrity error sources”, RAN2#111-e, ESA.

ARAIM extends the RAIM concept to account for multiple constellation threats rather than the single-fault assumption using GPS alone. It increases interoperability between the US GPS and EU Galileo navigation systems and defines the Integrity Support Message (ISM), which is further examined in the integrity methodologies submission [2].