3GPP TSG-RAN WG2 Meeting #112 Electronic	R2-2009419

Elbonia, 02 – 13 November 2020

Agenda item:	6.10.1

Source:	Nokia, Nokia Shanghai Bell, Ericsson

Title:	User consent principles

WID/SID:	NR_SON_MDT-Core - Release 16

Document for:	Discussion and Decision

# 1	Introduction

While finalizing Rel-16 NR MDT WI, RAN2 has sent LS to SA5 in [1]. The LS contained the sentence, that is conflicting with SA5 specification and requires further clarification:

In addition, RAN2 would like to inform SA5 that from RAN2 understanding, network shall not configure UE to report location information for SON/MDT purpose if network doesn’t get the user consent for this UE. The user consent requirement should also be applied to RLF reporting, CEF reporting and SCG failure case.

In this contribution, we analyse background for the currently standardized user consent requirements and clarify that NR MDT framework intends to follow LTE MDT framework worked out for the user consent.

# 2	Discussion

User consent handling has been carefully developed for the first MDT version in Rel-10. The aspect of security concerns and legal activation of MDT were heavily discussed among:

- RAN groups: RAN2, RAN3, RAN, and

- SA groups: SA3, SA5, SA2, SA1 and SA, and

- CT1 and CT4.

Several liaison statements were exchanged [3][4][5][6].

Given the SA3 recommendations in [6], for the way of applying user consent, all involved working groups took into account privacy and security aspects on the user involvement for the MDT.

## 2.1	RAN2 support for user consent since Rel-10

The RAN2 LS on common understanding reached out on user consent in Rel-10, has been sent in [6]:

- For the case when there is prior consent for a UE as part of user subscription profile: The following are RAN2 understanding and assumption: - For signaling based MDT, the OAM and/ or core network nodes will take into account user consent status when performing UE selection for MDT before contacting RAN node. - For management based MDT, since the eNB/RNC is the node who performs UE selection, the information of user consent status needs to be known by and informed to the eNB/RNC. - For the case when the user revokes/ withdraws its consent for MDT: RAN2 agreed to adopt the following mechanisms: - User consent revocation shall be performed in upper layer, e.g., using web portal. In this mechanism interworking between upper layer and AS layer is not needed. - In addition to that, RAN2 will also specify that logged MDT configuration and measurement log “shall” be released/ deleted whenever UE is switched off or detached.

Consequently, RAN2 introduced the requirement into TS37.320, in section 5.1.3. However, the core content of the standard for the user consent procedures is specified in SA5 specification: TS32.422, which is also key reference in the section on MDT user consent, in the abovementioned stage 2 TS37.320.

Observation 1: User consent procedures are transparent to RRC.

## 2.2	SA5 support for user consent

Based on SA3 recommendations, there has been consensus that user consent is explicitly required for both: Signalling and Management based MDT, and a corresponding standard text specifying the procedure can be found in TS 32.422, section 4.6.

Example of delivering user consent information can be shown in following figure:

# Figure 1: Example for handling user consent (based on TS32.422, Figure 4.6.2.1)

SA5 introduced user consent with the requirement toward RAN node, that it can select the UE for MDT data collection, only if the consent for the user has been retrieved and included in the UE context. The MDT activation, concern two kinds of MDT jobs:

- Job type= Logged MDT and

- Job type = Immediate MDT.

Observation 2: According to TS32.422, user consent is strictly required for Logged MDT and Immediate MDT. The user consent determines user selection by RAN node.

## 2.3	SA5 support for RLFreport and CEFreport

A standard text specifying the network procedures to collect RLFreports and CEFreports can be found in TS 32.422, section 4.8.

The procedures, distinguish another two kinds of MDT jobs:

- Job type= RLF reporting only, and

- Job type = RCEF reporting only.

The network procedures, for these two types of reporting do not require user consent.

Observation 3: According to TS32.422, user consent has not been required for RLFreport and CEFreport.

In our understanding, there are two main reasons to collect the data in a different way for RLF and CEF:

- RLFreport and CEFreport are not configured to the UE, thus there is no need to confront user selection with stored user consent, i.e. there is no MDT activation step nor RRC pre-configuration.

- RLFreport and CEFreport are one-shot kind of reports, without risking continued/logged measurements taken by the user

Observation 4: The RLF reports and CEF reports are collected without any prior-configuration from the gNB.

## 2.4	RAN2 goal with RLFreport and CEFreport

The RLF reports and CEF reports are key enablers for fundamental SON and MDT use cases: Coverage Optimization, Mobility Optimization. It has been carefully worked out in initial releases of SON and MDT to support the features mandatorily and record the events in the UE. They provide network vendors and operators in actual feedback from the field users on critical network issues.

For RLF and CEF reports, RAN2 has reused LTE baseline, with no prior configuration to the UE, but taking benefits of providing the reports by the users, when available.

We believe, the generic goal in RAN2 has been also to reuse well-worked out LTE framework for user consent also in NR.

Observation 5: RAN2 goal was to reuse LTE framework for user consent in NR.

Any intentional changes haven’t been particularly addressed in Rel-15 SI nor Rel-16 WI.

At the same time, it’s worth noting, that any need for user consent modifications, would require consulting other working groups (including SA3, and SA/RAN due to sensitivity of the procedures and system wide privacy requirements). While potential changes to user consent would eventually impact network side and require network interfaces support, we trust RAN2 did not have intention to trigger procedural changes to user consent without addressing the actual problems with LTE framework. Otherwise, it would limit the usability of the SON/MDT reports.

Observation 6: A need for new conditions for handling user consent should be identified jointly by relevant working groups.

While meeting privacy requirements cannot be compromised, in our understanding RAN2 had no intention to put new requirements on over-engineering user consent aspect at the expense of reduced usability and benefit of MDT.

Therefore, we think the intention of the LS should be clarified. In our understanding, the previous LS in [1]:

- primarily, targeted to ensure the SA5 receives latest status of RAN SON/MDT WI

- had no intention to modify the framework for user consent, especially for LTE (as SCG Failure is LTE report), but to highlight there is a new type of LTE report in MDT data collection

- categorized the new LTE report: SCG Failure report type together with RLFreport/CEFreport, while it concerns LTE data collection and NR data collection, respectively. Thus remains up to SA5 and RAN3 to define procedural handling and job types for the reports

Proposal: Clarify the intention of RAN2 discussions and send LS to SA5 as proposed in [7].

# 4	Conclusion

In this contribution we observe and propose the following:

Observation 1: User consent procedures are transparent to RRC.

Observation 2: According to TS32.422, user consent is strictly required for Logged MDT and Immediate MDT. The user consent determines user selection by RAN node.

Observation 3: According to TS32.422, user consent has not been required for RLFreport and CEFreport.

Observation 4: The RLF reports and CEF reports are collected without any prior-configuration from the gNB.

Observation 5: RAN2 goal was to reuse LTE framework for user consent in NR.

Observation 6: A need for new conditions for handling user consent should be identified jointly by relevant working groups.

Proposal: Clarify the intention of RAN2 discussions and send LS to SA5 as proposed in [7].

# 4	References

- R2-2006372, Reply LS on the status update of the SON support for NR works, RAN2

- S5-204542, Reply LS on the user consent for trace reporting, SA5

- S1-110172, Reply LS for MDT from SA1

- S3-110185, Reply LS on MDT user involvement

- R3-110931, LS on LS on User consent indication for MDT

- R2-111714, LS on MDT configuration with user consent

- R2-2009420, Draft reply LS on the user consent for trace reporting, Nokia