3GPP TSG-RAN WG2 Meeting #112e	R2-2009448

# Online, November 02 – 13 2020

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.306 | CR | 1792 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | RSS and relaxed monitoring capabilities for eMTC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC5-Core |  |  |  |  |  | Date: |  |  | 2020-10-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Optional UE features without capability signalling for RSS and relaxed RRM measurement, and new UE capability for RSS need to be captured as per LS in R2-2008443 and TS 36.133 v16.7.0. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Following changes are made. - ce-MeasRSS-Dedicated-r16 is clarified it is for UE supporting neighbor cell RSS measurement in any frequency location within the same MPDCCH. - ce-MeasRSS-DedicatedSameRBs-r16 is added as new UE capability for UE supporting neighbor cell RSS measurement in the same RB location as the serving cell RSS RB location. - For UEs in IDLE mode, optional features to support improved DL RSRP measurement accuracy through use of RSS and measurement of neighbour cell RSS in the same narrowband as the paging MPDCCH narrowband are added. - For UEs in IDLE mode, optional feature to support relaxation of RRM measurements for serving cell while using WUS is added. Impact analysis Impacted functionality: RSS measurement and RRM relaxation Inter-operability: No interoperability issues forseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RAN4 defined optional features and new RSS UE capability are missing from the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.6.39, 4.3.6.xx (new), 6.17.x (new), 6.17.y (new), 6.17.z (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 36.331 CR 4464 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## Start of change

### 4.3.6	Measurement parameters

## Skipped

#### 4.3.6.39	ce-MeasRSS-Dedicated-r16

This field indicates whether the UE supports improved DL RSRP measurement accuracy through use of RSS in RRC_CONNECTED and the measurement of neighbour cell RSS in the same narrowband as the MPDCCH narrowband when the UE is operating in coverage enhancement mode A or B as specified in 36.133 [16]. A UE indicating support of ce-MeasRSS-Dedicated-r16 shall also indicate support of ce-ModeA-r13.

#### 4.3.6.xx	ce-MeasRSS-DedicatedSameRBs-r16

This field indicates whether the UE supports improved DL RSRP measurement accuracy through use of RSS in RRC_CONNECTED and the measurement of neighbour cell RSS in the same 2-RBs as the serving cell RSS 2-RBs, when the UE is operating in coverage enhancement mode A or B as specified in 36.133 [16]. A UE indicating support of ce-MeasRSS-DedicatedSameRBs-r16 shall also indicate support of ce-ModeA-r13. A UE indicating support of ce-MeasRSS-DedicatedSameRBs-r16 shall not indicate support of ce-MeasRSS-Dedicated-r16.

## Next change

## 6.17	Idle mode measurements

## Skipped

### 6.17.x	Relaxed RRM measurements

It is optional for UE to support relaxation of RRM measurements for serving cell while using WUS, as specified in TS 36.133 [16]. This feature is only applicable if the UE supports ce-ModeA-r13 and also supports wakeUpSignal-r15 or groupWakeUpSignal-r16.

### 6.17.y	RSS based measurement improvement

It is optional for UE to support improved DL RSRP measurement accuracy through use of RSS in RRC_IDLE as specified in TS 36.133 [16]. This feature is only applicable if the UE supportsce-ModeA-r13 and serving cell resynchronization signals.

### 6.17.z	RSS based measurement in paging MPDCCH narrowband

It is optional for UE to support measurement of neighbour cell RSS in the same narrowband as the paging MPDCCH narrowband in RRC_IDLE as specified in TS 36.133 [16]. This feature is only applicable if the UE supports ce-ModeA-r13 and serving cell resynchronization signals.

## End of change