3GPP TSG RAN WG2 #112-e			R2-2009512

e-Meeting, November 2nd – 13th, 2020

Agenda Item:	8.10.3.1

Source:	Apple

Title:	Analysis of Mobility Management with Earth Fixed and Earth Moving Beams/Cells in NTN Networks

Document for:	Discussion/Decision

# Introduction

One of the offshoots of RAN2-lead objectives in the NTN WID [1], [2] for NTN architectures is the use of earth fixed and earth mobile cells/beams being discussed in [3]. In this contribution, we discuss our views on the same and analyze the various solutions in terms of their merits and demerits.

Multiple NTN based satellite configurations have been discussed in [4]. These not only depend on the relative height of the satellite in relation to the earth (GEO, LEO, MEO and HAPS) but also on their mobility patterns in relation to the earth. When the NTN satellite cells are moving in relation to the ground, they are termed earth moving cells else they are called earth fixed cells. Earth fixed cells use steerable beams to cover fixed locations on earth. Additionally, another architectural consideration to be made with NTN is the number of feeder links (link between the gateway and the satellite) the configuration possesses. In case a satellite can support two feeder links simultaneously a soft feeder link switchover is possible. If it cannot then the gateway will need to drop the connection before the satellite can make a new one making it a hard feeder link switchover. Each of these different configurations is shown in some modified form in Figure 1. The three cases indicated give rise to multiple issues outlined in [1]. The main set of issues (though not exhaustive) that arise due to the different architectures mentioned above can be briefly summarized as follows –

- Many connected mode UEs need to be handover over to the incoming satellite cell

- Many idle mode UEs need to reselect to the incoming satellite cell

- There might be a decently large service interruption delay due to the tearing down of one of service or feeder links and building of another especially when the satellite

[1] also proposes a list of the following solutions to tackle issues 1, 2 and 3. The solutions are –

- For the case of handing over multiple connected UEs

  - Enhanced Conditional handovers,

  - time distributed randomization of UE handovers,

  - group handover mechanisms and

  - leaving the entire procedure up to the network

- For the case of reselecting UEs to the incoming satellite

  - Stored configurations at UE informing of feeder link switch through SIB

  - Re-ordering UE reselection priorities with additional data based off of ephemeris

In the next section, we analyze our views on each of these solutions for the three cases shown in Figure 1. The three cases include – no gNB change with satellite change, gNB changes with satellite changes in the same tracking area and tracking area changes due to the satellite changes. We analyze the different solutions in terms of two main criteria – UE procedural and power impacts and Network procedural and signaling impacts.

Figure 1: a. NTN Transparent Mode Architecture with earth fixed beams/cells b. NTN Transparent Mode Architecture with earth moving beams/cells c. NTN Transparent Mode Architecture with single gNB to multiple gateways d. NTN Transparent Mode Architecture with independent gNBs connecting to different gateways e. NTN Transparent Mode Architecture with different 5GC for each satellite

# Discussion

## Case 1: Feeder link change with the two satellites connected to same gNB

This case was introduced in [3] and though not very common due to the potentially long fiber links needed to connect the gateways to the gNB is still valid. In this scenario, the gNB has to ensure that the UEs know of the incoming beam direction and send beam management commands using existing DCI based mechanisms. There is no need for any further enhancements. The situation is applicable for both earth fixed and earth moving cells and is manageable irrespective of whether the satellite uses a single beam to cover the cell or multiple beams to cover the cell. 3GPP needs to however look into mechanisms if all UEs can be sent one common DCI.

Proposal 1: For the case where two satellites are connected to the same gNB through different gateways, there is no handover and existing 3GPP mechanisms should be utilized as much as possible for notifying UEs of the satellite changes.

For the idle mode reselection scenarios, the cell ID change is an unnecessary power draw especially in certain NTN configurations (LEO, HAPS). A location based pre-stored configuration though useful in certain situations, is heavy on the UE power consumption due to the frequent location check that needs to be made. A solution that needs UE to report is location frequently is also unacceptable as it not only suffers from the power consumption issues discussed so far but also privacy issues to user.

Proposal 2: RAN2 to discuss and introduce new reselection mechanisms with UE power saving as a criterion for frequent feeder link changes to the same gNB.

Alternately, if a mechanism can be identified where a satellite change does not cause a cell ID to change, then the entire reselection mechanism on UE need not change.

Proposal 3: RAN2 to send a LS to RAN3 to confirm cell ID can remain fixed with satellite change.

## Case 2: gNB change with satellite change

In this scenario, a connected mode Xn handover needs to be completed by all UEs every time a satellite change happens. This procedure is expensive in terms of network signaling. We now proceed to analyze the different solutions proposed.

Using enhanced conditional handover procedures using location information as mentioned in the previous section is expensive on the UE. The network signaling impact is still not completely reduced due to the need for Handover Command in the CHO preparation phase and the need for all UEs receiving these individual commands to be acknowledged. In the CHO execution phase, all the UEs need to send a Handover Complete message to the target gNB.

Observation 1: Enhanced conditional handovers while not only reducing the signaling impact also cause UE power related issues.

With time distributed random-access procedure, though signaling towards network itself is time distributed, this scheme will not be completely feasible when the number of UEs in the system is very high. Also, this time distribution will cause additional delays on at least a set of the UEs.

Observation 2: Time distributed random-access procedure can add additional delays to handover completion at least to some UEs over baseline mechanisms.

A group handover message in this scenario would reduce the need for individual handover messages to each UE. However, the reception of such a message and the successful application of the following UE specific configuration messages however would depend on the number of UEs needing to request these specific configurations and will determine the network load. In this scenario, the UEs however, will experience additional control plane delay in terms of configuration completion and data plane continuity. Considering the long latency of the satellite links this control plane delay to re-establish data continuity might not be negligible.

Observation 3: While group HO procedures with UE specific signaling can reduce some downlink signaling load, there is a potential for additional control plane delay on the UEs for data resumption.

Leaving the entire procedure up to the network might also be a viable solution. One potential network-based mechanism which can save network signaling and also not introduce additional UE power considerations would be when the cell ID observed by the UE is not changed with every satellite change. 3GPP in RAN3#88-e has agreed to retain the mapping of Tracking area ID and physical earth co-ordinates similar to those in terrestrial networks today[5]. Considering the large cell sizes possible in satellite networks (due to their large coverage areas), a similar mechanism to mapping Cell ID to physical earth co-ordinates for would solve many issues related to increased UE and networks signaling. This, however, needs a LS to RAN3 as mentioned in Proposal 3. Considering that the proposed network-based solution is out of scope for RAN2, we have the following way forward on this topic.

Proposal 4: For the case of a gNB change with satellite change, RAN2 to wait for RAN3 to conclude on the cell ID mapping schemes for non-terrestrial networks before devising a viable solution for the problem of frequent handovers and reselections.

For the case of idle reselections, the concerns raised in section 2.1 and proposals 2 and 3 are still valid.

## Case 3: Satellite change leading to a gNB and AMF

In this scenario, an N2-based handover procedure is needed. This scenario is possible for cases where UE is highly mobile (UE on highspeed train on earth or inside a plane traveling in air). In such situations, UE along with the handover commands would also need to perform registration updates with the core thus increasing signaling. Currently, RAN2 does not include this scenario into its list of potential use cases.

Proposal 5: RAN2 to consider the case of potential AMF (5GC) change with satellite change as a use case for designing mobility solutions for non-terrestrial networks.

# Conclusion

In this contribution, we provided our views on cell selection and reselection enhancements for NTN. Our observations and proposals are as follows:

Observation 1: Enhanced conditional handovers while not only reducing the signaling impact also cause UE power related issues.

Observation 2: Time distributed random-access procedure can add additional delays to handover completion at least to some UEs over baseline mechanisms.

Observation 3: While group HO procedures with UE specific signaling can reduce some downlink signaling load, there is a potential for additional control plane delay on the UEs for data resumption.

Proposal 1: For the case where two satellites are connected to the same gNB through different gateways, there is no handover and existing 3GPP mechanisms should be utilized as much as possible for notifying UEs of the satellite changes.

Proposal 2: RAN2 to discuss and introduce new reselection mechanisms with UE power saving as a criterion for frequent feeder link changes to the same gNB.

Proposal 3: RAN2 to send a LS to RAN3 to confirm cell ID can remain fixed with satellite change.

Proposal 4: For the case of a gNB change with satellite change, RAN2 to wait for RAN3 to conclude on the cell ID mapping schemes for non-terrestrial networks before devising a viable solution for the problem of frequent handovers and reselections

Proposal 5: RAN2 to consider the case of potential AMF (5GC) change with satellite changes as a use case for designing mobility solutions for non-terrestrial networks.

# References

- RP-193234, Solutions for NR to support non-terrestrial networks (NTN), Sitges, Spain, Dec. 2019.

- RP-201256, Solutions for NR to support non-terrestrial networks (NTN), Jun. 2020.

- RAN2 Email discussions, [Post111-e#910][NTN] Impacts of earth fixed and moving beams (Ericsson)

- 3GPP TR 38.821, Solution for NR to support non terrestrial networks (NTN), v. 16.2.0, Dec. 2019.

- R3-205451, RAN3 Chairman’s notes, v. 3, Aug. 2020.